3GPP TSG-RAN WG4 Meeting # 116		R4-2509965

Bengaluru , IN, August 25 – 29, 2025

Agenda item:	7.3.3

Source:	Apple

Title:	TP to TR38.768 on low NR band aggregation via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

In the previous RAN4 meeting (Meeting #115, Malta, 19th – 23rd May, 2025), several aspects of the low NR band aggregation via switching feature were debated. As highlighted in the meeting WF [1], agreements were made on the time masks, switching period locations, as well as the addition of 70s to the list of switching period values, resulting in a set of 3 values (35s, 70s, and 140s) as possible values for the UE switching period capability when the UE supports the low NR band aggregation via switching feature. The TR38.768 [2] was updated to include the requirements for the time masks and switching period locations. However, the new value of the switching period (70s) was not added in the TR.

This contribution provides the following text proposals to TR38.768:

-	Change 1: Include 70s switching period value in all the clauses where the switching period values are captured.

-	Change 2: Add the applicability of the inter-band carrier aggregation aspects for a UE which supports the low NR band carrier aggregation via switching

-	Change 3: The configuration aspects related to low NR band carrier aggregation via switching

-	Change 4: Impact on MSD requirements

-	Change 5: Impact on ΔRIB,c and ΔTIB,c requirements

-	Change 6: Aspects related to the RMCs

# 2	References

[1]	R4-2508118, “WF on UE RF requirements for LB CA via switching,” by Apple, 3GPP RAN4 #115, Malta, 19th – 23rd May, 2025

[2]	R4-2508119, “TR 38.768-004” by Apple, 3GPP RAN4 #115, Malta, 19th – 23rd May, 2025

[3]	R4-2505439, “UE RF requirements for LB CA via switching,” Apple, 3GPP RAN4 #115, May 2025

# 3	Text Proposal

<< begin change 1 >>

## 5.1	Switching periods

To accommodate different UE implementations, RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs, 70μs, and 140μs for switching between Case 1 and Case 2 as an optional UE capability.

## 5.2	Switching period location

### 5.2.1	General

For what follows we consider switching between an FDD and SDL band for the CA-n5A-29A configuration in Table 4-1 but without loss of generality: one UL is configured in a paired band with the DL Scell configured in either a DL band of another paired band or in an SDL band.

.

The timing reference for the switching pattern and switching gap at the UE is the DL timing of the Pcell (in the FDD band). The uplink radio frame transmission on the Pcell takes place TTA = (NTA + TAoffset)Tc before the reception of the first detected path (as defined in TS 38.133) in time on the corresponding DL radio frame. This means that the timing advance must be considered at switching events.

The duration of the switching gap is RRC configured. The network ensures that the switching gap is large enough to cover the switching period (35 , 70μs, and  140 𝜇s according to the UE capability), transient periods and the TA, if needed, in both switch directions.

### 5.2.2	FDD to SDL switch

For the FDD to SDL switch the gNB configures a gap long enough to accommodate timing advance such that no collisions should occur. Figure 5.2.2-1 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods, switching period (35, 70μs, and  140 𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier.

The UL slot will always end before the DL slot, neglecting any timing errors in case TA = 0 µs. The worst case in terms of switching is that in which TTA = 0 µs, the DL and UL timing are the same (again neglecting the errors that are of the order of 1 µs) and the SDL starts RTD µs before the Pcell DL slot timing. The switching gap with respect to the end of the last FDD DL symbol must therefore be at least

actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period.

where the MRTD = [CP length/TAE] assuming that the FDD and SDL carriers are colocated.

### 5.2.3	SDL to FDD switch

For the SDL to FDD switch, the switching gap ends at end of slot on “switch from” carrier (SDL). The timing advance must also be accounted for; Figure 5.2.3-1 shows the case where the switch is assumed to occur before the start of the time advanced UL. The figure illustrates the applicable transient periods, switching period (35 , 70μs, and  140 𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

<< end change 1  >>

<< begin change 2  >>

## 5.3	Applicability of simultaneous Rx/Tx requirements

The following statement in section 5.2A.2 of TS38.101-1 captures the applicability of simultaneous Rx/Tx requirements when the UE supports the low NR band carrier aggregation via switching: “For low NR band inter-band configurations supported via switching [supportedLowBandSwitching-r19], the simultaneous Rx/Tx capability does not apply”. In addition, the following note in Table 5.2A.2.1-1 next to CA band combinations highlights the CA combinations that support the low NR band carrier aggregation via switching feature: “Simultaneous Rx/Tx is not applicable to UEs indicating support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19] for this band combination”.

<< end of change 2  >>

<< begin of change 3  >>

## 5.4	CA configurations

Low NR band inter-band CA configurations in which the UE is allowed to indicate support of the configuration via switching [supportedLowBandSwitching-r19] are indicated with the corresponding note in the configuration tables in sub-clause 5.5A.3.1 of TS38.101-1.

<< end of change 3 >>

<< begin of change 4  >>

## 5.5	MSD requirements

Notes in Table 7.3A.5-1 of TS38.101-1 can be used to indicate that the specified MSD is not applicable to UEs indicating support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19] in cases when existing MSD requirements apply to a low NR band CA configuration for which aggregation via switching is applicable.

<< end of change 4 >>

<< begin of change 5>>

## 5.6	ΔRIB,c and ΔTIB,c requirements

- For ΔRIB,c requirements, notes in Table 7.3A.3.2.1-1 of TS38.101-1 indicate if the specified ΔRIB,c is applicable to UEs indicating support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19].

- For ΔTIB,c requirements, notes in Table 6.2A.4.2.3-1 of TS38.101-1 indicate if the specified ΔTIB,c is applicable to UEs indicating support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19].

<< end of change 5>>

<< begin of change 6  >>

## 5.7	REFSENS requirements and RMC

The maximum throughput of the reference measurement channel will change when switching between FDD and SDL carriers as symbols will be blanked to accommodate the switching period.  Using the RAN1 agreements on the switching period location and the number of switching events within one frame, the frame structure shown in Figure 5.7-1 below can be used to define the reference measurement channel for REFSENS tests of UEs configured with low NR band carrier aggregation via switching [supportedLowBandSwitching-r19].

Figure 5.7-1: Frame structure for FDD-SDL

The information payload and max throughput averaged over one frame are recalculated based on this frame structure and introduced as new tables to TS38.101-1 Annex A.3.2.2 to enable the REFSENS tests of the UE.  For the UL RMCs the resource allocations for the FDD carrier (TS38.101-1 Annex A.2) are restricted to avoid configuring the test equipment with simultaneous Rx/Tx for UEs configured with low NR band carrier aggregation via switching [supportedLowBandSwitching-r19].

<< end of change 6 >>