3GPP TSG RAN WG4 Meeting #116	R4-2509995

# Bengaluru, India, 25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0658 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: Correction on EEIRP calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The coordinate system used for the EEIRP system is different from the reference coordinate system.However, in the normative annex, the coordinate system conversion is specified only for the case non-zero mechanical tilt is considered. The coordinate system conversion shall be applied to the EEIRP calculation regardless of the non-zero mechanical tilt consideration. Some editorial corrections required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The coordinate system conversion is applied to the EEIRP calculation without conditions. Duplicated text removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EEIRP cannot be calculated when non-zero mechanical tilt is not applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Appendix N |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

Annex N (normative):
Expected EIRP calculation and spatial sampling grid

N.1	General

The Expected EIRP calculation is based on two averaging processes:

-	Averaging over the test beam directions in clause.

-	Averaging over horizontal and vertical angles within specified vertical angles bins

The required sampling resolution for the EIRP measurement per test beam direction is described in annex N.3

N.2	Averaging processes

This clause describes the details of the calculation of the EEIRP from the measured EIRP samples.

N.2.1	Averaging over beamforming directions

The first averaging process involves averaging over different test beam directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=1Kk=1KwkEIRPk(θ,φ)

The power measured is the sum of the EIRP in both polarisations. The EIRPθ,φ is the average EIRPk(θ,φ) over the K number of test beam directions for the angle (θ,φ).

For the EEIRP calculation the reference coordinate system in clause 4.14 in this document needs to be converted to the coordinate system used by the test requirement (Figure 6.9.5.1-1). The angles in the reference coordinate system can be translated to the coordinate system with respect to horizon as:

θH=90-arccoscosφsinθ+90sin-θMT+cosθ+90cos-θMT

φH=argcosφsinθ+90cos-θMT-cosθ+90sin-θMT+jsinφsinθ+90

, where

θ	The angle in the reference coordinate system defined in sub-clause 4.14 between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

φ 	is the angle in the reference coordinate system defined in sub-clause 4.14 between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

θH 	is the angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

θMT	is the down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).

φH	The horizontal angle in the global coordinate system, defined with respect to the horizon.

N.2.2	Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore:

EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn

, where

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰           and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φmis the corresponding angle of the mth midpoint between the azimuth intervals

nLis the lowest elevation index of sampling points within the θHLi , θHHi range

nHis the highest elevation index of sampling points within the θHLi , θHHi range

Mis the number of azimuth points within the (-180°, 180°) range

Nis the number of elevation points within the θHLi , θHHi range

NOTE:	All angles are in degrees.

--------------End of change-------------