3GPP TSG-RAN WG4 Meeting #116	R4-2510009

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.6.5.1

Source:	MediaTek Inc.

Title:	Discussion on NSGO channel model for NTN testing

Document for:	Discussion

# Introduction

In this contribution, we provide our views based on the agreed WF for NSGO channel model [1].

# Discussion

Issue 1-1-3 Initial ephemeris information for LEO-600 satellite - agreement - Sets the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654685 , 4387308, 1593097, 14569, -34465, 120190), which corresponds to (-3451.0905 km, 5703.5004 km, 2071.0261 km, 0.87414 km/s, -2.0679 km/s, 7.2114 km/s), related with the initial elevation angle of 30 degrees Issue 1-1-4 Initial ephemeris information for LEO-1200 satellite - agreement - Reuse TS 38.508 Table 7.6.2.1-3a as the initial ephemeris information for LEO-1200 Table Initial ephemeris information for LEO-1200 stating from elevation angles of 30 degrees (Copy from TS 38.508 Table 7.6.2.1-3a, serving cell 30 degrees elevation angle conditions). Parameters; Unit; Values. positionX-r17; km; -3816.2176. positionY-r17; km; 6364.098. positionZ-r17; km; 1544.9954. velocityVX-r17; km/s; 0.51036. velocityVY-r17; km/s; -1.39884. velocityVZ-r17; km/s; 7.09518.

At RAN4#115, the initial ephemeris information for LEO-600 and LEO-1200 was established. Specifically, the initial ephemeris data for LEO-1200 at 30 degrees, as outlined in TS 38.508 Table 7.6.2.1-3a, was extracted from the accompanying Excel file referenced in [2]. Although there was consensus on the initial ephemeris information for LEO-600 in [1], we recommend adopting the same methodology to derive this data from the Excel file in [2]. This file provides the position and velocity vectors near the evaluation angle of 30 degrees for LEO-600, as detailed below.

Based on the Excel file in [2], we propose setting the initial ephemeris information for LEO-600 as follows: (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182), which corresponds to (-3450.5237 km, 5703.0883 km, 2072.4665 km, 0.87486 km/s, -2.06922 km/s, 7.21092 km/s). These values are associated with the initial elevation angle of 30 degrees

Proposal 1: Set the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182), which corresponds to (-3450.5237 km, 5703.0883 km, 2072.4665 km, 0.87486 km/s, -2.06922 km/s, 7.21092 km/s), related with the initial elevation angle of 30 degrees.

# Conclusion

In this contribution, we provide our views on demodulation performance requirements for NGSO channel model and summarize our proposals as below.

Proposal 1: Set the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182), which corresponds to (-3450.5237 km, 5703.0883 km, 2072.4665 km, 0.87486 km/s, -2.06922 km/s, 7.21092 km/s), related with the initial elevation angle of 30 degrees.

# Reference

- R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod, Samsung

- R5-237216, NGSO satellite Ephemeris file generation methodology for NTN NR UE testing, THALES