3GPP TSG-RAN WG4 Meeting # 116	                                                           R4-2510102

Bengaluru, India, Aug. 25 – 29, 2025

Agenda Item:      7.25.3.3

Source:	MediaTek Inc.

Title:	Discussion on conditional LTM

Document for:	Discussion

- Introduction

In this contribution, we provide our views on the left issues of conditional LTM.

- Discussion

2.1 Conditional cell switch delay and interruption

Last meeting, RAN4 reached the following progress for conditional cell switch delay and interruption requirements.

Issue 1-2-4: General on Tfirst-RS and TRS-proc < Agreement > - [Tfirst-RS and TRS-proc] are included in Tinterrupt. Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 < Agreement > ad hoc - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. - Note: the limitation of 160/480 ms from Rel-18 still applies.

The most controversial part is when UE is supposed to do T/F tracking. As well known, to skip T/F tracking, there are some conditions. In R18 LTM, if the time gap between UE activates TCI state and receives cell switch command is less than 160ms, then UE can skip T/F tracking during cell switch. As shown below, in R18 LTM, it is easy to make sure the time gap between UE activates TCI state and receives cell switch command is less than 160ms, as both TCI state activation and cell switch are under NW control. But here in conditional LTM, as NW don’t know when conditional cell switch will be triggered, it is quite difficult to guarantee the 160ms condition.

Observation 1: It is not easy for NW to guarantee that T/F tracking can be skipped during conditional cell switch.

When UE performs measurement, it is possible to do T/F tracking. But due to limited capability, the number of beams UE can perform T/F tracking on is much less than the capability that UE can measure. A possible way may be that UE starts T/F tracking on the cell when enter condition is met.

Active TCI state and C-LTM is triggered by L1 event: UE is supposed to do T/F tracking when performing L1 measurement. The basic assumption shall be UE not re-arrange its measurement behaviour. If TTT<160ms, then so T/F tracking can always be skipped. Otherwise, whether T/F tracking can be skipped will depend on L1 measurement period.

In FR1: Active TCI state, L1 measurement is not configured and C-LTM is triggered by L3 event: as the time gap between TCI state activation and the event is triggered may be very long, UE may already stop performing periodic T/F tracking. When UE finds the condition is met, UE can perform T/F tracking in the next measurement. There will be two potential issues. One issue is that TTT may be not long enough for UE to measure the cell again. In the test, TTT is usually set to 0.

Non-active TCI states: When UE finds the condition is met, UE can perform T/F tracking in the next measurement. TTT may be not long enough for UE to measure the cell again.

Based on the above analysis, it can be observed that it is possible to further optimize the conditions of skipping T/F tracking during conditional cell switch, but the impact of TTT shall be considered in the to-be-optimized conditions.

Observation 2: If TTT is long enough, it is possible for UE to do T/F tracking during TTT on the cell which meets the enter condition.

Proposal 1: Further optimization on the conditions of skipping T/F tracking during conditional cell switch is workable. But the length of TTT shall be considered in the to-be-optimized conditions.

In conditional LTM, it is difficult for NW to know when interruption will happen as well as whether T/F tracking can be skipped or not. We think the best solution is to reduce the overall interruption. As shown in Fig. 1 and Fig. 2, the time introduced by T/F tracking (Tfirst-RS + TRS-proc) includes two parts: the time spent on waiting for the SSB occasion “Twait_RS” and the time to do T/F tracking on SSB “TRS + TRS-proc”. Theoretically, TRS is the time needed to get ready to receive SSB on target cell plus the SSB duration. For SSB to be measured within MG, RF retuning time is needed. The SSB duration is less than 1 slot. Most of the components in HO/cell switch delay are defined in the unit of ms. We understand that the time needed for intra-frequency case can be shorter. To make the spec simpler, we prefer to define a unified value of TRS for all the scenarios. Here we propose TRS as 2ms. TRS-proc is time for SSB post-processing which can be up to 2 ms.

Here we include “TRS + TRS-proc” into interruption time due to there will be interruption on source cell to receive SSB on target cell, e.g.,

- Inter-f: UE needs to retune to another frequency

- Intra-f in FR2: Rx beam may be different from source cell

- Intra-f in FR1 with mixed numerology and UE incapable of mixed numerology

As the optimization of UE performing T/F tracking before Tprocessing has reduced the average interruption time significantly, we prefer to define a unified requirements for all the scenarios for the sake of spec simplification and readability.

Fig.1

Fig. 2

Proposal 2: For C-LTM, separate Tfirst-RS to two parts:

- Twait_RS: the time spent on waiting for the SSB occasion

- TRS: the time needed to get ready to receive SSB on target cell plus the SSB duration, which is 2 ms.

Proposal 3: Exclude Twait_RS the time spent on waiting for the SSB occasion from conditional cell switch interruption.

- Conclusion

In this contribution, we provide our views for conditional LTM and have the following proposals and observations.

Observation 1: It is not easy for NW to guarantee that T/F tracking can be skipped during conditional cell switch.

Observation 2: If TTT is long enough, it is possible for UE to do T/F tracking during TTT on the cell which meets the enter condition.

Proposal 1: Further optimization on the conditions of skipping T/F tracking during conditional cell switch is workable. But the length of TTT shall be considered in the to-be-optimized conditions.

Proposal 2: For C-LTM, separate Tfirst-RS to two parts:

- Twait_RS: the time spent on waiting for the SSB occasion

- TRS: the time needed to get ready to receive SSB on target cell plus the SSB duration, which is 2 ms.

Proposal 3: Exclude Twait_RS the time spent on waiting for the SSB occasion from conditional cell switch interruption.