3GPP TSG-RAN WG4 Meeting # 116	                                                      R4-2510107

Bengaluru, India, Aug. 25 – 29, 2025

Agenda item:	7.19.3.2

Source:	MediaTek Inc.

Title:	Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction and event prediction use-cases. In this contribution, we would like to discuss the related RRM requirements of measurement event prediction.

- Discussion

Typically, the triggering of measurement event only happens at cell boundary. For UE not at cell boundary, it does not make sense to mandate UE always activates the AI model for event prediction. A more reasonable UE implementation strategy is to only activate the AI model when the event is about to trigger. Since UE may not need to tell network exactly when it finishes the activation of the AI model, we think it should be up to UE implementation when to activate the AI model for event prediction as long as UE can meet the requirements.

Proposal 1: For measurement event prediction, it is up to UE implementation when to activate the AI model for prediction.

But when defining the requirements, we may need a reference UE behavior. We simulated the RSRP prediction accuracy for different PW windows. In the simulation, the settings are as below:

- Scenario: temporal prediction

- OW: 4 x 40ms, PW: 40ms, 80ms, 120ms, 160ms, 200ms, 240ms, 280ms, 320ms

- L1 filtering: Sliding window

- L3 filtering: k=4

- Inputs to AI model: RSRP of two cells (Serving cell and the target cell) at 4 occasions in OW (2x4)

- Outputs from AI model: RSRP of two cells at 8 predicted occasions (2x8)

- Measurement error is not added

- Inference performed when enter condition of A3 event (threshold = 2dB and hysteresis = 0) is met as in Fig.1.

Fig. 1

Table. 1 shows the performance of absolute RSRP difference at 5% and 95% tile points with different PWs. It can be observed that the longer PW is, the worse the absolute RSRP accuracy is.

Table 1

| Absolute RSRP accuracy (dB) |  | future time instance |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40ms | 80ms | 120ms | 160ms | 200ms | 240ms | 280ms | 320ms |
| Serving cell | 5% | -0.18 | -0.46 | -0.76 | -1.09 | -1.54 | -1.92 | -2.35 | -2.9 |
|  | 95% | 0.21 | 0.44 | 0.76 | 1.07 | 1.56 | 2.03 | 2.37 | 2.81 |
| Target cell | 5% | -0.21 | -0.48 | -0.83 | -1.18 | -1.69 | -2.17 | -2.64 | -3.08 |
|  | 95% | 0.18 | 0.44 | 0.76 | 1.17 | 1.58 | 1.89 | 2.42 | 2.74 |

RAN4 needs to discuss how to define the requirements, e.g., define different requirements for several TTT values, event thresholds or define a unified requirement. From our point, we prefer not to define a big set of requirements for different configuration parameters. As far as we know, 320ms is a typical value for TTT. If to define a unified requirement, we prefer to choose TTT as 320ms. Regard the reference UE behaviour, we prefer UE only starts to predict when enter condition of the event is met as in Fig.1. If some UE can predict more accurate, those UE can do the prediction earlier and PW may be longer.

Observation 1: The longer PW is, the worse the absolute RSRP accuracy is.

Proposal 2: Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for.

Propose 3: For measurement event prediction, the assumed UE behaviour is that UE starts to predict when enter condition of A3 event is met.

- Conclusion

In this contribution, we provide our views for measurement event prediction. Observation is summarized as follows.

Proposal 1: For measurement event prediction, it is up to UE implementation when to activate the AI model for prediction.

Observation 1: The longer PW is, the worse the absolute RSRP accuracy is.

Proposal 2: Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for.

Propose 3: For measurement event prediction, the assumed UE behaviour is that UE starts to predict when enter condition of A3 event is met.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.