# 3GPP TSG-RAN WG4 Meeting #116                               R4-2510151

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.21.4.2

Source: 	CMCC

Title: 			 Discussion on RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides discussion on RRM impacts for SBFD operation.

- Discussion

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, according to RAN1 agreements, there are two UE capabilities. UE capability 1 is CSI processing timeline and CPU counting are the same as legacy. UE capability 2 is to scale the CSI processing timeline Z/Z’by factor of 2 and keep the same CPU counting as legacy. As dicsussed in [3], the existing legacy CSI computation time can be up to 4ms. For UE support capability 2, the CSI processing time is up to 8 ms.

Following is the legacy CSI-RS based L1-RSRP measurement period. TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. According to TS38.331, it is possible that the value of TCSI-RS is smaller than 8 ms. TReport is configured periodicity for reporting. According to TS38.331, it is possible that the value of Treport is smaller than 8 ms. Taking above into account, it is reasonable to add a lower bound of 8ms on top of existing delay requirements, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. |  |

From TS 38.331 Clause 6.3.2: CSI-ResourcePeriodicityAndOffset ::= CHOICE { slots4 INTEGER (0..3), slots5 INTEGER (0..4), slots8 INTEGER (0..7), slots10 INTEGER (0..9), slots16 INTEGER (0..15), slots20 INTEGER (0..19), slots32 INTEGER (0..31), slots40 INTEGER (0..39), slots64 INTEGER (0..63), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639) } CSI-ReportPeriodicityAndOffset ::= CHOICE { slots4 INTEGER(0..3), slots5 INTEGER(0..4), slots8 INTEGER(0..7), slots10 INTEGER(0..9), slots16 INTEGER(0..15), slots20 INTEGER(0..19), slots40 INTEGER(0..39), slots80 INTEGER(0..79), slots160 INTEGER(0..159), slots320 INTEGER(0..319) } -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP -- ASN1STOP

However, it is noted that CSI processing timeline are used to process the CSI-RS across two DL subbands. For UE performing measurement on 1 DL subband, no need to consider CSI-RS processing timeline.

Proposal 1: for UE performing measurement on 2 DL subbands and UE indicating that CSI processing time is scaled by 2, it is proposed to add a lower bound of 8ms on top of existing CSI-RS based L1 measurement period, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case. Otherwise, no need to consider lower of 8ms.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: for UE performing measurement on 2 DL subbands and UE indicating that CSI processing time is scaled by 2, it is proposed to add a lower bound of 8ms on top of existing CSI-RS based L1 measurement period, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case. Otherwise, no need to consider lower of 8ms.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504480, Discussion on SBFD Operation, MediaTek inc.

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2420165, LS on CSI-RS measurement with SBFD operation

- R1-2501560, Reply LS on CSI-RS measurement with SBFD operation