# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510171

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.4

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on RRM requirements for A-IoT, and a WF was approved [2]. This contribution provides further discussion on this topic.

- Discussion

Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

In last meeting, RAN4 agreed the timing error Te for MSG X=1, and the value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. For MSG1 X=2, the same principle is used as starting point.

RAN1 agreements about timing are summarized as following. According to RAN1 design, Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2. Toffset1 is from a set of predefined values. Toffset2=αToffset1+βTmsg1, where Tmsg1 is the duration of the 1st Msg1 time domain resource, and α=0.25 and β=1.25. In our view, the RAN4 agreements on Te can be confirmed to be reused for MSG1 X=2.

Proposal 1: For D2R transmission for Msg1 transmission with X=2 time domain resource, the reference point for the device transmission timing is Toffset2 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of the first Msg1 time domain resource.

RAN1 agreements in RAN1#120bis Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. RAN1 agreements in RAN1#121 Agreement For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, - Toffset1 is not smaller than 30 us for CBRA. Agreement For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective: - Toffset1 is from a set of predefined values. Agreement Toffset2=αToffset1+βTmsg1, where - Tmsg1 is the duration of the 1st Msg1 time domain resource - α=0.25 and β=1.25 Agreement Define Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective. Agreement Define Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective. Working assumption For Toffset1, for CBRA, regardless of the use of FEC, - The reference D2R chip length is the largest D2R chip length among the scheduled FDMed Msg1 for Y>1 or the D2R chip length for Y=1. - If reference D2R chip length is 133.33 or 66,67us, Toffset1 = 20* 66.67=1333.4us - If reference D2R chip length is 33.33, 16.67, or 8.33us, Toffset1 = 20* 33.33=666.6us - If reference D2R chip length is 4.17us, Toffset1 = 4* 33.33=133.32us - If reference D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset1 = 4* 33.33=133.32us - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset1 = 3*11.11=33.33us - The R2D chip length refers to the corresponding R2D transmission triggering the D2R Agreement For Toffset3, for CBRA The reference D2R chip length is the largest D2R chip length among the scheduled FDMed Msg3 for Y>1 or the D2R chip length for Y=1. - when FEC is not used: - If reference D2R chip length is 133.33 or 66,67us, Toffset3 = 20* 66.67=1333.4us - If reference D2R chip length is 33.33, 16.67, or 8.33us, Toffset3 = 20* 33.33=666.6us - If reference D2R chip length is 4.17us, Toffset3 = 4* 33.33=133.32us - If reference D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset3 = 4* 33.33=133.32us - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset3 = 3*11.11=33.33us - The R2D chip length refers to the corresponding R2D transmission triggering the D2R - when FEC is used: down-select from the options below: - Opt1: same as when FEC is not used - Opt2: same as when FEC is not used + TFEC, where TFEC = [66.67]us - Opt3: - If reference D2R chip length is 133.33 or 66,67us, Toffset3 = 20* 66.67=1333.4us - If reference D2R chip length is 33.33, 16.67, or 8.33us, Toffset3 = 20* 33.33=666.6us - If reference D2R chip length is 4.17us, Toffset3 = 133.32us [+ 66.67us] - If reference D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset3 = 133.32us [+ 66.67us] - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset3 = 33.33+[66.67]us - The R2D chip length refers to the corresponding R2D transmission triggering the D2R Working assumption For Toffset3, for CBRA, when FEC is used - If reference D2R chip length is 133.33 or 66,67us, Toffset3 = 20* 66.67=1333.4us - If reference D2R chip length is 33.33, 16.67, or 8.33us, Toffset3 = 20* 33.33=666.6us - If reference D2R chip length is 4.17us, Toffset3 = 133.32us + X - If reference D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset3 = 133.32us+ X - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset3 = 33.33+X - The R2D chip length refers to the corresponding R2D transmission triggering the D2R - Where X= 133.3us Note: X=133.3us assumes the maximum size of Msg3 is 128 bits. Agreement For Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission. When FEC is not used: - If the D2R chip length is 133.33 or 66,67us, Toffset4 = 20* 66.67=1333.4us - If the D2R chip length is 33.33, 16.67, or 8.33us, Toffset4 = 20* 33.33=666.6us - If the D2R chip length is 4.17us, Toffset4 = 4* 33.33=133.32us - If the D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset4 = 4* 33.33=133.32us - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset4 = 3*11.11=33.33us - The R2D chip length refers to the corresponding R2D transmission triggering the D2R Agreement For Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission. When FEC is used - Same as when FEC is not used + TFEC, where - TFEC = 2X for TBS <=32bytes - TFEC = 4X for 32bytes<TBS <=64 bytes - TFEC = 8X for 64 bytes <TBS<= 125 bytes - Where X= 133.3us Agreement For the time interval from the end of a R2D transmission triggering CFRA to the starting time of the first D2R message time domain resource, from the device perspective, reuse Toffset3. Note: this assumes the maximum size of the first D2R message for CFRA is 128 bits.

RAN1 introduced Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective. Toffset3 is from a set of predefined values. Similar approach as MSG1 can be used. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG3.

Proposal 2: for MSG3, the reference point for the device transmission timing is Toffset3 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG3.

RAN1 introduced Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective. Toffset4 is from a set of predefined values. Similar approach as MSG1 can be used. For D2R transmission except for Msg1 and Msg3, the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of D2R transmission except for Msg1 and Msg3.

Proposal 3: for D2R transmission except for Msg1 and Msg3, the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of D2R transmission except for Msg1 and Msg3.

RAN1 #121 meeting Agreement It is up to RAN4 to define the value(s) for TD2R_min. - Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications RAN1 #120bis meeting Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device.

TD2R_min is the minimum time between a D2R transmission and the corresponding R2D transmission following it. A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. It can be seen that TD2R_min is about device processing time between D2R transmission and R2D reception, which could be X us, the value of X is FFS.

Proposal 4: considering that TD2R_min is about device processing time between D2R transmission and R2D reception, it is proposed that TD2R_min is X us, the value of X is FFS.

- Conclusion

This contribution provides discussion on RRM requirements for A-IoT. The proposals are:

Proposal 1: For D2R transmission for Msg1 transmission with X=2 time domain resource, the reference point for the device transmission timing is Toffset2 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of the first Msg1 time domain resource.

Proposal 2: for MSG3, the reference point for the device transmission timing is Toffset3 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG3.

Proposal 3: for D2R transmission except for Msg1 and Msg3, the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of D2R transmission except for Msg1 and Msg3.

Proposal 4: considering that TD2R_min is about device processing time between D2R transmission and R2D reception, it is proposed that TD2R_min is X us, the value of X is FFS.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC