3GPP TSG-RAN WG4 Meeting #116                                   R4-2510180

Bengaluru, India, August 25th – 29th, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	7.1.2.2.1

Document for:	Approval

- Introduction

In RAN#115, a WF was agreed upon in [1]. This document aims to progress the further open issues.

- 2. Discussion

Extension values/ratios

Agreement in RAN4#115:

- At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases

- It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides.

- It is FFS on details of the single side case.

Agreement in RAN4#115:

UE capability

Agreement in main session:

- No separate UE capabilities for scenario 1 and scenario 2

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only

  - Both dual-sided and single sided cases are supported.

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension

  - Both dual-sided symmetric and single sided cases are supported.

  - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced.

- Note: the other ratio number depends on discussion of issue 1-1-1

Based on all of the discussions from previous RAN4 meetings, and the UE capability agreements from May indicate that the following are supported:

- Symmetrical shifted OOBE, where the shift ratio on both sides may be 0.5 or 0.25

- Asymmetrical shifted OOBE, where a non-zero shift ratio on one side is applied, which may be 0.5 or 0.25

We do not see much of a need to apply also e.g. 0.5 shift ratio on left side and 0.25 shift ratio on the right side, as there are diminishing returns once you have all of the flexibility of the above options.

Proposal 1: RAN4 in May agreed to support the following cases:

- Symmetrical shifted OOBE, where the shift ratio on both sides may be 0.5 or 0.25

- Asymmetrical shifted OOBE, where a non-zero shift ratio on one side is applied, which could be 0.5 or 0.25

Proposal 2: Need for asymmetric shifted OOBE with non-zero shift ratios on both sides would need further justification of the gains.

Guard band for extended CBW

Agreement in RAN4#115:

- GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB

- Extended CBW is not limited to the existing UE CBW

- RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests

If extended CBW is not limited to existing CBW, it is difficult to understand how it can be tested just by measuring the existing CBW. Before discussing this, we should discuss what is the exact shift in OOBE requirements compared existing spec, and what is the LCRB assumed.

On whether to apply 0.5*UE CBW or 0.5*NRB, we have simulated the Symmetrical and Asymmetrical shifted OOBE for 0.5 * CBW and ceil (0.5 * NRB) shift (with original UE CBW guardband added). The 20MHz results below are for 15kHz SCS which is a worst case than 30kHz SCS in terms of NRB and spectrum utilization. Therefore, the equivalent ratios can apply for 30kHz SCS too. The Tables below show that in both cases, there should be no problem to apply either 0.5*UE CBW or ceil(0.5 * NRB).

Proposal 3: Either 0.5 * CBW and ceil(0.5 * NRB) shift are acceptable to be adopted, with the following requirements applicable for modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms for PC3:

- Maximum inner RB allocation of LCRB = NRB for symmetrical shift of 0.5* on each side, and LCRB = 2*NRB/3 for an asymmetrical shift of 0.5* on one side and zero on the other side. Maximum inner RB allocation of LCRB for any potential additional multiplier values smaller than 0.5*NRB are FFS.

- If the 0.5*NRB approach is selected, at least the guardband of the “original UE CBW” guardband should be added between the last RB of 0.5*NRB and the start of the shifted OOBE region.

Table 1a: Evaluation results for Symmetric shifted OOBE (20MHz CBW with 0.5* shift both sides

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 1b: Evaluation results for Asymmetrical shifted OOBE (20MHz CBW with 0.5* shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |

Table 1c: Evaluation results for Asymmetrical shifted OOBE (100MHz CBW with 0.5* shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |

Why to specify the extension as “shifted OOBE” rather than “extended CBW”

When defining the SEM/ACLR requirements as integrated according to UE CBW, but where the starting point is shifted from the channel edge, this is not the same as the approach taken for scaling SEM/ACLR integration over a wider CBW.

It cannot be assumed that the existing MPR rules can be applied for any combination of UE CBW and wider overlaid CBW in the same way as for a partial RB allocation operating in such a wider CBW. This would need all of the cases (including all UE CBW locations within each wider CBW) to be simulated for every combination of CBW and wider CBW, which appears to be a lot of effort for very limited gain.

Proposal 4: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

New inner RB region in extended CBW

Agreement in RAN4#115

- WF

- The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well

  - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

- Other approach is not precluded

- The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

The formula above appears to have an error because if an asymmetric OOBE extension of 0.5x is applied, then it assumes the LCRB may be ¾ of original NRB, when actually the maximum would be approximattely 2/3 of original NRB. Therefore 3 conditions are needed to calculate maximum LCRB size, and the smallest value of the 3 needs to be applied in our view. See example formula below.

LCRB ≤ floor [ min [((NRB + RBExt,Low+ RBExt,High )/2), ((NRB + RBExt,Low)*2/3), ((NRB + RBExt,High)*2/3) ]

where min() indicates smallest value of all arguments and floor(x) is greatest integer less than or equal to x.

The details of floor/ceil application may need more discussion depending on the exact shift in frequency ratio of OOBE.

Proposal 5: Modify the formula in the above agreement, accurately taking into account asymmetrical shift and 2/3 ratio case.

RRC parameters and UE capability signalling

Agreement in RAN4#115:

NW signalling and RRC parameters

Agreement in main session:

- The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities,

  - The details of the signaling including the contents are FFS

WF

- It is up to the NW to judge the gap width on both sides of the UE

- NW sends info of extension ratio (at least 1/2) per side

Proposal 6: Request RAN2 to define UE specific RRC signalling to indicate to the UE whether shifted OOBE is permitted, including the following additional information and RRC parameters:

- Whether the shifted OOBE permitted is “symmetrical” or “asymmetrical”.

  - If symmetrical, shift multiplier applies to both sides

- If asymmetrical, shift applies on one side, and zero on the other side. At least an indication of whether the shift applies to left or right side needed here.

- Shift ratio (0.5 or 0.25) with default value = 0.5

Proposed detailed RRC parameters:

| Name | Values | Conditions | Comments | UE or cell specific |
| --- | --- | --- | --- | --- |
| permittedShifted OOBE-Type | CHOICE (Symmetrical, Asymmetrical) |  |  | UE specific |
| asymmetricalShifted OOBE-Applicability | CHOICE (Left side, Right side) | Ignored by UE if permittedShifted OOBE-Type value = “Symmetrical” |  | UE specific |
| Shift ratio | 0.5 (default value); 0.25 |  | If not present, then 0.5 applies. Refer to RAN4 spec for definition of these values. | UE specific |

- 3. Conclusion & Proposals

Proposal 1: RAN4 in May agreed to support the following cases:

- Symmetrical shifted OOBE, where the shift ratio on both sides may be 0.5 or 0.25

- Asymmetrical shifted OOBE, where a non-zero shift ratio on one side is applied, which could be 0.5 or 0.25

Proposal 2: Need for asymmetric shifted OOBE with non-zero shift ratios on both sides would need further justification of the gains.

Proposal 3: Either 0.5 * CBW and ceil(0.5 * NRB) shift are acceptable to be adopted, with the following requirements applicable for modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms.

- Maximum inner RB allocation of LCRB = NRB for symmetrical shift of 0.5* on each side, and LCRB = 2*NRB/3 for an asymmetrical shift of 0.5* on one side and zero on the other side. Maximum inner RB allocation of LCRB for any potential additional multiplier values smaller than 0.5*NRB are FFS.

- If the 0.5*NRB approach is selected, at least the guardband of the “original UE CBW” guardband should be added between the last RB of 0.5*NRB and the start of the shifted OOBE region.

Proposal 4: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”.

Proposal 5: Modify the formula in the above agreement, accurately taking into account asymmetrical shift and 2/3 ratio case.

Proposal 6: Request RAN2 to define UE specific RRC signalling to indicate to the UE whether shifted OOBE is permitted, including the following additional information and RRC parameters:

- Whether the shifted OOBE permitted is “symmetrical” or “asymmetrical”.

  - If symmetrical, shift ratio applies to both sides

- If asymmetrical, shift applies on one side, and zero on the other side. At least an indication of whether the shift applies to left or right side needed here.

- Shift ratio (0.5 or 0.25) with default value = 0.5

Proposed detailed RRC parameters:

| Name | Values | Conditions | Comments | UE or cell specific |
| --- | --- | --- | --- | --- |
| permittedShifted OOBE-Type | CHOICE (Symmetrical, Asymmetrical) |  |  | UE specific |
| asymmetricalShifted OOBE-Applicability | CHOICE (Left side, Right side) | Ignored by UE if permittedShifted OOBE-Type value = “Symmetrical” |  | UE specific |
| Shift ratio | 0.5 (default value); 0.25 |  | If not present, then 0.5 applies. Refer to RAN4 spec for definition of these values. | UE specific |

- 4. References

- R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”, Moderator (Huawei)