3GPP TSG-RAN WG4 Meeting #116                                   R4-2510181

# Bengaluru, India, August 25th – 29th, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on NTN Ku band VSAT RF requirements

Agenda Item:	7.7.5

Document for:	Approval

- Introduction

In RAN4#115, the WF for NTN Ku band UE RF requirements [1] was agreed upon. However, there are still some unresolved issues that require further discussion, as shown below. In this paper, we will provide our views on the remaining issues.

- Issue 1-2-1: Clarification on the minimum peak EIRP requirements Agreement: - Currently the VSAT type 4 & type 5 covers both GSO and LEO, but companies show strong need on allowing VSAT support LEO only in Rel.19. - Consider a new type for LEO only based on the same VSAT antenna size assumption as type 4 & 5 as a working assumption - Issue 1-2-2: Minimum peak EIRP requirements for GSO Agreement: - Confirm minimum peak EIRP at 70 dBm for Rel.19 Ku band VSAT communicating with GEO. - Issue 1-2-3: Minimum peak EIRP requirements for LEO Agreement: - Confirm minimum peak EIRP at 60 dBm as min peak EIRP for both electronically steering VSAT and mechanical steering VSAT connecting to LEO. - Issue 1-2-4: RB configuration for EIRP requirement Agreement: - Consider both minimum peak & maximum EIRP to be defined using RB allocation around 10~20MHz for the NTN Ku band VSAT as the starting point - Consider the RB allocation number to be suitable for DFT-s-OFDM at least

- Issue 1-3-1: EIS requirement Wayforward: - Consider calculate the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3) - At least 2dB implementation margin and will be taken into account. - At least the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle can be taken into account. - Companies are encourage to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting Wayforward: - Companies are encouraged to further check whether the OCNG Pattern is needed for the NTN EIS requirement in the next RAN4 meeting. Issue 1-3-2: Maximum input level requirement Agreement: - RAN4 will define the maximum input level requirement for the NTN Ku band VSAT.

- Discussion

  - Clarification on the minimum peak EIRP requirements

In the last RAN4 meeting, some companies expressed a strong need to allow VSAT to support LEO only in Rel.19. However, the existing VSAT types 4 and 5 currently cover both LEO and GSO. Therefore, RAN4 has a tentative agreement to consider a new type for LEO only, based on the same VSAT antenna size assumption as types 4 and 5, as a working assumption. Some companies are concerned that adding a new VSAT type would increase the workload. Others worry that not defining a new VSAT type supporting LEO only would misalign with the Ka band specification. In our view, companies have already specified the RF requirements for LEO and GSO separately, and adding a new UE type would not increase the workload. Therefore, we suggest defining a new VSAT type, type 6, which supports LEO only, to align with Ka band specifications.

Proposal 1 : RAN4 to consider defining a new VSAT type, type 6, which supports LEO only, to align with Ka band specifications.

  - Minimum peak EIRP

In the last RAN4 meeting, RAN4 reached tentative agreements for the minimum peak EIRP: [60] dBm for LEO and [70] dBm for GSO. Based on our evaluation of the uplink budget analysis from the table below, we support using these two values as the minimum peak EIRP for LEO and GSO, respectively.

Table 1: NTN Ku-band UL link budget analysis

|  | Ku band |  |  |
| --- | --- | --- | --- |
| Satellite type | GSO | LEO 1200 | LEO 600 |
| Elevation angle (VSAT) | 35 deg | 35 deg | 35 deg |
| Distance (VSAT-SAN) | 38178.5 km | 1830.6 km | 967.3 km |
| UL frequency | 14500MHz | 14500MHz | 14500MHz |
| Free space path loss (dB) | 207.3 | 180.9 | 175.4 |
| Atmosphere loss (dB) | 2 | 2 | 2 |
| BW (MHz) | 10 | 100 | 100 |
| target SNR | -1 | -1 | -1 |
| SAN noise figure (dB) | 3 | 3 | 3 |
| SAN Rx gain (dBi) | 44 | 38.5 | 38.5 |
| Implementation margin | 2 | 2 | 2 |
| minimum EIRP | 71.3 | 60.5 | 54.9 |

Proposal 2 : RAN4 to consider a minimum peak EIRP of 70 dBm for Ku band VSAT communicating with GSO.

Proposal 3 : RAN4 to consider a minimum peak EIRP of 60 dBm for Ku band VSAT communicating with LEO.

  - EIS requirement

In the last meeting, RAN4 have reached some consensus to derive EIS requirements which are listed below

1.the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3)

2. 2dB implementation margin

3.the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle

For the beam gain reduction, this highly depends on the implementation. Based on our evaluation, it would be in the range of 4 to 6 dB. Here, we take the worst case of 6 dB for the EIS analysis

From VSAT implementation perspective, the minimum EIS can be derived below.

EISmin = 10*log10(k)+10*log10(BW)+60+IM+SNR - G/T

G/T is a crucial parameter for calculating the EIS of the VSAT. For the electrically steered beam antenna array, based on our evaluation, the overall noise figure of the VSAT, which is 6 dB, can be considered. The peak antenna gain for the 45x45 VSAT is assumed to be 35.6 dBi. Therefore, the antenna gain is in the range of 29.6 to 35.6 dBi at elevations from 35 to 90 degrees. Consequently, the G/T of the 45x45 VSAT ranges from -120.8 to -126.8 dBm. The peak antenna gain for the 78x78 VSAT is assumed to be 40.3 dBi. Therefore, the antenna gain is in the range of 34.3 to 40.3 dBi at elevations from 35 to 90 degrees. Consequently, the G/T of the 78x78 VSAT ranges from -125.5 to -131.5 dBm.

For the mechanically steered beam, the 60 cm parabolic antenna VSAT is assumed to have a gain of 34.6 dBi. The overall noise figure is assumed to be 2.5 dB. Therefore, the G/T of the 60 cm parabolic antenna VSAT is -130.6 dBm.

Table 2: EIS analysis from VSAT implementation perspective

| VSAT configuration | 45*45 antenna array | 78*78 antenna array | 60cm parabolic antenna |
| --- | --- | --- | --- |
| Antenna gain (dBi) Elevation angle from 35 to 90 | 29.6~35.6 | 34.3~40.3 | 34.6 |
| G/T (dB/K) | 0.2~5.2 | 4.9~10.9 | 11 |
| EIS for 50MHz (dBm) | -120.8~-126.8 | -125.5~-131.5 | -130.6 |

The table above summarizes the parameters for EIS analysis for three types of VSATs from the perspective of VSAT implementation. For the 45x45 antenna array supporting LEO, considering the worst case, the EIS for 50 MHz is -120.8 dBm. Therefore, the EIS for 50 MHz CBW for LEO should be less than -120.8 dBm, and the EIS for 10 MHz CBW could be -120.8 + 10*log(10/50) = -127.8 dBm.

For the 78x78 antenna array and the 60 cm parabolic antenna supporting GSO, considering the worst case, the EIS for 50 MHz is -125.5 dBm. Therefore, the EIS for 50 MHz CBW for GSO should be less than -125.5 dBm, and the EIS for 10 MHz CBW could be -125.5 + 10*log(10/50) = -132.5 dBm.

Proposal 4: RAN4 to consider the EISREFSENS_50MHz should be less than [-120.8]dBm and the EISREFSENS_10MHz should be less than [-127.8]dBm for LEO.

Proposal 5: RAN4 to consider the EISREFSENS_50MHz should be less than [-125.5]dBm and the EISREFSENS_10MHz should be less than [-132.5]dBm for GSO.

- 3	Conclusion

In this contribution, we provide our views on the remaining issues in the last RAN4 meeting. The proposals are summarized as follows:

Proposal 1 : RAN4 to consider define a new VSAT type which is type 6 supporting LEO only to align with Ka band specifications.

Proposal 2 : RAN4 to consider a minimum peak EIRP of 70 dBm for Ku band VSAT communicating with GEO.

Proposal 3 : RAN4 to consider a minimum peak EIRP of 60 dBm for Ku band VSAT communicating with LEO.

Proposal 4: RAN4 to consider the EISREFSENS_50MHz should be less than [-120.8]dBm and the EISREFSENS_10MHz should be less than [-127.8]dBm for LEO.

Proposal 5: RAN4 to consider the EISREFSENS_50MHz should be less than [-125.5]dBm and the EISREFSENS_10MHz should be less than [-132.5]dBm for GSO.

- 4	References

- R4-2508778, “Meeting minutes for [115][314] NR_NTN_Ku_Band_UE_SAN_RF”, CHTTL, RAN4#115.