3GPP TSG-RAN WG4 Meeting #116																	R4-2510215

Bengaluru, India, 25thth to 29th August 2025

Agenda Item:	7.7.3

Source:	Ericsson

Title:	On co-existence simulation results for NR NTN Ku band

Document for:	Approval

# 1	Introduction

In 3GPP RAN#104, a new Work Item (WI) was approved for Introduction of Ku band for NR NTN. The WI was further updated in RAN#106 [1]. One of the objectives of the WI is to –

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

WF [2] captured agreements on co-existence simulation assumptions in the last meeting.

This contribution shares our pre-liminary simulation results post the calibration campaign during the summer and triggers the discussion points for open issues.

# 2	Discussion

## 2.1	Assumption of Clutter Loss in NTN propagation model

During the offline calibration phase for NTN, it was agreed to consider only basic path loss () which accounts for the signal's free space propagation, clutter loss, and shadow fading in the calibration assumptions. The attenuations due to atmospheric gasses and scintillation losses were assumed to be 0.

The basic path loss in dB is modeled as follows, taken from Clause 6.6.2 TR 38.811 –

where FSPLd, fc is the free space path loss, is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~SF~N0, σSF2. When the UE is in LOS condition, clutter loss is set 0 dB in the path loss model. However, in NLOS condition the Clutter loss depends on the elevation angle for a particular scenario. The corresponding values used in Ka-band co-existence studies are shared in Table below from TR 38.811 for Urban scenario –

Table 6.6.2-2: Shadow fading and clutter loss for urban scenario

| Elevation | S-band |  |  | Ka-band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS |  | LOS | NLOS |  |
|  | σSF (dB) | (dB) | (dB) | σSF (dB) | σSF (dB) | (dB) |
| 10° | 4 | 6 | 34.3 | 4 | 6 | 44.3 |
| 20° | 4 | 6 | 30.9 | 4 | 6 | 39.9 |
| 30° | 4 | 6 | 29.0 | 4 | 6 | 37.5 |
| 40° | 4 | 6 | 27.7 | 4 | 6 | 35.8 |
| 50° | 4 | 6 | 26.8 | 4 | 6 | 34.6 |
| 60° | 4 | 6 | 26.2 | 4 | 6 | 33.8 |
| 70° | 4 | 6 | 25.8 | 4 | 6 | 33.3 |
| 80° | 4 | 6 | 25.5 | 4 | 6 | 33.0 |
| 90° | 4 | 6 | 25.5 | 4 | 6 | 32.9 |

It should be noted that corresponding values are not available for Ku band. Based on the LOS probability in Clause 6.6.1 TR 38.811 for Urban scenario, it is observed that at 35 degrees elevation angle the LOS probability is around 50%. This means the clutter loss assumption will play a significant role in NLOS condition. However, at 90 degrees the LOS probability is 99.2% for Urban scenario meaning close to negligible impact from the Clutter Loss assumption.

The clutter loss assumption plays significant role in the path loss calculations at 35 degrees elevation angle assumption.

However, some companies proposed to drop the clutter loss assumption at 35 degrees elevation angle, stating very low value SINR at NLOS condition which would be difficult to derive the ACIR requirements at 5%ile throughput loss. In our view similar model was also used for Ka band and there are scenarios where the ACIR requirements were derived on average through loss results. Moreover, if the link budget fails to work at 35 degrees or lower elevation angles, there are several ways to achieve the 5%ile through requirements without changing the propagation model. Similar assumption was also noted in Ka band co-existence studies.

If the link budget fails at 35 degrees or lower elevation angles, there are several other ways to make it work without changing the propagation model.

RAN4 to use the Ka band clutter loss reference value at 35 degrees elevation angle in Urban scenario for co-existence simulation assumptions.

## 2.2	Co-existence results with VSAT parabolic and phased array antenna assumptions

Among the prioritized scenarios from the last WF [2] highlighted in yellow, the following table summarizes the results for NTN GEO and LEO600 scenarios with VSAT parabolic and phased array 45x45 antenna assumptions:

Table 2.2-1 Required ACIR at 5%-ile for GEO NTN scenarios

| GEO |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ACIR |  |  |  |
|  |  |  |  |  | 90 degrees |  | 35 degrees |  |
| Scenarios | Aggressor | Victim | Contributing | Frequency | Parabolic | Phased array 45x45 | Parabolic | Phased array 45x45 |
| 1 (Prioritized) | NTN GEO UL | TN UL | NTN VSAT ACLR | 14 GHz | ~0 | 9 | 3 | 16 |
| 2 | TN UL | NTN VSAT UL | NTN VSAT ACS | 14 GHz | 10 | 37 | 6 (avg) | 33 (avg) |
| 3 | NTN VSAT UL | TN DL | NTN VSAT ACLR | 14 GHz | ~0 | ~0 | ~0 | ~0 |
| 4 (Prioritized) | TN DL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | ~0 | 14 | 10 (avg) | 11 (avg) |
| 5 (Prioritized) | TN DL | NTN GEO DL | NTN VSAT ACS | 11 GHz | 32 | 42 | 20 (avg) | 30 (avg) |
| 6 (Prioritized) | NTN GEO DL | TN DL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 | ~0 | ~0 |
| 7 | NTN GEO DL | TN UL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 | ~0 | ~0 |
| 8 | TN UL | NTN GEO DL | NTN SAN GEO ACS | 11 GHz | 25 | 25 | 17 (avg) | 17 (avg) |

For GEO Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS.

For GEO Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8.

For GEO Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage.

For GEO Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE.

Table 2.2-2 Required ACIR at 5%-ile for LEO600 NTN scenarios

| LEO600 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ACIR |  |  |  |
|  |  |  |  |  | 90 degrees |  | 35 degrees |  |
| Scenarios | Aggressor | Victim | Contributing | Frequency | Parabolic | Phased array 45x45 | Parabolic | Phased array 45x45 |
| 1 | NTN LEO UL | TN UL | NTN VSAT ACLR | 14 GHz | ~0 | ~0 | ~0 | 6 |
| 2 | TN UL | NTN VSAT UL | NTN VSAT ACS | 14 GHz | 18 | 43 | 9 (avg) | 33 (avg) |
| 3 | NTN VSAT UL | TN DL | NTN VSAT ACLR | 14 GHz | ~0 | ~0 | ~0 | ~0 |
| 4 | TN DL | NTN LEO UL | NTN SAN LEO ACS | 14 GHz | 11 | 12 | 13(avg) | 12 (avg) |
| 5 | TN DL | NTN GEO DL | NTN VSAT ACS | 11 GHz | 29 | 38 | 23 (avg) | 33 (avg) |
| 6 | NTN LEO DL | TN DL | NTN SAN LEO ACLR | 11 GHz | ~0 | ~0 | ~0 | ~0 |
| 7 | NTN LEO DL | TN UL | NTN SAN LEO ACLR | 11 GHz | ~0 | ~0 | ~0 | ~0 |
| 8 | TN UL | NTN LEO DL | NTN SAN LEO ACS | 11 GHz | 29 | 28 | 17 (avg) | 18 (avg) |

For LEO600 Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS.

For LEO600 Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8.

For LEO600 Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage.

For LEO600 Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE.

In the simulation results, Scenario 5 for VSAT ACS derivation for both GEO and LEO600 represents the most challenging case. In Scenario 5, the ACIR values are higher than TN BS ACLR of 37 dB for VSAT phased array antennas. This would require further discussion in RAN4 to ensure co-existence between NTN and TN and how to evaluate results where 5%ile throughput cannot be achieved.

Scenarios 5 represents the most challenging case to derive NTN-VSAT ACS and needs further discussion within RAN4.

Based on the results presented in Table 2.2-1 for GEO and Table 2.2-2 for LEO600, Table 2.2-4 summarizes the SAN-VSAT ACLR ACS co-existence results suggested values assuming the following TN-UE ACLR ACS values in Table 2.2-3 (referenced from IMT studies in TR 38.921 for 10 GHz and TR 38.922 for 15 GHz) -

Table 2.2-3 ACLR and ACS of TN

|  | TN |  | Values |
| --- | --- | --- | --- |
| 11 GHz (IMT studies for 10 GHz from TR 38.921) | BS | ACLR | 37 dB |
|  |  | ACS | 40 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 31 dB |
| 14 GHz (IMT studies for 15 GHz from TR 38.922) | BS | ACLR | 30 dB |
|  |  | ACS | 30 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 24 dB |

Table 2.2-4: Co-existence results suggested ACLR and ACS of NR-NTN for the worst-case 35 degrees elevation angle

| Scenario | Contributing | ACLR ACS values |  |
| --- | --- | --- | --- |
|  |  | Parabolic | Phased array 45x45 |
| 1 | NTN VSAT ACLR | GEO: 2.9 dBc LEO600: 0 dBc | GEO: 15.3 dBc LEO600: 6.02 dBc |
| 2 | NTN VSAT ACS | GEO: 6.06 dBc LEO600: 9.1 dBc | - |
| 3 | NTN VSAT ACLR | 0 dBc | 0 dBc |
| 4 | NTN SAN ACS | GEO: 10.04 dBc LEO600: 13.08 dBc | GEO: 11.05 dBc LEO600: 12.06 dBc |
| 5 | NTN VSAT ACS | GEO: 20.08 dBc LEO600: 23.1 dBc | GEO: 30.96 dBc LEO600: - |
| 6 | NTN SAN ACLR | 0 dBc | 0 dBc |
| 7 | NTN SAN ACLR | 0 dBc | 0 dBc |
| 8 | NTN SAN ACS | GEO: 17.9 dBc LEO600: 17.9 dBc | GEO: 17.9 dBc LEO600: 19.2 dBc |

Based on the summarized co-existence values, following SAN-VSAT ACLR ACS values are proposed in Table 2.2-5 -

Table 2.2-5: ACLR and ACS of NR-NTN

|  | SAN |  | VSAT |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Parabolic | Phased array 45x45 |
| ACLR (dBc) | 18 | 18 | 16 | 16 |
| ACS (dBc) | 18 | 20 | 25 | 31 |

Based on the simulation results, the SAN-VSAT ACLR ACS values highlighted in Table 2.2-5 can be considered for co-existence between TN and NTN.

## 2.3	Simulation assumptions in Annex

Based on the agreements so far in the WF, the simulation assumptions have been consolidated and captured in the Annex below.

RAN4 to treat the co-existence simulation assumptions for Ku-band provided in the Annex, as a running document.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The clutter loss assumption plays significant role in the path loss calculations at 35 degrees elevation angle assumption.

Observation 2	If the link budget fails at 35 degrees or lower elevation angles, there are several other ways to make it work without changing the propagation model.

Observation 3	For GEO Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS.

Observation 4	For GEO Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8.

Observation 5	For GEO Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage.

Observation 6	For GEO Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE.

Observation 7	For LEO600 Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS.

Observation 8	For LEO600 Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8.

Observation 9	For LEO600 Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage.

Observation 10	For LEO600 Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE.

Observation 11	Scenarios 5 represents the most challenging case to derive NTN-VSAT ACS and needs further discussion within RAN4.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to use the Ka band clutter loss reference value at 35 degrees elevation angle in Urban scenario for co-existence simulation assumptions.

Proposal 2	Based on the simulation results, the SAN-VSAT ACLR ACS values highlighted in Table 2.2-5 can be considered for co-existence between TN and NTN.

Proposal 3	RAN4 to treat the co-existence simulation assumptions for Ku-band provided in the Annex, as a running document.

# References

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2508649, “WF for Ku band”, 3GPP RAN4#115

# 6a		Annex: Simulation assumptions

## 6a.1	Co-existence simulation scenario for 11/14GHz

Scenarios for the coexistence study for 11/14GHz are listed in Table 6a.1-1

Table 6a.1-1: Scenarios for NTN-NTN/TN co-existence

| 11/14GHz |  | NTN |  |  |
| --- | --- | --- | --- | --- |
|  |  | GEO | LEO 600km | LEO 1200km |
| NR | Rural | N/A | N/A | N/A |
|  | Urban macro | X | X | X |
|  | Dense Urban | N/A | N/A | N/A |

The aggressor and victim combinations for 11/14GHz cases are listed in Table 6a.1-2.

Table 6a.1-2: Aggressor and victim

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

The frequency and bandwidth are listed in Table 6a.1-3.

Table 6a.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Urban macro | 11/14 GHz | 100MHz | TDD | 1 |
| GEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |
| LEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |

## 6a.2	Co-existence simulation assumption for 11/14GHz

### 6a.2.1	Network layout model

#### 6a.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN and TN network layout.

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z)

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6a.2.1.1-1.

Table 6a.2.1.1-1: Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1a | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 2a | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN | Only the active TN cells in central NTN beam |
| 3a | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 4a | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5a | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s) |
| 6a | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO, interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7a | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 8a | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN | All active TN cells in central NTN beam |

### 6a.2.2	System parameters

#### 6a.2.2.1	Satellite parameters

Set-1 satellite parameters are listed in Table 6a.2.2.1-2 according to TR 38.821 [6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6a.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration Ku-band | NRB (100MHz BW) |
| --- | --- |
| SCS 30 kHz | 273 |

Table 6a.2.2.1-2: Satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 4 m | 0.714 m |
| G/T |  | 16.5 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 44 dBi | 38.5 dBi |

Table 6a.2.2.1-3: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 14GHz(UL) / 11GHz(DL) |  |
| The number of active UE (UL) | 10 UEs and 27RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6a.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 6a.2.2.1-4 |  |
| Handover margin | 3dB |  |

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 |
| --- | --- | --- |
| G/T (dB K-1) | 16.5 | 10.9 |
| NF (dB) | 3 | 3 |

#### 6a.2.2.2	NTN UE parameters

The NTN UE antenna characteristics are given in Table 6a.2.2.2-1 and other NTN UE parameters are given in Table 6a.2.2.2-2.

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| 1 | VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921 | TR 38.921 | Circular Aperture TR 38.863 |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 | 3.5 | N/A |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis | N/A |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB | N/A |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements | 78x78 elements | 60 cm diameter aperture |
| 1.7 | Number of supported polarizations, P | 1 | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda | N/A |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 | N/A |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.9 | 9.1 | N/A |
| 1.11 | Tx Transmit power | 10 Watts for GEO and for LEO | 50 Watt for GEO | 10 Watts for GEO and for LEO |
| 1.12 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees | 0~360 degrees |
| 1.13 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees | 0~60 degrees |
| 1.14 | Normal direction | Toward X+ axis | Toward X+ axis | Toward X+ axis |
| 1.15 | Max EIRP Value | 76.6 dBm | 88.2 dBm | 76.6 dBm |
| 1.16 | Noise Figure | 6 dB | 6.0 dB for GEO | 2.5 dB for both LEO and GEO |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |  |  |

The altitudes of NTN UE for different use case are defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-2: NTN UE altitude

| Use case | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- |
| Altitude | 22.5m | 3-14km | 1.5m |
| Note 1: Co-existence evaluation is only limited to L-ESIM. |  |  |  |

#### 6a.2.2.3	TN parameters

TN parameters for co-existence study are given in Table 6a.2.2.3-1 and Table 6a.2.2.3-2.

Table 6a.2.2.3-1: Simulation assumptions of TN NR

| Carrier frequency in GHz | 11 and 14 |
| --- | --- |
| Size of each nominal channel BW in MHz | 100 |
| Transmission bandwidth in MHz | 98.28 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 450m |
| UE number per cell | 3 |
| System loading and activity | 20% |
| Network location | See Table 6a.2.1.1-2 |
| DL subcarrier spacing | 30kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 6a.2.6.2 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| BS height | 25 |
| UE antenna height in meter | 1.5 |
| UE TX power in dBm | -40 to 23 |
| Building penetration loss | 3GPP TR 38.803 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 6a.2.9 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 6a.2.8 |

Table 6a.2.2.3-2: ACLR/ACS for TN (11 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 37 dB |
|  | ACS | 40 dB |
| UE | ACLR | 24 dB |
|  | ACS | 31 dB |

Table 6a.2.2.3-3: ACLR/ACS for TN (14 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 30 dB |
|  | ACS | 30 dB |
| UE | ACLR | 24 dB |
|  | ACS | 24 dB |

### 6a.2.3	Antenna and beam forming pattern modelling

#### 6a.2.3.1	Satellite parameters

In Rel-18, antenna pattern in section 6.4.1 of TR38.811 is reused for NTN SAN and NTN UE. Use of phased array antenna model needs further discussion.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθπ180kasinθπ1802        for 0<θ≤90°

For UE, 4J1kasinπ2kasinπ22          for θ>90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’;

-	ais the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin() function is in radian.

The beam layout definition for a single satellite simulation in Ka-Band is defined in Table 6a.2.3.1-1.

Table 6a.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Same with Table 6.2.3.1-1 |
| --- | --- |
| Number of beams | Same with Table 6.2.3.1-1 |
| UV plane illustration (extracted from [19]) | Same with Table 6.2.3.1-1 |
| UV plane convention | Same with Table 6.2.3.1-1 |
| Adjacent beam spacing on UV plane | Same with Table 6.2.3.1-1 |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 35° for GEO and LEO |
| Frequency-reuse factor | FRF=2 Do not consider interference leakage from adjacent beams as starting point for co-ex study. For example for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam. |
| Polarization re-use | Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor |
| UE distribution | Same with Table 6.2.3.1-1 |
| UE configuration | Mobile VSAT for GSO and NGSO |
| UE orientation | Random Ideal Tracking serving beam |
| UE attachment | RSRP |

#### 6a.2.3.2	TN BS antenna and beam forming pattern modelling

The parameters in Table 6a.2.3.2-1 are used for BS antenna pattern in the coexistence study.

Table 6a.2.3.2-1: TN BS antenna modelling for Urban macro scenario for 11 GHz

| Parameters | Values for 11 GHz |
| --- | --- |
| Scenario | Urban macro |
| Antenna pattern | AAS model see below (from TR 38.922) |
| Element gain GE,max (dBi) | 6.4 |
| Horizontal 3dB /vertical 3dB beam width of single element | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 8x16 |
| Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) (Note 6) | 3 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 6 |

Table 6a.2.3.2-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

Table 6a.2.3.2-3: TN BS antenna modelling for Urban macro scenario for 14 GHz

| Parameters | Values for 14GHz |
| --- | --- |
| Scenario | Urban macro |
| Antenna pattern | Extended AAS model see below (from TR 38.922) |
| Element gain GE,max (dBi) | 6.4 |
| Horizontal 3dB /vertical 3dB beam width of single element | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 24 elements |
| Horizontal/Vertical radiating element spacing | H = 0.5 λ V = 2.8λ |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 17.14 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 |
| Mechanical down-tilt (degrees) | 6 |

Table 6a.2.3.2-4: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

#### 6a.2.3.3	TN UE antenna model

The UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

### 6a.2.4	ACIR model

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6a.2.2.1-3 and Table 6a.2.2.4-1 respectively.

### 6a.2.5	Propagation model

Propagation models between NTN and TN components used for the coexistence study are listed in Table 6a.2.5-1.

Table 6a.2.5-1: Propogation models between NTN and TN

| Link | Propagation model |
| --- | --- |
| TN BS to VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

### 6a.2.6	Transmission power control model

#### 6a.2.6.1	UL TPC

Following UL power control model for both NTN and TN are used as starting point.

Where,

-	Pmax = maximum UE Tx power, i.e. 23 dBm for TN UE;

-	Rmin = minimum power reduction ratio, i.e. -63 dB for TN UE, and [-63 dB by assuming NTN UE min Tx power as -30dBm as starting point] for NTN UE;

-	CLx-ile and γ are set as following:

-	γ = 1;

-	CLx-ile = –SNR_target + Pmax – Thermal Noise – BS/SAN_NoiseFigure - 10*log10(BW), considering SNR_target is 15dB and BW is actual UL BW.

#### 6a.2.6.2	DL TPC

For downlink scenario, no power control scheme is applied.

### 6a.2.7	Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

### 6a.2.8	Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

### 6a.2.9	Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

## 6a.3	Co-existence simulation methodology for 11/14GHz

### 6a.3.1	Simulation procedure

See Section 6.3.1 of TR 38.863, noting the FRF and network deployment for NTN and TN in above 10GHz are specified accordingly.

Following approaches for the deployment of NTN UE and satellite are used:

-	The satellite should be generated in the visible area/sky of NTN UE;

-	NTN UEs point to the satellite accurately;

-	The position of the satellite should guarantee that NTN UE vertical angle towards the satellite;

-	For calibration, use 35 degree instead of the range;

-	SAN elevation angle value, 35, & 90 degrees (computed from the centre of the beam), for co-existence -simulation.

-	Horizontal angle of NTN UEs should be calculated based on the satellite position;

-	Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.

### 6a.3.2	Methods and principle to process co-existence simulation results

In order to process the co-existence simulation results received for all different scenarios and assumptions, the following steps are adopted:

-	Step 1: Discuss and agree on the most stringent scenario(s) for each scenario (Scenario 1, 2, 3…,8);

-	Step 2: Discuss and determine the required ACIR from results of the most stringent case(s) for each scenario;

-	Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each scenario

It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case.

Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario:

-	If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

-	If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

# C.1	Calibration assumptions at 11/14 GHz

Assumptions in Section 6a.2 are adopted as the baseline for calibration. It should be noted there are different parts which are listed in Table C.1-1 and Table C.1-2.

Table C.1-1: NTN Assumptions for calibration

| Propagation model 38.811[5] considerations | Basic path loss | Yes (including Clutter Loss from Table 6.6.2-1 TR 38.811) |
| --- | --- | --- |
|  | Atmospheric loss | 0 |
|  | Ionospheric or scintillation loss | 0 |
|  | O2I / building-entry loss | N/A |
| NTN SINR | SINR statistics target | Central beam (UL/DL) |
|  | Interference | Co-channel interference from 6 adjacent beams |
|  | BW / #UE | 100MHz / 1 DL, 10UL |
|  | Polarization gain with 3dB | not considered |
|  | Elevation angle | 35 degrees for GEO and LEO |
|  | Scenario | Urban |
| VSAT phased array | Antenna characteristics | 45x45 antenna elements - Refer to Table 6a.2.2.2-1 |
|  | VSAT NF | 6 dB |

Table C.1-2: TN Assumptions for calibration

| Calibration assumptions |  |  |
| --- | --- | --- |
| TN AAS BS | Antenna characteristics | Refer to Table 6a.2.3.2-1 and Table 6a.2.3.2-2 for 11 GHz Refer to Table 6a.2.3.2-3 and Table 6a.2.3.2-4 for 14 GHz |
|  | Scenario | Urban |
|  |  | #UE: 1 DL/ 3 UL |
|  |  | 100% Outdoor |
|  |  | ISD 0.45 KM |
| TN UE | Antenna characteristics | Omni-directional radiation pattern with 0 dBi gain |