3GPP TSG-RAN WG4 Meeting # 116 	R4-2510246

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on the AIoT system parameter

Agenda Item:	7.22.2

Document for:	Approval

- Introduction

D2R bandwidth

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

R2D channel raster

-	Define 10KHz channel raster

-	FFS on channel raster offset

For the system parameter of AIoT, the agreements in the last meeting are listed below:

In this contribution, we provide our views on remaining issues.

- Discussion

  - D2R bandwidth

Normally the channel bandwidth indicates the maximum frequency resource that UE can use and it also related to the peak data rate that UE can achieve. As we agreed in the last meeting, the D2R channel bandwidth is expressed as equation and related to the R value. In our understanding, it is not reasonable to define D2R channel bandwidth based on all R value defined in RAN1, since only R=1 can reflect same intention as NR UE channel bandwidth.

Observation 1: The channel bandwidth typically indicates the maximum frequency resource that UE can used and related to peak data rate, and in AIoT only minimum R of each Tc align with such intention.

In RAN1, the Tb and R are agreed as table below:

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 |  |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 |  |
| 1.39 |  |  |  |  |  |  |  |  | R=1 |

Under same Tc, D2R CBW is different for each R value since the Tc determine the center frequency of D2R but the transmission bandwidth changed with R value. If the D2R CBW is only defined for the minimum R value, the spectrum resource could be wasted in some configurations, so it is better to list all D2R CBW of possible RAN1 configuration.

Observation 2: The D2R channel bandwidth under same Tc is still different for each R value, and only define the D2R CBW with minimum R value may restrict the flexibility of configuration and cause unnecessary waste of spectrum resources.

The Tb/Tc/R are the terminologies in RAN1 and directly use them in RAN4 may cause ambiguity or additional efforts on the explanation. In fact, the Tb determine the transmission bandwidth and the Tc determine the center frequency offset, and we can use these two concepts directly to define the D2R CBW rather than use RAN1 term.

Proposal 1: The D2R channel bandwidth for device is translated as the relationship with center frequency and transmission bandwidth rather than R/Tb/Tc, as listed below:

| D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 16.5 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  |
| 30 |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

Similar to NR, the guard band can also be considered listed independently.

Proposal 2: The guard band for each D2R CBW can also be considered to be listed in the sepc:

| D2R guard band (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 1.5 | 2.25 | 3.75 | 6.75 | 12.75 | 24.75 | 48.75 | 96.75 |  |
| 30 |  | 3 | 4.5 | 7.5 | 13.5 | 25.5 | 49.5 | 97.5 |  |
| 60 |  |  | 6 | 9 | 15 | 27 | 51 | 99 |  |
| 120 |  |  |  | 12 | 18 | 30 | 54 | 102 |  |
| 240 |  |  |  |  | 24 | 36 | 60 | 108 |  |
| 480 |  |  |  |  |  | 48 | 72 | 120 |  |
| 960 |  |  |  |  |  |  | 96 | 144 |  |
| 2880 |  |  |  |  |  |  |  |  | 288 |

- Conclusion

In this contribution, we provide our view on device D2R channel bandwidth definition.

Observation 1: The channel bandwidth typically indicates the maximum frequency resource that UE can used and related to peak data rate, and in AIoT only minimum R of each Tc align with such intention.

Observation 2: The D2R channel bandwidth under same Tc is still different for each R value, and only define the D2R CBW with minimum R value may restrict the flexibility of configuration and cause unnecessary waste of spectrum resources.

Proposal 1: The D2R channel bandwidth for device is translated as the relationship with center frequency and transmission bandwidth rather than R/Tb/Tc, as listed below:

| D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 16.5 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  |
| 30 |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

Proposal 2: The guard band for each D2R CBW for device can also be considered to be listed in the sepc:

| D2R guard band (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 1.5 | 2.25 | 3.75 | 6.75 | 12.75 | 24.75 | 48.75 | 96.75 |  |
| 30 |  | 3 | 4.5 | 7.5 | 13.5 | 25.5 | 49.5 | 97.5 |  |
| 60 |  |  | 6 | 9 | 15 | 27 | 51 | 99 |  |
| 120 |  |  |  | 12 | 18 | 30 | 54 | 102 |  |
| 240 |  |  |  |  | 24 | 36 | 60 | 108 |  |
| 480 |  |  |  |  |  | 48 | 72 | 120 |  |
| 960 |  |  |  |  |  |  | 96 | 144 |  |
| 2880 |  |  |  |  |  |  |  |  | 288 |

- References

- R4-2508116, WF on A-IOT device requirements, CMCC