3GPP TSG-RAN WG4 Meeting #116	R4-2510256

Bengaluru, India, 25th – 29th August 2025

Title:	Reply LS on FR2 Beam Correspondence in Initial Access

Response to:	R5-253653

Release:	Rel-18

Work Item:	NR_RF_FR2_req_Ph3-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:	-

Contact Person:

Name:	Hao DU

Tel. Number:

E-mail Address:	duhao.txyjy@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R4-2315864

1. Overall Description:

RAN4 thanks RAN5 for the LS on FR2 BC in initial access and the clarifications are provided below:

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer: RAN4 define the requirements of BC in initial access based on assuming that after autonomously beam selection in IDLE mode, the uplink beam will not change during whole test procedure.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

- Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: Beamlock function should be performed after beam selection is finished and does not prevent UE to autonomously choose uplink beam. RAN4 requirements are only applied to the first selected corresponding uplink beam after beam selection.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: RAN4 sent LS to RAN5 check the feasibility of Beamlock Function in IDLE mode in R18 and define the requirements for BC in initial access based on the assumption that beam lock function is feasible and can be used.

2. Actions:

To RAN5 group.

ACTION:

RAN4 respectfully ask RAN5 to take the clarification above into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting#116bis	13th – 17th October 2025	Prague, CZ

TSG-RAN4 Meeting#117		17th – 21st November 2025	Dallas, US