3GPP TSG-RAN WG4 Meeting #116	R4-2510358

# Bengaluru, India, August 25th – 29th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	7.7.5

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from UE RF perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

  - 2.1 RB configuration for EIRP requirement

- Issue 1-2-4: RB configuration for EIRP requirement Agreement: - Consider both minimum peak & maximum EIRP to be defined using RB allocation around 10~20MHz for the NTN Ku band VSAT as the starting point - Consider the RB allocation number to be suitable for DFT-s-OFDM at least

Regarding RB allocation for EIRP requirement, it was defined only for max EIRP for Ka band as follows. From our understanding, 13RBs is based on the Ka band coexistence simulation assumption. In coexistence simulation assumption, it assumes system bandwidth is 200MHz with 120kHz SCS, the number of active UE is 10 and per UE is 13RBs allocation with 120kHz SCS.

However, in Ku band coexistence simulation assumption [3], it assumes that system bandwidth is 100MHz with 30kHz SCS, 10 UEs and 27RBs per UE, which is shown in the following tables. Hence, for Ku band, we propose to define RB configuration only for max EIRP requirement and using RB allocation 27RBs with 30kHz for band n248 and n247.

For Ku band n509 and n508, we can also assume the system bandwidth is 100MHz with 120kHz SCS and 10UEs in UL. Hence, RB configuration for band n509 and n508 can be defined as 6RBs.

Proposal 1: It is proposed to define RB configuration only for max EIRP requirement and using RB allocation 27RBs with 30kHz for band n248 and n247, 6RBs with 120KHz for band n509 and n508.

  - 2.2 OTA reference sensitivity level

- Issue 1-3-1: EIS requirement Wayforward: - Consider calculate the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3) - At least 2dB implementation margin and will be taken into account. - At least the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle can be taken into account. - Companies are encourage to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting Wayforward: - Companies are encouraged to further check whether the OCNG Pattern is needed for the NTN EIS requirement in the next RAN4 meeting.

For EIS calculation, according to TR 38.863, there are two approaches to define EIS requirement for Ka band. One is based on DL budget calculation assuming the worst case with 30 degree elevation angle. If we use the same approach to define EIS requirement for Ku band and assume 35 degree elevation angle as the worst case, which is used in Ku band coexistence simulation, the DL budget calculation is shown below to get the DL received signal at OTA (effective isotropic received signal) for Ku band. In this approach, EIS50MHz requirement should be less than -118dBm for GSO and -121dBm for LEO600.

Table 2.2-1: The minimum EIS calculation

|  | GSO |  | LEO600 |  |
| --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 172.98 | 168.83 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 |
| SNR [target SNR] | -1.00 | -1.00 | -1.00 | -1.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN EIRP density [dBW/MHz] | 43 | 43 | 7 | 7 |
| SAN_Tx EIRP [dBm] | 86.50 | 90.00 | 50.50 | 54.00 |
| VSAT DL Received signal level at OTA [dBm] | -118.40 | -114.34 | -122.48 | -114.83 |

Another approach is assuming G/T value, and using the following equation to calculate the EIS requirement.

EIS=10*log10(k)+30+10*log10(BW)+SNR+IM-G/T

BW is channel bandwidth. (candidate value: 50MHz)

IM is implementation margin. 2dB can be assumed.

SNR is the required SNR for demodulation. -1dB can be assumed.

K: is Boltzmann constant, 1.38*10-23 J/K

For G/T value, we consult some VSAT datasheets, e.g. HL1100W Hughes LEO Terminal and Intellian OW50M-Rac/OW70M-Rac served by LEO only, RESA M Ku served by GEO and LEO, which is shown in the following figures. It can be observed that G/T is larger than 9dB for LEO and 11dB for GEO.

Figure 2.2-1: datasheet for HL1100W Hughes LEO Terminal

Figure 2.2-2: datasheet for Intellian OW50M-Rac/OW70M-Rac

Figure 2.2-3: datasheet for RESA M Ku

To calculate the EIS requirement, we can assume G/T is 9dB for LEO600 and 11dB for GSO. Considering 3.5dB beam gain reduction at the 30~35 degree elevation angle, EISREFSENS_50MHz is -126dBm for LEO600 and -128dBm for GSO.

Based on the above analysis, the first approach is mainly dependent on SAN Tx EIRP. But, in actual, SAN transmit power is declared by the manufacturer. We prefer the second way to define EIS requirement based on VSAT G/T value. Hence, EISREFSENS_50MHz should be less than as -126dBm for mobile VSAT served by LEO only and -128 dBm for mobile VSAT served by GSO and LEO, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for band n248 n247, n509 and n508.

Proposal 2: EISREFSENS_50MHz should be less than as -126dBm for mobile VSAT served by LEO only and -128 dBm for mobile VSAT served by GSO and LEO, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for band n248 n247, n509 and n508.

  - 2.3 Maximum input level

- Issue 1-3-2: Maximum input level requirement Agreement: - RAN4 will define the maximum input level requirement for the NTN Ku band VSAT.

For the maximum input level, we also can use same way as Ka band to derive the requirement, which is calculated based on the link budget. The calculation for the maximum input level is shown in the following table. The parameters used in the table is based on the coexistence simulation assumptions.

Table 2.3-1: The maximum input level calculation

|  | GSO |  | LEO600 |  |
| --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 172.98 | 168.83 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 |
| SAN EIRP density [dBW/MHz] | 43 | 43 | 7 | 7 |
| SAN_TxPower [dBm] | 87.00 | 90.00 | 51.00 | 54.00 |
| MaxInputPower | -119.92 | -116.35 | -123.99 | -116.84 |

Proposal 3: To consider the maximum input level -116 dBm for all channel bandwidths for Ku bands.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to define RB configuration only for max EIRP requirement and using RB allocation 27RBs with 30kHz for band n248 and n247, 6RBs with 120KHz for band n509 and n508.

Proposal 2: EISREFSENS_50MHz should be less than as -126dBm for mobile VSAT served by LEO only and -128 dBm for mobile VSAT served by GSO and LEO, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for band n248 n247, n509 and n508.

Proposal 3: To consider the maximum input level -116 dBm for all channel bandwidths for Ku bands.

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2508778, Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2508774, Ku band coexistence simulation assumptions running document, Ericsson