3GPP TSG-RAN WG4 Meeting #116	R4-2510395

Bangalore, India, 25th – 29th August 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR NTN less than 5MHz SAN demodulation requirements

Agenda item:	7.6.4.6

Document for:	Discussion

# Introduction

Agreement:

Issue 1-2: PUCCH – Scope of the SAN demodulation requirements

- Define PUCCH format 2 cases for SAN

Issue 1-3: Simulation assumption for SAN requirements

- The following simulation assumption can be considered for evaluation SAN PUCCH performance with less than 5MHz

- A note can be added into the table to capture the value of ‘First PRB after frequency hopping’, which is 3.

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

In RAN#106, a revised WID [1] on NR NTN less than 5MHz was approved for Release 19. In RAN4#115 meeting, companies agreed to define SAN demodulation requirements for PUCCH format 2 [2].

Agreement:

Issue 1-5: Applicability rules for PUCCH requirements

- Introduce the following test applicability rule in TS 38.181:

For a SAN declaring to support less than 5MHz (see D.XX in table 4.6-1) a UCI BLER performance test with PUCCH Format 2 shall be tested in 3MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 can be skipped if applicable.

Open Issue:

Issue 1-6: Applicability rules for PRACH requirements

Options:

- Option 1: Introduce applicability rules for PRACH

  - Proposal 1a:

For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

- Option 2: No applicability rule is needed for PRACH

The only remaining open issue is for applicability rules for PRACH requirements, which is discussed in this contribution.

# SAN Demodulation Requirements

2.1 Applicability rules for PRACH requirements

Unless otherwise stated, for the subcarrier spacing to be tested, the test requirements shall apply only for anyone channel bandwidth declared to be supported (see D.7 in table 4.6-1).

The test applicability rules for PRACH in TS 38.181 is defined as follows:

Further, the current minimum requirements are defined for short RACH preamble formats with 15kHz and 30KHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839. Since there, are no SAN requirements for test preambles with LRA = 1151 and LRA = 571(as in terrestrial networks), therefore it not required to update the current PRACH applicability rules.

Proposal 1: No need to modify the current applicability rule for PRACH performance requirements SAN conformance testing.

# Conclusion

Based on the above discussion, the following proposal is captured:

Proposal 1: No need to modify the current applicability rule for PRACH performance requirements SAN conformance testing.

# References

[1] RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, 3GPP TSG RAN Meeting #106

[2] R4-2508725, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#115, Malta.