3GPP TSG-RAN WG4 Meeting #116	R4-2510396

# Bangalore, India 25th – 29th August 2025

Source:	Ericsson

Title:	Simulations on NR NTN less than 5MHz SAN demodulation requirements

Agenda item:	7.6.4.6

Document for:	Information

# Introduction

In RAN4#115 meeting, the companies agreed to define requirements for PUCCH format 2. Based on WF [1], we re-submit our simulations results.

# Simulations

# 2.1 Simulation assumptions and results for PUCCH format 2 UCI BLER performance requirements

Table 2.1-1: Test Parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) (Note 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Note 1: For 3MHz, the first PRB after frequency hopping is 3. |  |

Table 2.1-2: Minimum requirements for PUCCH format 2, 15 kHz SCS and 3/5 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex D) | SNR (dB) for 3 MHz CBW |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 8.42 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 0.79 |

# Conclusions

The simulations results are delivered for SAN PUCCH format 2 UCI BLER requirements.

# References

[1]   R4-2508725, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#115, Malta.