3GPP TSG-RAN WG4 Meeting # 116                                    R4-2510449

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.25.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for event triggered L1 reporting.

Discussion

According to the WID, RAN4 should discuss the impact on RRM requirements depends on the RAN1/2 specification introduced for Event triggered L1 measurement reporting. In the pervious meetings, we have reached consensus on Event-triggered L1 reporting delay requirement and accuracy requirement from high level and detail should be discussed.

The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- Other potential impact of event-triggered L1 reporting

Below, we will express our views on the above issues.

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

For this issue, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to introduce or update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133, which is similar as legacy event triggered L3 measurement reporting. It’s commom understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. Base on this understanding, Event triggering L1 reporting for LTM maybe not support if there is no update on requirements for reporting criteria per measurement category. In addition, the number of events to be supported by UE for LTM shall be discussed in RAN1 first.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS 38.133 need to be updated to cover event triggered L1 measurement reporting.

As for how to define relevant capabilities to support of event triggering L1 measurement reporting, there are two intuitive methods. As mentioned by some companies, compare to leagcy capability for events triggered measurement reporting, same or separate capability can be considered for supported number of events triggered L1 measurement reporting. If using same capability as legacy, Ecat. should be updated to cove L3 and L1 events due to the number of events is too limited. In addition, the number of events to be supported by UE for LTM shall be discussed in RAN1 first. To avoid affecting L3 measurement reports, we prefer to define a separate capability for the number of L1 measurement reports triggered by supported events.

proposal 2: RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

- Issue 2-4: Impact on other LTM requirements Agreement in RAN4#114bis: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS Candidate options: - Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei) - For FR1: - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met - The related cell switch delay and TCI state activation requirements are applicable - The above optimization is not applicable to FR2 - Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia) - Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report. - Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) - Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple) - Number of active TCI does not exceed that UE can support. - Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK) - Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE) - Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///) - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

In this issue, event-triggered L1 report is considered to reduce the cell switch delay or TCI state activation delay. It’s benefical to extend the cell switch delay and TCI state activation requirements to more reasonable scenarios. For FR1, UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, if leaving condition of the event is configured and corresponding condition is not met, it means the measurement is not stopped utill receiving command and the reported TCI state is known. Meanwhile, if the leaving condition of the event is NOT configured, there maybe no measurement after event-triggered L1 report. However, according to the known conditions for TCI state, the TCI state is considered as known during 1280ms after event-triggered L1 report. In our understanding, even without configuring the leaving condition, if we can ensure that the TCI state is known at the time of receiving the cell switch command or TCI state activation command, it still comfores to the applicable rule of R18 LTM requirements.

The requirements in this section apply when - the target cell is known, and - the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or - the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, - PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and - For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

Observation 1：according to the existing spec, the related cell switch delay and TCI state activation requirements are applicable for both FR1 and FR2 during 1280ms after event-triggered L1 report. In addition, they are also applicable for FR1 if the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command or TCI state activation command.

Proposal 3: RAN4 should discuss whether the case is avaibilable that event entering condition is configured but leaving condition is not configured.

For FR2, some companies mentioned that UE will prioritize L1-RSRP measurement on those active TCI state(s) and may never measure the beam which is not activated if NW activates some TCI state(s). So UE may stop measuring the beam if NW activates some other TCI state(s) after event-triggered L1 report. In this case, the report TCI state maybe unknown and the related cell switch delay and TCI state activation requirements can not applicable. Therefore, if we want to extend the usage conditions to FR2, further restrictions may need to be imposed on the conditions, such as there is no activated TCI states.

Observation 2: Further conditions are needed if the optimization in FR1 is applied to FR2

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 need to be updated to cover event triggered L1 measurement reporting.

proposal 2: RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

Observation 1：according to the existing spec, the related cell switch delay and TCI state activation requirements are applicable for both FR1 and FR2 during 1280ms after event-triggered L1 report. In addition, they are also applicable for FR1 if the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command or TCI state activation command.

Proposal 3: RAN4 should discuss whether the case is avaibilable that event entering condition is configured but leaving condition is not configured.

Observation 2: Further conditions are needed if the optimization in FR1 is applied to FR2

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple