3GPP TSG-RAN4 Meeting #116	R4-2510506

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 – Antenna performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the antenna pointing accuracy and performance requirements for NTN Ku-bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add FCC requirements related to antenna performance From the analysys so far, there is no ETSI EN related requirements for the Ku-bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN Ku-bands VSAT requirements will not be properly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

### 9.6.2	Antenna performance

#### 9.6.2.1	Minimum requirements for bands n510 and n511

The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510 and communicating with Geostationary Satellite Orbit (GSO) SAN.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.1-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.1-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3.5° ≤ θ ≤ 7° | 32 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 10.9 |
| 9.2° ≤ θ ≤ 19.1° | 35 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 3 |

The off-axis cross-polarization gain in the plane tangent and in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-3.

Table 9.6.2.1-3: Cross-polarization gain limit

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 19 – 25log10(θ) |

#### 9.6.2.2	Requirement for network signalling value “NS_203N” and “NS_207N”

When "NS_203N" or "NS_207N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

The following requirements are applicable to NTN VSAT operating in band n247 or in band n508 and communicating with Geostationary Satellite Orbit (GSO) SAN.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.2-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 1.5° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-2. This envelope may be exceeded by up to 6 dB in 10% of the range of θ angles from ±3–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2.2-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The off-axis cross-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-3.

Table 9.6.2.2-3: Cross-polarization gain limit in the plane tangent to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 1.8° ≤ θ ≤ 7° | 19 – 25log10(θ) |

The off-axis cross-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.2-4.

Table 9.6.2.2-4: Cross-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 7° | 19 – 25log10(θ) |

<End of the change>