# 3GPP TSG-RAN WG4 Meeting #116	R4-2510523
Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.6.3.3.1

Source: 	OPPO

Title: 	Discussion on RF requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and some agreements are approved in WF [1].

This contribution further discussed the open issue for UE RF RF requirements for IoT NTN HPUE.

- Discussion

Issue 1-1-5: A-MPR for NB-Iot based Iot-NTN PC2

- Agreement:

  - Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N (band 254)

  - No A-MPR is needed for [NS_03N] and NS_02N for NB-IoT PC2 in square brackets

  - Define [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3

    - The exact value can be revisited once MPR is agreed.

In the AMPR evaluation for NR NTN PC2 for NS_24, our simulation result shows that the AMPR is 4.5 dB for 5MHz CBW with 1RB allocation and this 1RB is with 5MHz offset from the lowest edge of protected frequency range.

The total power backoff for NR is maximum (MPR, AMPR), but the total power backoff for LTE is MPR + AMPR. And PC2 NB-IOT NTN UE will have 3 dB MPR at least, therefore, the total power backoff for PC2 NB-IOT NTN UE will be 8.5dB (3+3.5+2). I think it’s ok for PC2 NB-IOT NTN UE compared with 9.5dB (4.5+5dB margin) total power backoff for NR NTN PC2 1RB within 5MHz CBW.

Proposal 1: 5.5 dB A-MPR for NS_24 for PC2 NB-IoT based and MTC based IoT NTN UE.

In the AMPR evaluation for NR NTN PC2 for NS_02N, our simulation result shows that no AMPR is needed for 5MHz CBW, therefore, no AMPR is needed for PC2 NB-IOT UE.

Proposal 2: No A-MPR is need for NS_02N for PC2 NB-IoT based and MTC based IoT NTN UE.

Issue 1-1-6: ACLR for eMTC based Iot-NTN for PC1;

- Agreement:

  - [34dB]

34dB ACLR for PC1 NR NTN has been confirmed as a compromise value based on the coexistence study in last meeting. To keep align with PC1 NR NTN, we prefer to remove the square bracket of ACLR for PC1 eMTC based IoT-NTN.

Proposal 3: Remove the square bracket for 34dB ACLR for PC1 eMTC based IoT-NTN.

About relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation, we prefer to increase 0.8dB on the upper tolerance of MOP not introduce capability signaling.

Proposal 4: Just increase 0.8dB on the upper tolerance of MOP for PC2 NB-Iot based Iot-NTN.

Proposal 5: Apply 0 dB RSD for PC1 non-handheld full duplex eMTC based IoT NTN UE.

- Conclusion

In this paper we give further discussion on the RF requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: 5.5 dB A-MPR for NS_24 for PC2 NB-IoT based and MTC based IoT NTN UE.

Proposal 2: No A-MPR is need for NS_02N for PC2 NB-IoT based and MTC based IoT NTN UE.

Proposal 3: Remove the square bracket for 34dB ACLR for PC1 eMTC based IoT-NTN.

Proposal 4: Just increase 0.8dB on the upper tolerance of MOP for PC2 NB-Iot based Iot-NTN.

Proposal 5: Apply 0 dB RSD for PC1 non-handheld full duplex eMTC based IoT NTN UE.

- Reference

[1] R4-2508122, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#115.