3GPP TSG-RAN WG4 #116	R4-2510553

Bengaluru, India, 25 - 29 August 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM performance requirements for Rel-19 ATG

Agenda Item:	7.8.6

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The TU schedule of #114bis shows that the performance part for Rel-19 ATG is starting. This paper presents our observations and views on RRM impacts of performance parts on ATG CA.

RRM performance requirement analysis

In this part, we provide our analysis of the performance requirements to support CA in case-by-case manner in the following part.

Test applicability rule

The agreement reached in the last meeting is concluded with the test configuration that is captured below. And there is one issue that the test applicability rule needs to discuss.

Issue 2-1-1: Test applicability rule - Proposal 1: - UE is only required to be tested in one of the supported test configurations - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration - Proposal 2: - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

Whether or not to let UE require to test one configuration only, the inter-band CA SCell deactivation and activation need to be considered. For SCell activation and deactivation, the PCell and SCell’s configuration should cover all TDD and FDD duplex modes. So, the test applicability rule should be: UE is required to be tested in one of the supported test configurations randomly.

Proposal 1: UE is required to be tested in one of the supported test configurations randomly.

Test setups

The following content is the performance part of the test setup discussed in the last meeting.

Issue 2-1-2: Test setups - TDD pattern for SCell - Proposal 1: Reuse the legacy TDD pattern (CATT, LGE, CMCC, ZTE) - Proposal 1-1: (CATT) - For intra-band CA, TDD pattern for SCell need to be the same with PCell. - For inter-band CA, the pattern used in R18 ATG test cases can also be reused. - Proposal 1-2: (ZTE) - For TDD case, the TDD pattern for SCell could be same as PCell (ZTE) - Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG (HW) - Proposal 3: Define all ‘D’ slot TDD pattern for ATG (Ericsson)

The TDD pattern has been discussed in the Rel-18 ATG; no new TDD pattern has been introduced. In our view, there is no need to define a new TDD pattern for ATG Rel-19 CA enhancement, and the TDD pattern of SCell is able to use the legacy that is the same TDD pattern as the PCell.

Proposal 2: The TDD pattern of SCell is able to use the legacy that is the same pattern as the PCell.

Test scope

In the last meeting, the companies provided their views on which test cases the Rel-19 ATG is needed for. The following is the summary captured from WF[1].

Issue 2-1-4: Test scope FFS based on following summary: - In RAN4#115, following test cases supported by no less than 5 companies - Interruptions during measurements on deactivated NR SCC in FR1 - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification) - Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification) - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) - In RAN4#115, no supporting company for following test cases - Direct SCell Activation at RRC Resume - non-simultaneous Timer based BWP switch delay on multiple CCs - In RAN4#115, 1-4 supporting companies for following test cases - Direct SCell activation at handover with known SCell in FR1 - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX - Inter-band SSB-less SCell Activation based on A-TRS - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode - Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs - Simultaneous RRC-based Active BWP Switch on multiple CCs - SCell dormancy switch - Event triggered reporting on SCC with deactivated SCell test under non-DRX - Test case for Pre-configured measurement gap requirement - Intra-frequency measurement(with scheduling restriction verified as well) - Inter-frequency measurement(with scheduling restriction verified as well) - L1-RSRP measurement - CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX - The interruption of SCell addition/release

After the last meeting, several test configurations are required for further discussion. The details are provided below:

      - Inter-band SSB-less SCell Activation test case

Regarding the Release 19 ATG CA, which introduced the SSB-less SCell activation requirement, the details on how ATG UE with an antenna array in FR1 can achieve this requirement remain unclear or are based on the UE implementation. This aspect may impact the validity of SSB-less test cases. In our view, SSB-less SCell activation test cases should be restricted to ATG UE explicitly supporting the capability of case 4.

Proposal 3: Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting the capability of case 4.

      - Test case for the Pre-configured measurement gap requirement

The enhancement of Rel-19 ATG is DL CA enhancement, and the pre-configured measurement gap seems to be the Rel-18 requirement. So RAN4 to discussing whether or not to introduce this test case in Rel-19 ATG enhancement.

Proposal 4: RAN4 to discuss whether or not to define the test case for pre-configured measurement gap requirement in Rel-19 ATG.

Conclusion

Proposal 1: UE is required to be tested in one of the supported test configurations randomly.

Proposal 2: The TDD pattern of SCell is able to use the legacy that is the same pattern as the PCell.

Proposal 3: Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting the capability of case 4.

Proposal 4: RAN4 to discuss whether or not to define the test case for pre-configured measurement gap requirement in Rel-19 ATG.

References

[1] R4-2508263 WF on RRM requirements for NR_ATG_enh