3GPP TSG-RAN WG4 Meeting #116	R4-2510576

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on performance requirements of mobility enhancement phase 4

Source: 	Huawei, HiSilicon

Agenda item:	7.25.4

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

The contribution provides preliminary discussion on the performance requirements for R19 mobility.

Discussion

Accuracy of CSI-RS based L1 RSRP on candidate cells

It is agreed in RAN4#114bis [2] that SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline.

In addition, the conditions of CSI-RS L1-RSRP measurement on candidate cells shall be:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Proposal 1: The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

It is agreed that Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. And as per the intra-frequency definition of CSI-RS based measurement, gap is not needed in intra-frequency measurement.  Then the following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified as well:

- Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

Proposal 2: The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

- Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

In addition, it is straight forward that the legacy SS/CSI-RSRP measurement L1 report mapping for serving cell specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Proposal 3: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Test case list

The followings provide the preliminary category of test cases which are to be specified for R19 mobility.

Event triggered L1 reporting

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

Proposal 4: For Event triggered L1 reporting, the following test case category shall be specified:

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

CSI-RS based L1 measurement on candidate cells

As discussed during CSI-RS L1-RSRP measurement on candidate cell, the baseline measurement procedure is “3-step” (i.g., L3 measurement, SSB based L1 measurement, CSI-RS based L1 measurement). So the test case shall verify 3-step CSI-RS L1-RSRP measurement.

It shall be noted that as event triggered reporting test cases for CSI-RS based L1-RSRP measurement are definitely to be defined, it is unnecessary to define specific test for CSI-RS based L1-RSRP measurement procedure.

Proposal 5: As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement.

Proposal 6:  In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified.

The accuracy requirements on CSI-RS RSRP L1 measurement on neighbor cell shall be verified. As per the intra-frequency definition of CSI-RS based L1 measurement, gap is not needed. For inter-frequency CSI-RS based L1 measurement, gap is needed. It is also agreed that NOT to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell, therefore it means that no requirements for inter-frequency CSI-RS based L1 measurement.

Proposal 7: FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified.

CLTM delay requirements and subsequent CLTM delay requirements

Proposal 8: For conditional LTM, the following test case category shall be specified for FR1 and FR2:

- RACH based Intra-frequency conditional PCell switch

- RACH based Inter-frequency conditional PCell switch

- RACH less Intra-frequency conditional PCell switch

- RACH less Inter-frequency conditional PCell switch

Proposal 9: For subsequent conditional LTM, the following test case category shall be specified:

- RACH based Intra-frequency subsequent conditional PCell switch

- RACH based Inter-frequency subsequent conditional PCell switch

- RACH less Intra-frequency subsequent conditional PCell switch

- RACH less Inter-frequency subsequent conditional PCell switch

Conclusion

The contribution provides analysis on performance part. The following proposals are proposed:

Proposal 1: The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Proposal 2: The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

- Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

Proposal 3: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Proposal 4: For Event triggered L1 reporting, the following test case category shall be specified:

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

- FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

Proposal 5: As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement.

Proposal 6:  In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified.

Proposal 7: FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified.

Proposal 8: For conditional LTM, the following test case category shall be specified for FR1 and FR2:

- RACH based Intra-frequency conditional PCell switch

- RACH based Inter-frequency conditional PCell switch

- RACH less Intra-frequency conditional PCell switch

- RACH less Inter-frequency conditional PCell switch

Proposal 9: For subsequent conditional LTM, the following test case category shall be specified:

- RACH based Intra-frequency subsequent conditional PCell switch

- RACH based Inter-frequency subsequent conditional PCell switch

- RACH less Intra-frequency subsequent conditional PCell switch

- RACH less Inter-frequency subsequent conditional PCell switch

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508461, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple