3GPP TSG-RAN WG4 Meeting # 116	R4-2510580

Bengaluru, India, August 25 – 29, 2025

Title: 	Discussion on testability and interoperability issues in AIML mobility

Source: 	Huawei, HiSilicon

Agenda item:	7.19.4

Document for:	Discussion

# Introduction

According to RAN4 #115 WF [1], we discuss the remaining issues of testability and interoperability in AIML mobility.

# 2. Testing goal

Two study goals are defined in RAN2 for verifying the gain introduced by AI. One is to reduce the measurement overhead, another is to enhance handover performance. From RAN4 perspective, both study goals are related to measurement prediction accuracy in RRM measurement prediction and in indirect HO failure/RLF/measurement event prediction.

General aspects in RAN2 AI mobility #126: - Conclusion 1: 1st study goal of evaluation is to reduce measurement overhead - Conclusion 2: 2nd study goal of evaluation is to enhance handover performance

In RAN4 #113 meeting, the testing goal is agreed to verify whether the minimum performance of AI/ML functionality/feature can be achieved. This means that RAN4 will define requirement with the granularity of AI/ML functionality/feature rather than AI/ML model.

RAN4 #113 WF for AI-mobility: Issue 3-2-1: Testing goal Agreement: As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Regarding to the principle of defining the minimum performance in the testing goal needs further discussion. Two options are identified as below.

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy margin requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

Proposal 1: For testing goal, the principle of defining the minimum performance can be discussed based on the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

With option 1, RAN4 can ensure that the accuracy of the predicted measurement will not be worse than that of legacy measurement. Therefore, no handover performance degradation compared to non-AI will be introduced. However, it seems restrictive to force prediction method to achieve the same accuracy as measurement method. In addition, there is lack of evidence that handover performance is bound to degrade with slight measurement accuracy loss. For example, proactive prediction of RRM measurement variation trend will help NW to make handover decision earlier before performance starts to degrade, the gain of which cannot be achieved by using passive measurement method. In this case, as long as the RRM measurement variation trend is accurate enough, there may be some room to relax the requirement of prediction accuracy compared to measurement accuracy.

With option 2, whether adopting the predicted measurement will have an impact on handover performance may need to be evaluated. If there is no handover performance degradation or the degradation is acceptable, the benefit of AI can still be verified in terms of reducing the cost of RRM measurement.

It is also observed that in some cases, for example, in the inter frequency prediction, the measurement reduction helps reduce the measurement gap. If the measurement gap can be reduced, more resources for data transmission can be obtained. In these cases, additional benefit of AI is seen in terms of improving the user experience of services like XR in mobility.

Observation 1: AI based prediction may help enlarge measurement interval due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from enlarging measurement interval and measurement gap reduction shall also be taken into account.

# 3. New testability aspects

## Testing in FR1

RAN4 #115 WF[1]: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

In FR1-to-FR1 prediction, the input to the model is the measurement of the Tx beam. Following the principles of R19 NR_AIML_Air, reaching a consensus on channel models for RRM measurement prediction is essential, with TE capability taken into account. For example, whether it is possible to generate the same channel condition among different testing runs to ensure the ground truth is available if the ground truth is the measurement reporting. It would be helpful if TE vendors provide more analysis.

For the approach to extracting ground truth, it is straightforward to use the measurements reported by the DUT at the time instance when the DUT is supposed to perform measurement or prediction. Otherwise, it is critical to align the timing at which the UE performs measurement and prediction.

Proposal 3: Taking TE capability into account for defining the ground truth, including

- generating the same channel conditions across different testing runs

- generating the Tx power according to UE trajectory assumptions

## Testing in FR2

It is observed that the testability issues in FR2 is similar in that of AI enabled BM, since the model input is the beam-level measurement. According to the latest progress in RAN4 #112 WF, the testability is still under discussion. The open issues are shown as follows.

RAN4 #112 WF in AI-BM: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs - Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

The numbers of beams transmitted simultaneously and the overall number of beams transmitted during the test both need to check the capability of the test equipment. In order to avoid duplicate discussion, we proposed to postpone the discussion to wait the progress of the testability study in AI-BM.

Proposal 4: Postpone the testability discussion for FR2-to-FR2 prediction to wait the progress of the testability study in AI-BM.

## Measurement error model

It is observed that both BB error and RF error are modelled for AI BM. Similar solutions with a few modifications for generating the BB error and RF error also apply to AI mobility.

For modelling BB error, L1-RSRP measurement error can be generated for each SINR based on LLS simulation. After performing L1 filtering and L3 filtering, the L3-RSRP measurement error can be obtained. Gaussian distribution is employed to model the error distribution. In SLS simulation, the error is generated based on the error distribution selected using the SINR for each Tx beam.

For modelling RF error, Gaussian distribution can be used where the mean value and the variance can be decided during WI.

Proposal 5: Use LLS to generate the measurement error model for each SINR, where the error is calculated by comparing the L1-filtered and L3-filtered of the measured L1-RSRP versus L3-RSRP ground truth.

Proposal 6: Use Gaussian distribution to model RF error, the mean value and variance of which will be decided during WI.

# 3. Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Observation 1: AI based prediction may help enlarge measurement interval due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from enlarging measurement interval and measurement gap reduction shall also be taken into account.

Proposal 3: Taking TE capability into account for defining the ground truth, including

- generating the same channel conditions across different testing runs

- generating the Tx power according to UE trajectory assumptions

Proposal 4: Postpone the testability discussion for FR2-to-FR2 prediction to wait the progress of the testability study in AI-BM.

Proposal 5: Use LLS to generate the measurement error model for each SINR, where the error is calculated by comparing the L1-filtered and L3-filtered of the measured L1-RSRP versus L3-RSRP ground truth.

Proposal 6: Use Gaussian distribution to model RF error, the mean value and variance of which will be decided during WI.

# 4. Reference

- R4-2508269, “WF on AI/ML Mobility”, 3GPP RAN4#115, May, 2025.