3GPP TSG-RAN WG4 Meeting #116	R4-2510595

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance for Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

The enhancement on fast SCell activation for UE supporting EMR was completed in previous meetings. In last RAN4 meeting, RAN4 initially discussed the performance part for this feature with agreements and open issues captured in [1]. In this paper we further provide our views on remaining issues.

# Discussion

In last RAN4 meeting, the performance part for fast SCell activation for UE supporting Rel-18 EMR were discussed with status as follows:

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode ### Issue 2-2-2: Details of test case Further discuss this issue. ### Issue 2-2-3: Applicability rules of test case Further discuss this issue.

Regarding the detailed test case list, one remaining issue is whether to introduce TC for direct SCell activation for FR2. From our understanding, for the core part discussion, it is identified that direct SCell activation is a promising scenario for this feature that gNB can activate the SCell upon UE entering connected mode within the same RRC message. Thus, we think RAN4 shall introduce TC for direct SCell activation at SCell addition for FR2.

Proposal 1: RAN4 to define TC for direct SCell activation at SCell addition for FR2

One may have concerns about too many TC cases considering normal SCell activation/PUCCH SCell activation/direct SCell activation. The applicability rules can be considered as previous release. Particularly, for PUCCH SCell activation, it also verifies the UL procedures compared with normal SCell activations. Thus, it is proposed to defined similar applicability rules as:

- For UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

Proposal 2: UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

For the applicable configurations, three cases are considered:

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

From our understanding, UE may only support one of these cases. Thus, RAN4 shall define test cases for all configurations. But to limit the number of TCs that UE is required to pass, applicability rules can be discussed when UE supporting multiple configurations.

Proposal 3: RAN4 to define test case for all following configurations, and RAN4 to discuss the applicability rules when UE supports multiple configurations.

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

# Conclusions

Proposal 1: RAN4 to define TC for direct SCell activation at SCell addition for FR2

Proposal 2: UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

Proposal 3: RAN4 to define test case for all following configurations, and RAN4 to discuss the applicability rules when UE supports multiple configurations.

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

# References

[1] R4-2508266 WF on RRM requirements for NR_RRM_Ph5_Part2