3GPP TSG-RAN WG4 Meeting #116	R4-2510596

Bengaluru, India, August 25th – 29th, 2025

Agenda item:    7.23.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES OD-SSB

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

## OD-SSB processing time.

In last RAN4 meeting, RAN4 achieve following agreements on OD-SSB processing time.

Issue 1-4-3: Additional Processing time for OD-SSB activation Agreement - RAN4 should introduce UE capability with values of 2 ms and 5 ms. Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

For additional processing time for OD-SSB parameter update, to simplify gNB implementation, we can compromise to adopt the same value introduced deactivated SCell measurement. But it should be clearly clarified that the additional processing time only apply to scenario 2 and scenario 3B, to respect previous RAN4 agreements.

Proposal 1: Additional processing time 5ms or 2ms introduced for scenario 2 (OD-SSB activation before SCell activation) can also apply to scenario 3B (OD-SSB parameter updating after SCell is activated).

For OD-SSB deactivation timeline, this was controversial discussion in last meeting whether to add additional process time for OD-SSB deactivation. Companies argued that UE may not be able to stop the measurement immediately. Thus, for the next OD-SSB, UE may still perform measurement on it and cause scheduling restriction when applicable. From our understanding, this is an important issue which may not only impact RAN4 requirements but also RAN1 design, e.g. rate matching. For the case mentioned by companies supporting to have additional processing time, we believe RAN1 also already consider some rules to protect the UE implementation. The relevant RAN1 agreements are shown as follows:

Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

By looking at the highlighted part, there are already some margin on top of “HARQ +3 ms”. For instance, if time instance B is within an on-demand SSB burst, gNB will keep transmit the SSB until the end of last SSB burst, and for the next “candidate OD-SSB burst”, there is at least 5 ms which is the OD-SSB periodicity. For the other cases, there is still some margin to wait for the next “candidate OD-SSB burst”. The only extreme case is that time instance B is exactly the beginning of next OD-SSB. However, this is a concern case to be considered. Based on the analysis above, we see not need to consider additional processing time. From RAN4 RRM requirements perspective, we don’t have tight time sequence for a “deactivation procedure”. If it is identified some additional procedure needs to be considered (e.g. data scheduling related), RAN4 does not have the expertise to evaluate.

Proposal 2:  No UE additional processing time for OD-SSB deactivation.

## L3 measurement (case 2)

Regarding the deactivated SCell measurement for case#2, RAN4 reached following agreements in last meeting:

Issue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation. Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion. Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies) Sub issue 1: RAN2 LS for scenario 3 Agreement: - RAN4 to confirm RAN2’s WS to introduce a new servingCellMO. Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion. Issue 1-2-5: Scheduling restriction for L1 measurement after SCell activation Agreement - In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement. The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on

For Alt Time C2, based on discussion in last meeting, companies mentioned that the uneven distribution of the SSB may cause additional complexity of UE implementations. Thus, as baseline, we can define requirements based on OD-SSB periodicity, which does not require UE to combine the OD-SSB and AO-SSB measurement.

Proposal 3: For Alt Time C2, RAN4 to define baseline requirements based on OD-SSB.

For the case when AO-SSB and OD-SSB are in different frequency, RAN4 will further discuss the scenario. The relevant RAN1 and RAN2 progress are summarized as follows:

RAN1#120 Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB RAN1#120bis Agreement For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications RAN2#129bis R2-2503019 In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not. RAN2#130 RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

Based on above RAN1/RAN2 progress, we provide our analysis on UE measurement behavior for L3 measurement.

For serving Cell measurement, it is clear that UE is only required to measure one SSB (i.e. either OD-SSB or AO-SSB). Specifically, it depends on whether OD-SSB is activated. If OD-SSB is activated UE measures serving Cell based servingCellMO-OD; otherwise, UE measures serving Cell based on servingCellMO.

Observation 1: When OD-SSB and AO-SSB are in different frequencies, for serving Cell L3 measurement:

- If OD-SSB is activated, UE measures serving Cell based servingCellMO-OD (i.e. OD-SSB);

- If OD-SSB is deactivated, UE measures serving Cell based on servingCellMO (i.e AO-SSB);

The missing part is how to perform neighbor cell measurement. From our understanding, there are following possible alternatives:

- Alt1: UE perform neighbor cell measurement on the same frequency as serving Cell

- Alt2: UE always perform neighbor cell measurement based on servingCellMO

- Alt3: UE perform neighbor cell measurement on both servingCellMO and servingCellMO-OD

- Alt4: gNB indicates on which frequency that UE to perform neighbor cell measurement.

Alt1 is straightforward from UE implementation perspective. However, since L3 measurement is for mobility purpose, it cannot be guaranteed that the SSB of neighbor Cells are also dynamically switched together with serving Cell. One typical deployment is that the SSB of all neighbor Cells are still located at AO-SSB frequency to guaranteed that mobility performance, based on this assumption, Alt2 seems reasonable. However, it restricts the gNB implementation that all neighbor Cell has to transmit AO-SSB. For Alt3, it seems safer for mobility perspective, but it greatly increases UE implementation complexity. For alt4, it seems to be a promising solution that gNB indicate which frequency that UE is required to perform neighbor cell measurement based on gNB implementation, and UE is on required to perform measurement on one of frequency layers. But when the serving Cell carrier and neighbor cell carrier are different, whether UE is required to perform simultaneous measurement or still complicated.

Proposal 4: Based on RAN1/2 progress, when OD-SSB and AO-SSB are in different frequencies, for serving Cell L3 measurement:

- If OD-SSB is activated, UE measures serving Cell based servingCellMO-OD (i.e. OD-SSB);

- If OD-SSB is deactivated, UE measures serving Cell based on servingCellMO (i.e AO-SSB);

Proposal 5: When OD-SSB and AO-SSB are in different frequencies, RAN4 to discuss the neighbor cell measurement behavior and requirements, considering following alternatives:

- Alt1: UE perform neighbor cell measurement on the same frequency as serving Cell

- Alt2: UE always perform neighbor cell measurement based on servingCellMO

- Alt3: UE perform neighbor cell measurement on both servingCellMO and servingCellMO-OD

- Alt4: gNB indicates on which frequency that UE to perform neighbor cell measurement.

## Scenario 3B

Based on previous RAN1 agreements, for scenario 3B, following parameters can be updated:

Observation 2: Following parameters could be updated per RAN1 agreements for far for Scenario 3B:

- SSB periodicity of OD-SSB

- SSB positions within OD-SSB (for case1 and case 2 different frequency)

- N of od-SSB burst to be transmitted after OD-SSB is indicated

Regarding scenario 3B, RAN2 achieved following agreements in last meeting.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

From our understanding, the above approach is intended for scenario 3B L3 measurement. For deactivated SCell measurement, RAN4 already agree on the UE measurement behavior and requirements (e.g. FMW based measurement).

Observation 3: OD-SSB specific SMTC based measurement is only for Scenario 3B serving Cell measurement.

Proposal 6: For scenario 3B, UE perform serving Cell measurement based on OD-SSB specific SMTC (both case 1 and case 2)

For L1 measurement, RAN4 achieved following agreements in last meeting, which is shown as follows:

Issue 1-4-7: L1 measurement in scenario 3B Agreement - When MAC-CE indicates OD-SSB updating in scenario 3B, - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB. - When DRX is configured, follow legacy principle to define requirements - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

For L1 measurement impact in Scenario, there is on common issue for OD-SSB and SSB adaptation which is also analysed in our paper for SSB adaption in last meeting. For L1 measurement restriction requirements, when SSB-based L1-RSRP and CSI-RS are overlapped, UE is only required to measure one of them. Based on existing requirements, it is up to UE to choose either SSB or CSI-RS for measurement. However, OD-SSB parameters updating in Scenario 3, SSB are occasionally triggered by DCI. Since the Cell specific CSI-RS is configured based on normal SSB pattern without collision, the newly triggered the SSB (different periodicity or different position) may colliding the CSI-RS and gNB may not guarantee that both SSB and CSI-RS are available to UE when they are overlapping. Thus, if UE choose CSI-RS for measurement, it may cause wrong measurement results (e.g. wrong L1-RSRP results/ wrongly triggered BFD). Thus, it should be discussed whether legacy measurement restriction can apply when adapted SSB colliding with CSI-RS.

Observation 4: Based on legacy measurement restriction when CSI-RS and SSB collides, UE can choose either of them for measurement, which implicitly assumed that both CSI-RS and SSB are available.

Observation 5: When OD-SSB parameters updating in Scenario 3, the dynamic change of SSB periodicity and positions may collides with CSI-RS and the CSI-RS may be dropped.

Proposal 7: For OD-SSB parameters updating in Scenario 3, RAN4 discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

## Others

One issue is about the case when OD-SSB collides with MG in the FMW. The status are summarized as follows:

Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

From our understanding, MG occasion shall be kept. Otherwise, RAN4 needs to consider the MG cancelation which is discussed in XR WI, which is rather complicated. For OD-SSB based measurement in FMW, it is not that urgent compared with XR service. It will ease the UE implantation complexity when MG occasion is still kept.

Proposal 8: When OD-SSB in FMW collides with MG occasion, legacy principle applies that UE is allowed to skip the OD-SSB measurement when the OD-SSB collides with MG occasion.

Another issue is whether to define OD-SSB based SCell activation requirements for Rel-18 SCell activation with L3 report. During the discussion in last meeting, companies commented that in Rel-18 RRM enhancement, similar features are introduced. We summarized the relevant features in Rel-18 RRM as follows [2].

There are three UE capabilities were introduced in Rel-18 RRM:

- 31-1: Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available

- 31-2: Beam sweeping factor reduction for FR2 unknown SCell activation

- 31-3: Shorter measurement interval for unknown SCell activation

For UE supporting 31-1 and Rel-19 OD-SSB, RAN4 shall define requirements that UE follows OD-SSB periodicity during SCell activation. This operation is common for all SCell activation procedure (e.g. normal SCell activation/PUCCH SCell activation). Otherwise, UE behaviour and requirements are not clear when UE support both capabilities.

During the discussion in last meeting, companies commented that the UE behaviour discussed in Rel-19 NES (i.e. use OD-SSB periodicity instead of SMTC) was similar as 31-3. Thus, it was proposed that UE shall mandatory support 31-3 when UE support Rel-19 OD-SSB. From our understanding, the logic is not correct. It we look at the components of 31-3, there are two components: one is using SSB periodicity instead of SMTC and another one is performing L1-RSRP in non-DRX mode. Considering a case that UE only wants to support Rel-19 OD-SSB and do not want to support Rel-18 RRM 31-3, it will cause unnecessary restriction on UE implementation. What’s more, it is abnormal to force UE to support a capability in previous release because UE wants to support a capability in further release.

Observation 6: UE’s behaviour during SCell activation based on OD-SSB are common, regardless of normal SCell activation/PUCCH SCell activation/Rel-18 SCell activation with L3 report.

Observation 7: Without RRM requirements, for UE supporting Rel-18 SCell activation with L3 report (i.e. 31-1) and Rel-19 OD-SSB, the UE behaviour and requirements are not clear, and it will cause problem on gNB configuration (e.g. whether and how to configure SMTC).

Observation 8: It is unnecessary/abnormal to force UE to support a capability in previous release which may cause NBC problem.

Based on analysis above, it is proposed to define requirements for Rel-18 SCell activation with L3 report based on OD-SSB.

Proposal 9: RAN4 to define requirements for Rel-18 SCell activation with L3 report based on OD-SSB.

Another issue is whether to introduce OD-SSB based L3 measurement reporting. UE can trigger the measurement report if there is valid measurement report and UE can stop fast measurement. The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window. By legacy case, what we can imagine is gNB to artificially configured a very low threshold to trigger the event and reconfigure it after report to avoid UL overhead and wrongly triggered event. The is very clumsy and situation will be more severe for case#2. We think we can have similar approach as introduced in Rel-18 RRM “report on SCell activation”, and the workload is manageable in RAN2 since framework can be reused. RAN2 discussed the issue ins last meeting, and achieved following agreements.

RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Observation 9: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 10: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

# Conclusions

Proposal 1: Additional processing time 5ms or 2ms introduced for scenario 2 (OD-SSB activation before SCell activation) can also apply to scenario 3B (OD-SSB parameter updating after SCell is activated).

Proposal 2:  No UE additional processing time for OD-SSB deactivation.

Proposal 3: For Alt Time C2, RAN4 to define baseline requirements based on OD-SSB.

Observation 1: When OD-SSB and AO-SSB are in different frequencies, for serving Cell L3 measurement:

- If OD-SSB is activated, UE measures serving Cell based servingCellMO-OD (i.e. OD-SSB);

- If OD-SSB is deactivated, UE measures serving Cell based on servingCellMO (i.e AO-SSB);

Proposal 4: Based on RAN1/2 progress, when OD-SSB and AO-SSB are in different frequencies, for serving Cell L3 measurement:

- If OD-SSB is activated, UE measures serving Cell based servingCellMO-OD (i.e. OD-SSB);

- If OD-SSB is deactivated, UE measures serving Cell based on servingCellMO (i.e AO-SSB);

Proposal 5: When OD-SSB and AO-SSB are in different frequencies, RAN4 to discuss the neighbor cell measurement behavior and requirements, considering following alternatives:

- Alt1: UE perform neighbor cell measurement on the same frequency as serving Cell

- Alt2: UE always perform neighbor cell measurement based on servingCellMO

- Alt3: UE perform neighbor cell measurement on both servingCellMO and servingCellMO-OD

- Alt4: gNB indicates on which frequency that UE to perform neighbor cell measurement.

Observation 2: Following parameters could be updated per RAN1 agreements for far for Scenario 3B:

- SSB periodicity of OD-SSB

- SSB positions within OD-SSB (for case1 and case 2 different frequency)

- N of od-SSB burst to be transmitted after OD-SSB is indicated

Observation 3: OD-SSB specific SMTC based measurement is only for Scenario 3B serving Cell measurement.

Proposal 6: For scenario 3B, UE perform serving Cell measurement based on OD-SSB specific SMTC (both case 1 and case 2)

Observation 4: Based on legacy measurement restriction when CSI-RS and SSB collides, UE can choose either of them for measurement, which implicitly assumed that both CSI-RS and SSB are available.

Observation 5: When OD-SSB parameters updating in Scenario 3, the dynamic change of SSB periodicity and positions may collides with CSI-RS and the CSI-RS may be dropped.

Proposal 7: For OD-SSB parameters updating in Scenario 3, RAN4 discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

Proposal 8: When OD-SSB in FMW collides with MG occasion, legacy principle applies that UE is allowed to skip the OD-SSB measurement when the OD-SSB collides with MG occasion.

Observation 6: UE’s behaviour during SCell activation based on OD-SSB are common, regardless of normal SCell activation/PUCCH SCell activation/Rel-18 SCell activation with L3 report.

Observation 7: Without RRM requirements, for UE supporting Rel-18 SCell activation with L3 report (i.e. 31-1) and Rel-19 OD-SSB, the UE behaviour and requirements are not clear, and it will cause problem on gNB configuration (e.g. whether and how to configure SMTC).

Observation 8: It is unnecessary/abnormal to force UE to support a capability in previous release which may cause NBC problem.

Proposal 9: RAN4 to define requirements for Rel-18 SCell activation with L3 report based on OD-SSB.

Observation 9: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 10: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

# References

[1] R4-2504906 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R4-2321521 WF for [109][208] NR_RRM_enh3_part1

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 | Lower bound on OD-SSB periodicity for measurement in fast measurement window | A lower bound on OD-SSB periodicity of [10] ms for measurement in deactivated SCell fast measurement window | At least one among 61-1/ 61-2/61-2a/ 61-3/ 61-4/61-4a | Yes | n/a | There is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window | Per BC | n/a | n/a | Yes | If UE does not indicate this capability, there is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window. | Optional with capability signalling |
|  | 54-2 | Additional processing timing for OD-SSB activation | Additional OD-SSB processing time for OD-SSB reception. | At least one among 61-3/ 61-4/61-4a | Yes | n/a | UE is not able to receive OD-SSB before the processing time | Per UE | n/a | n/a | Yes | Candidate value :{[2 or 3]ms, 5ms}. The additional processing time does not apply when OD-SSB activation comes together with SCell activation. | Optional with capability signalling |