3GPP TSG-RAN WG4 Meeting # 116	R4-2510603

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.26.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on Performance requirements for Rel-19 XR

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we provide our views on potential impact on performance part.

# Discussion

Based on the discussion and conclusion for core requirements, in Rel-19, RAN4 only defined requirements for MG skipping for type 1 and type 2 gaps. And following requirements are impacted:

| 9.2.6 | Intra-freq measurements with gaps |
| --- | --- |
| 9.3.4 | Inter-freq measurements with gaps |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements |
| 9.15.5 | Inter-frequency L1-RSRP with MG |

RAN4 shall define test cases to verify following behaviours/requirements for this feature:

- The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly.

- UE shall meet the extended measurement delay requirements.

Proposal 1: RAN4 to define test cases for MG skipping to verify following behaviour and requirements:

- The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly.

- UE shall meet the extended measurement delay requirements.

RAN4 also introduce a capability to differentiate different processing time for MG skipping (e.g. 3ms and 5ms). Thus, it UE supporting 3ms processing capability, it should be verified in the test cases. In details, the DCI for MG skipping shall be transmitted 3ms before MG occasion.

Proposal 2: The processing time 3ms/5ms shall be verified in the test cases. If UE supporting additional 3ms processing capability, 3ms shall be verified.

# Conclusions

Proposal 1: RAN4 to define test cases for MG skipping to verify following behaviour and requirements:

- The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly.

- UE shall meet the extended measurement delay requirements.

Proposal 2: The processing time 3ms/5ms shall be verified in the test cases. If UE supporting additional 3ms processing capability, 3ms shall be verified.

# References

[1] R4-2508285 WF on RRM requirements for XR_Ph3