3GPP TSG-RAN WG4 Meeting #116	R4-2510712

# Bangaluru, India, August 25th – 29th 2025

Source:	Ericsson

Title:	Discussion on the methodology of NR SCM for demodulation

Agenda item:	7.12.3

Document for:	Discussion

# Introduction

In the RAN4#115 meeting, companies mainly discussed the detailed of derivation of CDL based model and TDL based model. In WF [1], agreed CDL model truncation procedures are captured. The model profiles per CDL model options are also discussed by email before the RAN4#115 meeting.

In this contribution, the open issues on common methodologies, pros and cons of CDL/TDL based model and metrics for alignment and comparison are further discussed.

# Discussion

## 2.1	Common for all Methodologies

#### PDSCH ‘Fixed’ PMI Selection for comparison

Way Forward:

At least random to be considered for both 4 and 8 layer.

For 8 Layer:

Collectively Chosen Fixed, (4,0,0) initially

For 4 Layer:

Random

Collectively Chosen Fixed, (4,0,0) initially

According to the quick discussion among companies, it seems hard to align fixed PMI configuration based on companies’ follow-PMI results. It might be caused by different implementation on how UE choose PMI based on estimated channel. In our simulations [2], our preferred PMI is i1,1,i1,2,i2=(1,0,0) and i1,1,i1,2,i2=1,0,1 in histogram statistics, but they show very large per CW performance difference (similar as i1,1,i1,2,i2=(2,0,0)). The PMI i1,1,i1,2,i2=(4,0,0) proposed by other companies in WF can show relatively closer performance per CW, see figure below.

Figure 2.1-1 8 layers PDSCH performance with fix PMI

Large difference on preferred PMI among companies due to potential different implementations on PMI selection.

#### Antenna Element Spacing

Agreement:

(Confirmation of previous agreement and additional agreement on both UE and gNB)

RAN4 to use λ/2 spacing of antenna elements.

The larger antenna element separation 4λ is proposed by the company for PMI comparison simulation. The motivation is to have similar spatial filter as a large subarray. The simulation results show that larger separation could achieve better performance on eType-II codebook on 4 layers case. This gives an idea to simplify the simulation setup by replacing subarray virtualization with larger AE separation in some special case, but the AE radiation pattern should be modified according to subarray pattern, for example larger vertical separation should use smaller vertical HPBW to represent larger column subarray.

By applying 32Tx on both type-I and eType-II codebook without SNR normalization (as discussed in chapter 2.3 later), the eType-II could show much better performance than type-I on both SNR at target NThp and Gamma value in follow PMI case. See figures below and section 2.2.2 in [2]. But we don’t see obvious performance gain from eType-II codebook with 8Tx. With larger antenna element separation, the performance of eType-II is further improved.

If we do normalization at receiver side (e.g., use -5dB scaler) for 32Tx, it’s performance would be worse than 8Tx.

eType-II codebook shows much better performance than type-I code book with 32Tx without SNR normalization.

It would be more realistic to use smaller AE vertical HPBW when larger vertical separation is applied.

## 2.2	CDL based methodologies

#### CDL Table Truncation

Agreement:

Use a single truncated CDL, with a unified table.

The untruncated CDL model profile with 20 clusters will be captured in the description of derivation procedure for truncated CDL model in the TR.

Channel property characteristic differences caused by cluster merging may be captured in the TR, based on company reviews.

The truncated CDL model was agreed to be used for alignment and comparison. The proponent companies have delivered the method descriptions, but some companies still can’t get the exactly same model profile as agreed. The main diverse part is the spatial filter creation which is not clear in the method description. It would be better to give more information on spatial filter generation in the final TR.

Proposal 1	Capture more descriptions on spatial filter creation in truncated CDL model generation in final TR.

Frequency Generalisation

Options

- Option 1:  To select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

- Option 2: Use 3.5GHz for all FR1 bands and duplexing, following legacy and SCM alignment.

Generally, parameter differences caused by frequency band are more like scaling based on same derivation formula, then the corresponding performance are comparable based on experience. For example, we use different TDL models in HST for FDD band and TDD band separately to differentiate Doppler impact. In CDL based model derivation, desired AS, Doppler shift and antenna configuration might be relevant to frequency band selection.

- In TS38.827 Table 7.2-4, it is indicated that the desired AS is evaluated with frequency 6GHz which could be enough to cover whole FR1 band (the difference is in deployment scenario, e.g., Uma or Umi). Current CDL based model profile is derived from CDLC model in TR38.901 with desired AS values in TR38.827.

- The delay spread selection in TR38.827 Table 7.2-1 is also appliable for whole FR1 band but different for deployment scenario.

- Companies agree to use one-by-one mapping from CSI-port to antenna-port, then there would be no difference among FR1 bands on antenna configurations.

- The current Doppler shift is evaluated from the center frequency and UE velocity vector.

In that case, the profile besides Doppler shift should be OK for whole FR1 band. It would not be very complicated to handle Doppler impact in simulation or TE implementation. For Rel-19, it is time to wrap up the study. It is suitable to just use one model per methodology for alignment and comparison. The parameter difference between frequency bands is independent of methodology itself and could be considered in the future release for specific scenarios.

The agreed CDL based model profile besides Doppler shift can be applied for whole FR1 bands.

Proposal 2	Only consider 3.5GHz for Doppler evaluation in Rel-19 and consider other values in future release for specific scenarios.

#### SNR Normalisation with CDL

Way Forward:

As per previous agreements, Channel (inc. signal power, AAV gain) normalisation to be done by companies.

Although we agree not to align SNR normalization method for CDL model, it should be noted that the fixed normalization factor, such as maximum gain obtained at receiver side, would much degrade received power for the configuration with large Tx antenna array or more CSI ports. It might lead to strange results that larger antenna array or more CSI ports will show worse performance (e.g., SNR at target normalized throughput) after normalization which is not aligned with observation in real network.

On the other hand, the receiver performance should be decoupled from Tx antenna configuration. So any “extra” gain causing by larger Tx antenna array or by more CSI ports is expected to be removed. Currently implementation is to use one-by-one CSI port to antenna element mapping in CDL model to minimize the impact, but it would limit the benefit in simulation of some MIMO features which are more suitable for large antenna.

In that case, it is suggested not to compare performance of CDL model between different Tx antenna array size or different number of CSI port configurations. For example, it is not suitable to compare throughput performance between 8Tx and 32Tx in PMI cases.

It is not suitable to compare throughput performance between different Tx antenna array sizes or different number of CSI ports no matter if a simple SNR normalization is applied or not.

## 2.3	Comparison of methodologies

Channel Properties

In our simulations (section 2.3 in [2]), we give analysis on time coherence (TC), frequency coherence (FC) and power angular distribution of CDL model and legacy TDL model.

For time coherence and frequency coherence, both CDLC models and TDLC models show similar properties, and the results are kind of aligned with expectation regarding configured RMS DS and RMS Doppler. It should be noted that the “side peaks” in FC figures are caused by delay offset between strongest clusters/taps.

Table 2.3-1 Coherence comparison

| Coherence type | UE speed | CDLC A4 (12 clusters) | CDLC A2 (20 clusters) | CDLC 901 (24 clusters) | TDLC300 |
| --- | --- | --- | --- | --- | --- |
| Time (s) | 3km/h | 0.28 | 0.28 | 0.34 | 0.2 |
|  | 30km/h | 0.03 | 0.03 | 0.03 | 0.02 |
| Frequency (PRB) | 3km/h | 8 | 8 | 8 | 7 |
|  | 30km/h | 7 | 7 | 7 | 7 |

The following observations could be made:

CDLC A4 and CDLC A2 model have almost same TC and FC properties.

TDLC model shows a sharper peak in both TC and FC compared to CDLC models.

We delivered the PAD of AoD at PRB10 and Rx port #1. Compared to large BS antenna configuration AAV (8, 8, 2), AAV (1, 4, 2) shows similar angular distribution in azimuth dimension but less differentiation in zenith dimension. Also compared with channel profile of CDLC A4, the PAD is also aligned with the strong clusters’ direction.

In field channel analysis [3], PAD of UL signal also shows long-term dominant direction. The difference from simulation is the BS have much larger antenna than UE which could get clearer spatial preference in UL than DL.

The angular power distribution of CDLC A4 is aligned with expectation.

The properties of long-term angular preference of CDL-based model is aligned with the observation from field measurement results.

Performance comparison

For CDLC A4 and CDLC A2, very similar performance can be seen in most cases. To simplify the channel profile and TE implementation, truncated model A4 with 12 clusters could be accepted. But it is known that the truncation processing depends on antenna configuration and BS/UE locations in GCS, then A2 with 20 clusters could generate different truncated models in that case. A2 model is agreed to be captured in the TR for future reference, but it would be good to investigate how to create less variant truncation models for most typical antenna configurations, e.g., using 13 clusters than 12 clusters.

CDLC A2 (20 clusters) and CDLC A4 (12 clusters) show similar performance in most simulation cases.

It would be better to further investigate how to create less variant truncation models for most typical antenna configurations.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Large difference on preferred PMI among companies due to potential different implementations on PMI selection.

Observation 2	eType-II codebook shows much better performance than type-I code book with 32Tx without SNR normalization.

Observation 3	It would be more realistic to use smaller AE vertical HPBW when larger vertical separation is applied.

Observation 4	The agreed CDL based model profile besides Doppler shift can be applied for whole FR1 bands.

Observation 5	It is not suitable to compare throughput performance between different Tx antenna array sizes or different number of CSI ports no matter if a simple SNR normalization is applied or not.

Observation 6	CDLC A4 and CDLC A2 model have almost same TC and FC properties.

Observation 7	TDLC model shows a sharper peak in both TC and FC compared to CDLC models.

Observation 8	The angular power distribution of CDLC A4 is aligned with expectation.

Observation 9	The properties of long-term angular preference of CDL-based model is aligned with the observation from field measurement results.

Observation 10	CDLC A2 (20 clusters) and CDLC A4 (12 clusters) show similar performance in most simulation cases.

Observation 11	It would be better to further investigate how to create less variant truncation models for most typical antenna configurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture more descriptions on spatial filter creation in truncated CDL model generation in final TR.

_Toc206185787

Proposal 2	Only consider 3.5GHz for Doppler evaluation in Rel-19 and consider other values in future release for specific scenarios.

# References

[1] 	R4-2508776, Way Forward for [115][322] FS_NR_demod_SCM, Nokia,

[2]	R4-2510713, Simulation results for NR SCM for demodulation, Ericsson, RAN4#116

[3]	R4-25xxxxx, Channel Properties for Comparison in SCM, BT, Ericsson, RAN4#116

[7]