3GPP TSG-RAN WG4 Meeting # 116	R4-2510776

# Bengaluru, India, 25th – 29th Aug, 2025

Source:	LG Electronics

Title:	Discussion for UE RF Tx requirements of NTN_NR_HPUE

Agenda item:	7.6.3.2.1

Document for:	Discussion

Introduction

In RAN4#115 meeting, WF on NR NTN HPUE requirements was approved. Based on the WF, we would like to share our  view for the requirements.

Discussion

PC1 MPR

1Tx PC1  MPR simulation

The 1Tx PC1 simulation was conducted by modifying the mRAPP PA model according to the PC1 calibration settings. The simulation results are presented in Table 1 (RBstart, LCRB).

Proposal 1: PC1 1Tx MPR value can be discussed based on Table 1.

Table 1 1Tx PC1  MPR simulation results for 20 MHz CBW

| PC1 1Tx |  | edge |  |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,6) | (0,8) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 10.03 | 6.78 | 6.28 | 0.59 | 0 | 0 |
| 16QAM |  | 10.12 | 7.03 | 6.53 | 0.91 | 0 | 0 |
| 64QAM |  | 10.16 | 7.03 | 6.34 | 0.91 | 0 | 0 |
| QPSK | CP-OFDM | 10.84 | 8.28 | 7.19 | 2.03 | 0 | 0 |
| 16QAM |  | 10.56 | 8.28 | 7.28 | 2.03 | 0 | 0 |
| 64QAM |  | 11.03 | 8.63 | 7.28 | 2.03 | 0 | 0 |

4Tx PC1  MPR simulation

The 4Tx PC1 simulation was conducted using the PC2 mRAPP PA model, with offsets applied to both SE and SEM. The simulation results are presented in Table 2 (RBstart, LCRB).

Proposal 2: PC1 4Tx MPR value can be discussed based on Table 2.

Table 2 4Tx PC1  MPR simulation results

| PC1 4Tx |  | edge |  |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,6) | (0,8) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 10.41 | 7.25 | 6.78 | 2.56 | 0 | 0 |
| 16QAM |  | 10.34 | 7.34 | 6.69 | 2.59 | 0 | 0 |
| 64QAM |  | 10.41 | 7.22 | 6.47 | 2.56 | 0 | 0 |
| QPSK | CP-OFDM | 11.06 | 8.53 | 7.59 | 4.28 | 0 | 0 |
| 16QAM |  | 11.03 | 8.53 | 8.03 | 4.28 | 0 | 0 |
| 64QAM |  | 10.78 | 8.62 | 7.78 | 4.28 | 1.88 | 0.94 |

PC1.5 MPR

The PC1.5 1Tx simulation was performed by modifying the mRAPP PA model according to the PC1.5 calibration settings. The simulation results are presented in Table 3 (RBstart, LCRB)

Proposal 3: PC1.5 1Tx MPR value can be discussed based on Table 3.

Table 3 PC1.5 MPR simulation results

| PC1.5 1Tx |  | edge |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,2) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 8.38 | 7.56 | 0.81 | 0 | 0 |
| 16QAM |  | 8.44 | 7.53 | 0.81 | 0 | 0 |
| 64QAM |  | 8.53 | 7.44 | 0.78 | 0 | 0 |
| QPSK | CP-OFDM | 9.03 | 8.09 | 2.31 | 0 | 0 |
| 16QAM |  | 9.03 | 8.16 | 2.34 | 0 | 0 |
| 64QAM |  | 9.09 | 8.06 | 2.31 | 0 | 0 |

NS_24 A-MPR

The NS_24 A-MPR simulations were conducted for PC2, PC1.5, and PC1, using the same PA model and calibration settings that were previously applied in the MPR simulations. The simulation results are presented in the table below.

Table 4 NS_24 A-MPR simulation data

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |  |  | Region C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS | A-MPR DFT-s-OFDM | A-MPR CP-OFDM | RBend*12*SCS | LCRB*12*SCS | A-MPR DFT-s-OFDM | A-MPR CP-OFDM | RBend*12*SCS | LCRB*12*SCS | A-MPR DFT-s-OFDM | A-MPR CP-OFDM |
|  |  | MHz | MHz |  |  | MHz | MHz |  |  | MHz | MHz |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | 3.5 | 5.5 |  |  |  |  |  |  |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | 3.69 | 3.53 |  |  |  |  |  |  |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | 7.28 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | 9.03 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | 9.31 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | 8.5 | 8.5 |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | 7.16 | 7.06 |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | 10.5 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | 12.09 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | 12.53 | - |  |  |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | 11 | 13 | >3.6 | >1.08 ≤1.98 | 5 | 6.5 | ≤3.6 | ≤1.98 | 4 | 4 |
|  |  |  |  |  |  |  | ≤1.08 | 10 | 12 |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | 12.09 | 13.03 | >3.6 | >1.08 ≤1.98 | 12.56 | 12.84 | ≤3.6 | ≤1.98 | 11.75 | 11.53 |
|  |  |  |  |  |  |  | ≤1.08 | 14.01 | 14.41 |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | 15.3 | - | >3.6 | >1.08 ≤1.98 | 15.81 | - | ≤3.6 | ≤1.98 | 14.78 | - |
|  |  |  |  |  |  |  | ≤1.08 | 17.16 | - |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | 16.5 | - | >3.6 | >1.08 ≤1.98 | 17.31 | - | ≤3.6 | ≤1.98 | 16.38 | - |
|  |  |  |  |  |  |  | ≤1.08 | 18.78 | - |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | 16.94 | - | >3.6 | >1.08 ≤1.98 | 17.72 | - | ≤3.6 | ≤1.98 | 16.78 | - |
|  |  |  |  |  |  |  | ≤1.08 | 19.03 | - |  |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | 8.5 | 8.5 |  |  |  |  |  |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | 5.97 | 6.03 |  |  |  |  |  |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | 9.22 | - |  |  |  |  |  |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | 7 | - |  |  |  |  |  |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | 7.59 | - |  |  |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | 11 | 13 | ≥7.2 | >1.08 ≤4.32 | 5 | 6.5 | <7.2 | ≤4.32 | 4 | 4 |
|  |  |  |  |  |  |  | ≤1.08 | 10 | 12 |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | 8.41 | 10.72 | ≥7.2 | >1.08 ≤4.32 | 8.22 | 8.16 | <7.2 | ≤4.32 | 6.31 | 6.41 |
|  |  |  |  |  |  |  | ≤1.08 | 8.28 | 8.38 |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | 10.53 | - | ≥7.2 | >1.08 ≤4.32 | 11.53 | - | <7.2 | ≤4.32 | 9.59 | - |
|  |  |  |  |  |  |  | ≤1.08 | 11.53 | - |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | 11.59 | - | ≥7.2 | >1.08 ≤4.32 | 13.09 | - | <7.2 | ≤4.32 | 11.28 | - |
|  |  |  |  |  |  |  | ≤1.08 | 13.03 | - |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | 12.03 | - | ≥7.2 | >1.08 ≤4.32 | 13.47 | - | <7.2 | ≤4.32 | 11.66 | - |
|  |  |  |  |  |  |  | ≤1.08 | 13.44 | - |  |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | 18 | 19 | <3.06 |  | 18 | 19 | ≥3.06 | >1.44 | 10 | 12 |
|  |  |  |  |  |  |  |  |  |  | <5.76 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | 30 | 30 | <3.06 |  | 7.16 | 7.06 |  |  | 9.03 | 8.91 |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | 34 | - | <3.06 |  | 10.28 | - |  |  | 12.22 | - |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | 34 | - | <3.06 |  | 12.03 | - |  |  | 13.78 | - |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | 34.5 | - | <3.06 |  | 12.25 | - |  |  | 14.22 | - |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | 11 | 13 | ≥10.8 | >1.08 ≤6.84 | 5 | 6.5 | <10.8 | ≤6.84 | 4 | 4 |
|  |  |  |  |  |  |  | ≤1.08 | 10 | 12 |  |  |  |  |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  |  | 7.31 | 9.78 | ≥10.8 | >1.08 ≤6.84 | 6.97 | 8.91 | <10.8 | ≤6.84 | 2.41 | 2.34 |
|  |  |  |  |  |  |  | ≤1.08 | 5.66 | 5.69 |  |  |  |  |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  |  | 9.06 | - | ≥10.8 | >1.08 ≤6.84 | 8.69 | - | <10.8 | ≤6.84 | 6.03 | - |
|  |  |  |  |  |  |  | ≤1.08 | 9.72 | - |  |  |  |  |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  |  | 9.59 | - | ≥10.8 | >1.08 ≤6.84 | 9.31 | - | <10.8 | ≤6.84 | 7.78 | - |
|  |  |  |  |  |  |  | ≤1.08 | 13.41 | - |  |  |  |  |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  |  | 10.25 | - | ≥10.8 | >1.08 ≤6.84 | 9.88 | - | <10.8 | ≤6.84 | 8.16 | - |
|  |  |  |  |  |  |  | ≤1.08 | 13.41 | - |  |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | 18 | 19 | <3.78 |  | 18 | 19 | ≥3.78 | >1.44 | 10 | 12 |
|  |  |  |  |  |  |  |  |  |  | <8.64 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | 27.59 | 30 | <3.78 |  | 4.47 | 4.84 | ≥3.78 | >1.44 | 7.59 | 7.62 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | 30.56 | - | <3.78 |  | 8.09 | - | ≥3.78 | >1.44 | 11.03 | - |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | 34 | - | <3.78 |  | 10.03 | - | ≥3.78 | >1.44 | 12.78 | - |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | 34.5 | - | <3.78 |  | 10.38 | - | ≥3.78 | >1.44 | 13.09 | - |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | 18 | 19 | <4.68 |  | 18 | 19 | ≥4.68 | >2.16 | 10 | 12 |
|  |  |  |  |  |  |  |  |  |  | <12.96 |  |  |  |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | 27.47 | - | <4.68 |  | 1.28 | - | ≥4.68 | >2.16 | 6.62 | - |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | 30.42 | - | <4.68 |  | 4.19 | - | ≥4.68 | >2.16 | 8.69 | - |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | 34 | - | <4.68 |  | 6.38 | - | ≥4.68 | >2.16 | 9.62 | - |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | 34.5 | - | <4.68 |  | 6.56 | - | ≥4.68 | >2.16 | 10.22 | - |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | 18 | 19 | <5.58 |  | 18 | 19 | ≥5.58 | >1.44 | 10 | 12 |
|  |  |  |  |  |  |  |  |  |  | <11.52 |  |  |  |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | 27.47 | 30 | <5.58 |  | 3.34 | 3.47 | ≥5.58 | >1.44 | 10.29 | 10.29 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | 30.5 | - | <5.58 |  | 6.53 | - | ≥5.58 | >1.44 | 13.94 | - |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | 34 | - | <5.58 |  | 8.38 | - | ≥5.58 | >1.44 | 15.97 | - |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | 34.5 | - | <5.58 |  | 8.62 | - | ≥5.58 | >1.44 | 16.34 | - |

No highlight: TN 38.101-1 NS_24 QPSK A-MPR value

Yello highlight: simulated A-MPR NS_24 PC2 1Tx QPSK

Sky highlight: simulated A-MPR NS_24 PC1.5 1Tx QPSK

Green highlight: simulated A-MPR NS_24 PC1 1Tx QPSK

Orange highlight: simulated A-MPR NS_24 PC1 4Tx QPSK

The A-MPR region for NS_24 can be determined by factors such as IMD3, PA characteristics, and filtering effects. While output power increases may influence emission levels, they are not expected to significantly alter the A-MPR region itself. Therefore, reusing the existing region definitions is a reasonable approach, as it can reduce the time and effort required for further discussions and standardization. Additionally, by sampling a few representative simulation points within the already defined region, it is possible to derive appropriate MPR values. This method can also reduce the overall simulation workload while maintaining consistency with existing definitions.

It was found that inconsistent results occurred when comparing the HPUE simulation outcomes with those of PC3.For example, in the case of PC3 with A5 A-MPR, the PC2 QPSK A-MPR simulation results show a very large variation ranging from 30 to 1.28 dB. Therefore, it may be difficult to directly apply the corresponding A-MPR value. However, the difference between the simulated A-MPR values of PC2 and PC1.5 is 4 to 1.7 dB, and the difference between PC2 and PC1 is 6.3 to 3 dB. In many cases, we observed a tendency where the delta corresponds to the total increased power. It is proposed that, when defining the HPUE A-MPR value, the difference between the PC3 A-MPR value and the HPUE A-MPR value should not significantly deviate from the total increased power.

Proposal 4: Reuse the NS_24 A-MPR region for NTN PC1, PC1.5 and PC2

NS_02N A-MPR

The NS_02N A-MPR simulation was conducted in PC2, reusing the PA model and calibration settings previously applied in the MPR simulations. Two sets of simulations were conducted: one considering a 10 dB additional filter and the other without it.

Table 5 NS_02N A-MPR simulation for DFT-s-OFDM QPSK

| 5MHz | 1629 < Fc ≤ 1654 | (0,11) | (0,1) |
| --- | --- | --- | --- |
| PC2 W/O 10dB |  | 0 | 1.47 |
| PC2 W 10dB |  | 0 | 1.47 |
| 10MHz | 1631.5 < Fc ≤ 1641.5 | (0,24) | (0,1) |
| PC2 W/O 10dB |  | 0 | 2.09 |
| PC2 W 10dB |  | 0 | 2.09 |
| 10MHz | 1641.5 < Fc ≤ 1655.5 | (0,24) | (0,1) |
| PC2 W/O 10dB |  | 0 | 2.03 |
| PC2 W 10dB |  | 0 | 2.03 |
| 15MHz | 1634 < Fc ≤ 1649 | (0,38) | (0,1) |
| PC2 W/O 10dB |  | 0.31 | 4.03 |
| PC2 W 10dB |  | 0.31 | 4.03 |
| 15MHz | 1649 < Fc ≤ 1653 | (0,38) | (0,1) |
| PC2 W/O 10dB |  | 0.34 | 4.16 |
| PC2 W 10dB |  | 0.34 | 4.16 |
| 20MHz | 1636.5 < Fc ≤ 1640.5 | (0,19) | (0,51) | (0,1) | (50,1) |
| PC2 W/O 10dB |  | 0 | 1.19 | 3.69 | 3.28 |
| PC2 W 10dB |  | 0 | 0.38 | 3.69 | 3.28 |
|  | 1640.5 < Fc ≤ 1650.5 | (0,38) | (0,1) | (0,1) | (50,1) |
| PC2 W/O 10dB |  | 0 | 0.38 | 3.72 | 3.09 |
| PC2 W 10dB |  | 0 | 0.38 | 3.72 | 3.09 |

Red highlight : NS_02N limiting factor

According to the simulation results, only the case of PC2 without the 10 dB filter at 20 MHz bandwidth and center frequency of 1636.5 MHz (RBstart: 0, LCRB: 51) was determined by NS_02N for the A-MPR value. In all other cases, the A-MPR value was determined by factors other than NS_02N, such as ACLR or SEM. Furthermore, when the 10 dB filtering was applied, none of the scenarios showed A-MPR values defined by NS_02N

Table 6 NS_02N A-MPR simulation

| 20MHz | 1636.5 < Fc ≤ 1640.5 | (0,51) |
| --- | --- | --- |
| DFT-s-OFDM | QPSK | 1.19 |
|  | 16QAM | 1.41 |
|  | 64QAM | 1.44 |
| CP-OFDM | QPSK | 1.91 |
|  | 16QAM | 1.94 |
|  | 64QAM | 1.91 |

Additionally, further simulations were conducted as shown below to determine the A-MPR region.

Table 7 NS_02N A-MPR simulation

| 20MHz | 1636.5 < Fc ≤ 1640.5 | (0,51) | (0,49) | (0,47) | (0,45) | (0,43) | (0,41) | (0,39) | (0,37) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | QPSK | 1.19 | 0.97 | 0.72 | 0.41 | 0.34 | 0.38 | 0.41 | 0.41 |

Red highlight : NS_02N limiting factor

Proposal 5: NS_02N A-MPR for PC2 can be discussed based on table 6 and table 7.

Table 6.x.x.x : A-MPR for NS_02N

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 20MHz | 1990 < Fc ≤ 1995 |  | >16.2 | A1 |

Table 6.: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 |
| --- | --- | --- |
|  |  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | DFT-s-OFDM PI/2 BPSK | 1.19 |
| DFT-s-OFDM QPSK | DFT-s-OFDM QPSK | 1.19 |
| DFT-s-OFDM 16 QAM | DFT-s-OFDM 16 QAM | 1.41 |
| DFT-s-OFDM 64 QAM | DFT-s-OFDM 64 QAM | 1.44 |
| CP-OFDM QPSK | CP-OFDM QPSK | 1.91 |
| CP-OFDM 16 QAM | CP-OFDM 16 QAM | 1.94 |
| CP-OFDM 64 QAM | CP-OFDM 64 QAM | 1.91 |

Conclusion

Proposal 1: PC1 1Tx MPR value can be discussed based on Table 1.

Table 1 1Tx PC1  MPR simulation results for 20 MHz CBW

| PC1 1Tx |  | edge |  |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,6) | (0,8) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 10.03 | 6.78 | 6.28 | 0.59 | 0 | 0 |
| 16QAM |  | 10.12 | 7.03 | 6.53 | 0.91 | 0 | 0 |
| 64QAM |  | 10.16 | 7.03 | 6.34 | 0.91 | 0 | 0 |
| QPSK | CP-OFDM | 10.84 | 8.28 | 7.19 | 2.03 | 0 | 0 |
| 16QAM |  | 10.56 | 8.28 | 7.28 | 2.03 | 0 | 0 |
| 64QAM |  | 11.03 | 8.63 | 7.28 | 2.03 | 0 | 0 |

Proposal 2: PC1 4Tx MPR value can be discussed based on Table 2.

Table 2 4Tx PC1  MPR simulation results

| PC1 4Tx |  | edge |  |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,6) | (0,8) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 10.41 | 7.25 | 6.78 | 2.56 | 0 | 0 |
| 16QAM |  | 10.34 | 7.34 | 6.69 | 2.59 | 0 | 0 |
| 64QAM |  | 10.41 | 7.22 | 6.47 | 2.56 | 0 | 0 |
| QPSK | CP-OFDM | 11.06 | 8.53 | 7.59 | 4.28 | 0 | 0 |
| 16QAM |  | 11.03 | 8.53 | 8.03 | 4.28 | 0 | 0 |
| 64QAM |  | 10.78 | 8.62 | 7.78 | 4.28 | 1.88 | 0.94 |

Proposal 3: PC1.5 1Tx MPR value can be discussed based on Table 3.

Table 3 PC1.5 MPR simulation results

| PC1.5 1Tx |  | edge |  | outer | inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (0,1) | (0,2) | (0,106) | (27,52) | (12,1) |
| QPSK | DFT-s-OFDM | 8.38 | 7.56 | 0.81 | 0 | 0 |
| 16QAM |  | 8.44 | 7.53 | 0.81 | 0 | 0 |
| 64QAM |  | 8.53 | 7.44 | 0.78 | 0 | 0 |
| QPSK | CP-OFDM | 9.03 | 8.09 | 2.31 | 0 | 0 |
| 16QAM |  | 9.03 | 8.16 | 2.34 | 0 | 0 |
| 64QAM |  | 9.09 | 8.06 | 2.31 | 0 | 0 |

Proposal 4: Reuse the NS_24 A-MPR region for NTN PC1, PC1.5 and PC2

Proposal 5: The difference between the PC3 A-MPR value and the HPUE A-MPR value should not significantly deviate from the total increased power.

Proposal 6: NS_02N A-MPR for PC2 can be discussed based on table 6 and table 7.

Table 6.x.x.x : A-MPR for NS_02N

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 20MHz | 1990 < Fc ≤ 1995 |  | >16.2 | A1 |

Table 6.2.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 1.19 |
| DFT-s-OFDM QPSK | 1.19 |
| DFT-s-OFDM 16 QAM | 1.41 |
| DFT-s-OFDM 64 QAM | 1.44 |
| CP-OFDM QPSK | 1.91 |
| CP-OFDM 16 QAM | 1.94 |
| CP-OFDM 64 QAM | 1.91 |

- References

# [1] R4-2508008, WF on HPUE requirments for NR-NTN (Samsung)