3GPP TSG-RAN WG4 Meeting #116																		R4-2510807

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.5

Source: 	OPPO

Title: 	RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Approval

- Discussion

In RAN1 #110bis meeting, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 		UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In RAN #107 meeting, case 2a and case 2b have been removed from the Revised WID[1], and following objectives have been agreed.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In above cases, now as agreed in RAN4 #110 meeting, RAN4 will not define positioning accuracy requirements for case 3a/3b. Case2a/2b have been removed. And for case1, in RAN1 #115 meeting, following agreements have been made,

Agreement(for case1):

- Discuss how to define a measurement delay requirement(reporting delay)

- Do not define any positioning accuracy requirement

So, RAN4 does not need to define positioning accuracy requirement for AI/ML based Positioning accuracy enhancement.

Observation 1: RAN4 does not define positioning accuracy requirements for AI/ML-based positioning accuracy enhancement.

For core requirements, further consideration is required on how to define the delay requirement (reporting delay)  for Case 1 and Case 3.

- Case 1: Since UE needs to conduct both the measurement and AIML-based positioning processing before reporting in Case 1, RAN4 should account for these factors when defining the delay requirement. This includes the impact of measurement(PRS measurement) and AI/ML processing (may involve model activation delay and inference delay).

- Case 3: For Case 3, there are no significant changes to the UE’s measurement reporting process, nor does the UE perform additional AI inference tasks. Therefore, RAN4 does not need to impose additional measurement delay requirements (reporting delay) specifically for Case3.

Proposal 1: For case1, the impact of measurement(PRS measurement) and AI/ML processing (may involve model activation delay and inference delay) should be taken into account when defining the delay requirement.

Proposal 2: For case3, RAN4 does not need to impose additional measurement delay requirements (reporting delay) specifically for AI/ML based Positioning enhancement.

- Conclusions

In this contribution, we have got the following proposals:

Observation 1: RAN4 does not define positioning accuracy requirements for AI/ML-based positioning accuracy enhancement.

Proposal 1: For case1, the impact of measurement(PRS measurement) and AI/ML processing (may involve model activation delay and inference delay) should be taken into account when defining the delay requirement.

Proposal 2: For case3, RAN4 does not need to impose additional measurement delay requirements (reporting delay) specifically for AI/ML based Positioning enhancement.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2