3GPP TSG-RAN WG4 Meeting #116	R4-2510832

Bengaluru, India, 25 August – 29 August 2025

Agenda Item:	7.4.6

Source:	Ericsson

Title:	UE demodulation requirements for 7MHz CBW

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on the UE demodulation and CSI requirements for 7MHz [1], where RAN4 agreed to define CA PDSCH demodulation requirements with 7MHz CBW. This contribution provides our simulation results to complete the performance part.

# 2	Simulation results

Table 1 summarizes our simulation results for 7MHz CBW.

Table 1	Simulation parameters for PDSCH demodulation for 7MHz CC

| Band width | SCS | MCS | Rank | TBS | Propagation condition | Antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7MHz (35 RBs) | FDD 15kHz | 16QAM, 0.48 | 2 | 17424 | TDLA30-10 | 2x2, ULA Low | 10.4 | 12.4 |
| 7MHz (35 RBs) | FDD 15kHz | 16QAM, 0.48 | 2 | 17424 | TDLA30-10 | 2x4, ULA Low | 6.8 | 8.8 |

# References

- R4-2508757, “WF on demodulation and CSI requirements for 7MHz”, Huawei, HiSilicon