3GPP TSG-RAN WG4 Meeting #116	R4-2510866

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Discussion on less than 5MHz for NR NTN SAN demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.6.4.6

Document for:	Discussion

Background

During RAN4#115 meeting, WF [1] for NR IoT NTN less than 5MHz demod was approved. In this contribution, we share our view on less than 5MHz for NR NTN SAN demodulation performance requirements.

Discussion

Applicability rules for PRACH requirements

Options: - Option 1: Introduce applicability rules for PRACH - Proposal 1a: For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply - Option 2: No applicability rule is needed for PRACH

Since there is no any PRACH requirements with LRA=1139 in current TS 38.108, we don’t think such applicability rule is needed for NTN.

Do not introduce the test applicability rule for PRACH.

Proposals

In this contribution, we share our view on less than 5MHz for NR NTN SAN demodulation performance requirements. Our observations and proposals are:

- Do not introduce the test applicability rule for PRACH.

Reference

R4-2508725, WF for NR IoT NTN less than 5MHz demod, RAN4#115, Ericsson