3GPP TSG-RAN WG4 Meeting #116	R4-2510869

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Discussion on NR NTN Phase 3 demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.27.7

Document for:	Discussion

Background

During RAN4#115 meeting, Way Forward [1] for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved. In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements.

Discussion

UE side

Test case of PDSCH for RedCap 1Rx

Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ - MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes - MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x1, 52RBs - MCS13, NTN-TDLC5-200, 1x1, 52RBs - Option 3: Similar test configurations as the existing PDSCH cases

For RedCap 1Rx UE, we propose to use the same test configurations as the existing Rel-17 NTN PDSCH cases, otherwise there is different test coverage for RedCap 1Rx UE and RedCap 2Rx UE.

For RedCap 1Rx UE, use the same test configurations as the existing Rel-17 NTN PDSCH cases.

- MCS4, NTN-TDLA100-200, 1x1, 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

Test case of PDSCH for eRedCap 1Rx

Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes - MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes - MCS13, NTN-TDLA100-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RBs) - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RBs) - Option 3: Similar test configurations as the existing PDSCH cases

For eRedCap 1Rx UE, we propose to define following cases to ensure the test coverage, otherwise there will be no test cases for some NTN features.

For eRedCap 1Rx UE, define following cases with test applicability rule for Cases for MCS 13 with 16 HARQ processes with and without reduced BB BW, i.e. If UE pass the case without reduced BB BW, the case with reduced BB BW can be skipped.

- MCS4, NTN-TDL A100-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

Test case of PDSCH for eRedCap 2Rx

Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RB), Disabled HARQ - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes - MCS4, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 32 HARQ processes - MCS13, NTN-TDLA100-200, 1x2, without reduced BB BW (40RBs), 16 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RBs) - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RBs) - Option 3: Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

For eRedCap 2Rx UE, we propose to define following cases to ensure the test coverage, otherwise there will be no test cases for some NTN features.

For eRedCap 2Rx UE, define following cases with test applicability rule as stated in Proposal 2 for eRedCap 1Rx.

- MCS4, NTN-TDL A100-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

Applicability of time-varying Doppler shift and propagation delay model for NGSO

Way forward: - Option 1: Apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements. - Option 2: Not to apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

Since both topics are in the same release, we propose to not apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

Not apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

SAN side

OCC parameters

Agreement: - Apply inter-slot OCC length 2. - Apply PUSCH type A repetitions Way forward: - FFS whether to define PUSCH requirements with inter-slot OCC length 4.

Based on RAN1 agreements, inter-slot OCC length 2 and inter-slot OCC length 4 is supported for DFT-s-OFDM PUSCH repetition Type A is supported, and UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. So we propose to only consider inter-slot OCC length 2 since it is enough to verify SAN performance under this feature.

Only consider inter-slot OCC length 2

Detailed PUSCH parameters

Way forward: - Assume to set the following parameters: - FDD SCS=15KHz with 5MHz CBW and FDD SCS=30KHz with 10MHz CBW - Both 1Rx and 2Rx, with test applicability rule for different Rx number - Same MCS for two UEs for PUSCH with OCC - Same channel model for two UEs for PUSCH with OCC - Frequency offset between two UEs - Baseline assumption: same frequency offset for two UEs in one OCC group. FFS what frequency offset value is assumed. - FFS whether to consider different frequency offsets values for two UEs additionally. - FFS for other parameters.

For the frequency offset, we think baseline assumption is enough. If the frequency offset value difference between two UEs are large, it is not reasonable for SAN to schedule such two UEs in one OCC group. For the frequency offset value, 0.1 ppm frequency offset can be regard as the baseline that is same as the RF requirements.

Only consider same frequency offset for two UEs in one OCC group, 0.1 ppm frequency offset can be regard as the baseline.

For the other parameters, the Table 8.2.2.1-1 in TS 38.104 can be reused unless otherwise stated.

For the other parameters, the Table 8.2.2.1-1 in TS 38.104 can be reused unless otherwise stated.

Proposals

In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements. Our observations and proposals are:

- For RedCap 1Rx UE, use the same test configurations as the existing Rel-17 NTN PDSCH cases.

- MCS4, NTN-TDL A100-200, 1x1, 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

For eRedCap 1Rx UE, define following cases with test applicability rule for Cases for MCS 13 with 16 HARQ processes with and without reduced BB BW, i.e. If UE pass the case without reduced BB BW, the case with reduced BB BW can be skipped.

- MCS4, NTN-TDL A100-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

For eRedCap 2Rx UE, define following cases with the same test applicability rule as stated in Proposal 2.

- MCS4, NTN-TDL A100-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

Not apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

Only consider inter-slot OCC length 2.

Only consider same frequency offset for two UEs in one OCC group, 0.1 ppm frequency offset can be regard as the baseline.

For the other parameters, the Table 8.2.2.1-1 in TS 38.104 can be reused unless otherwise stated.

Reference

R4-2508755, Way Forward for [115][333] NR_NTN_Ph3_demod, RAN4#115, Ericsson