# 3GPP TSG-RAN WG4 Meeting #116	R4-2510914

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	7.23.3.2

Source: 	Ericsson

Title:	Remaining issues on SSB adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 achieved the following agreements about SSB adaptation as follow[1].

### Issue 1-1-1: Processing time for SSB adaption ### Agreement ### RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. ### FR1: 2ms FR2: 3ms ### FFS L3 transition period measurement ### Issue 1-2-2: L1 requirements after transition - DRX ### Agreement: ### For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period ### Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition ### Agreement: ### No spec. impact is identified for releasing the measurement results. ### It’s up to UE implementation. ### Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell ### Agreement ### RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement. ### The cell identification and measurement period requirements apply based on the longer period before or after the transition. ### Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell ### Agreement ### Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity. ### Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related ### Agreement: ### For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

In this contribution, we continue to discuss the remaining issues on the Rel-19 NES operation in Obj. 3 from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

L3 measurement – deactivated SCell

In last meeting, RAN4 agreed the L3 measurement requirement for an active serving cell. The open issue is the requirement for deactivated SCell. In our understaindg, the deactivated SCell measurement is based on measCycleSCell. Different with OD-SSB activation/deactivation, SSB adaptation cannot disable the SSB. We don’t think RAN4 shall define the same fast UE behaviour in two different features. Therefore, we prefer to follow legacy measurement framework based on measCycleScell.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell Agreement - Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity. For deactivated SCell, discuss following options: - Option 1: legacy framework based on SMTC and measCycleScell - Option 2: reuse same framework as OD-SSB case#2.

Proposal 1: After UE receiving SSB adaptation indication, UE still follows legacy deactivated SCell measurement framework based on measCycleScell.

L3 measurement – collision with MG

One of the remaining issues for SSB adaptation is how to handle the adapted SMTC collision with configured MG. In RAN2 #129b meeting, RAN2 introduced multiple SMTC configurations linked with adapted SSBs. From our understanding, the general Rel-15 principles shall not be changed due to any SSB/SMTC adaptation. In other words, the MG based measurement should have the highest priority. After SSB adaptation, if the updated SMTC is fully overlapped with MG, UE shall perform the adapted SMTC measurement within gap; otherwise, UE shall perform the adapted SMTC measurement outside gap.

RAN2 #129b agreement - For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. - Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly

Proposal 2: After SSB adaption, if the updated SMTC is collided with MG, the legacy MG collision rule is reused.

- When adapted SMTC is fully overlapping with MG, UE performs the measurement within gap;

- Otherwise, UE performs the measurement outside gap

L1 Requirement – measurement restriction

In last RAN4 meeting, another open issue is how to handle the collision between adapted SSB with CSI-RS. From our understanding, the legacy measurement resitrcition rule can also be reused. When adapted SSB collides with CSI-RS, existing L1 measurement restriction can also apply.

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Proposal 3: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply.

- Summary

In this contribution, we have discussed the SSB adaptation requirements. Based on the discussions, we made the following proposals and observations:

Proposal 1: After UE receiving SSB adaptation indication, UE still follows legacy deactivated SCell measurement framework based on measCycleScell.

Proposal 2: After SSB adaption, if the updated SMTC is collided with MG, the legacy MG collision rule is reused.

Proposal 3: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply.

- References

- R4-2508278, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei