# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510985

# Banalore, India, 25th – 29th August 2025

# Title: 	Discussion on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.4

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

In the following section, we will provide the detailed discussions for 6Rx UE demodulation requirements.

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have some open issues[2]. Next, we would like to address in the following for 6Rx UE demodulation requirements.

General

Test applicability rule for 6Rx test cases

- Proposals - Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2. - Option 2: Introduce test applicability rule only for 6 layers cases - Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design

Regarding test applicability rule, this will affect further testing options for 6Rx UE, which has different devices to support different layers. According to previous meeting conclusion, only FWA devices can support 6 layers, and 4 layers can apply to FWA and handled UE devices. Starting from this conclusion, RAN4 may need to define different demodulation requirements for different devices. Currently, RAN2 has not yet made a decision on this signalling. Therefore, we propose to postpone the discussion after RAN2 finished the UE capability signalling design.

Proposal 1. Propose to postpone the discussion after RAN2 finished the UE capability signalling design.

Specification structure

- Proposals - Option 1: Add a new clause for 6Rx - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

In legacy specification, RAN4 created new sections for 2Rx, 4Rx, and 8Rx. We prefer to follow the same approach for 6Rx.

Proposal 2. Propose to modify the current section and put 6Rx requirements between 4Rx and 8Rx.

MIMO correlation for 4 MIMO layers for Handheld UE testing

- Agreement - Introduce correlation matrix for 6Rx in the specification: RUE6Rx=1β125β425β925β1625ββ125∗1β125β425β925β1625β425∗β125∗1β125β425β925β925∗β425∗β125∗1β125β425β1625∗β925∗β425∗β125∗1β125β∗β1625∗β925∗β425∗β125∗1 - Proposal - Option 1: with α = 0 and β = 0.07179 - Option 2: with α = 0.3 and β = 0.07179 - Option 3: with α = 0 and β = 0

As discussed before, due to the limited size for handle UE, MIMO correlation shall be considered in evaluation. However, during the feasibility phase, companies measured the correlation from handle devices, which can be very lower than current definition. Regarding the gNB correlation, we prefer to consider low correlation. Therefore,we prefer to consider option 1 or option 3 as handle UE correlation assumption.

Proposal 3. Propose to consider option 1 or option 3 for handle UE correlation assumption.

Test cases for 4 MIMO layers for FWA and Handheld UE

- Proposals - Option 1: Define one single test case - Option 1a: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 1b: One single test case for 4 MIMO layers corresponding to a Handheld UE - Option 2: Define two sets of 4 layers tests with different MIMO correlation

In the last meeting, RAN4 agreed to introduce a single test case for 6 MIMO layers for FWA device. Regarding 4 MIMO layers, to reduce the testing burden, we also prefer to define a single test case, which means all potential assumptions shall be aligned with handled UE scenario.

Proposal 4. Propose to define one test case for 4 MIMO layers corresponding to handled UE.

MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

- Agreement: Down select the following two options in the next RAN4 meeting - Option 1: MCS 20 (Table 2) - Option 2: MCS 17 (Table 1)

Regarding the MCS, we prefer to consider MCS 17.

On the other side, we evaluated the performance of MCS 17 and MCS 20 in the following Figures. From our simulation results, the 70% maximum throughput of SNR points are about 12.6dB and 23.4dB for MCS 17 and MCS 20 (Table 2) under low correlation condition. Considering the impairment results and margin, the SNR point could be a litter higher for MCS 20. Therefore, we prefer to consider MCS 17 for 4 MIMO layer cases.

| Figure 1. TDD 30kHz 4T6R MCS 17 TDLA30-10 ULA Low | Figure 2. TDD 30kHz 4T6R MCS 20 TDLA30-10 ULA Low |
| --- | --- |

Proposal 5. Propose to only consider MCS 17 for 4 MIMO layers.

DMRS configuration for 6 MIMO layers case

- Proposals - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-18 eType1 DMRS 1+1

In the previous feasibility phase, RAN4 evaluated different DMRS configurations, which can bring different system throughput gain. And now, from the perspective of the testing, Rel-18 DMRS configuration is a optional feature, not all UEs could support this default configuration. In RAN4 legacy requirements, DMRS 2+2 is also used in Rel-18 8Rx  requirements definition. Hence, we prefer to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 6. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

CSI requirements for 6Rx

CQI test for 6Rx

- Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel

It is clearly mentioned in WID[1] that RAN4 needs to specify CSI requirements to support 6Rx, from our point of view, it is better to define CSI requirements in this item to follow the RAN plenary decision.

Proposal 7. Propose to define one CQI requirements with rank4 under static channel.

RI test

- Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

Regarding RI test, which is used to verify that the reported rank indicator accurately represents the channel rank. For FWA and handled UE 6Rx, we did not find it necessary to define requirements, since we also did not define requirements for 8Rx.

Proposal 8. Propose to do not define RI requirements for 6Rx.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx requirements, The conclusions are:

Proposal 1. Propose to postpone the discussion after RAN2 finished the UE capability signalling design.

Proposal 2. Propose to modify the current section and put 6Rx requirements between 4Rx and 8Rx.

Proposal 3. Propose to consider option 1 or option 3 for handle UE correlation assumption.

Proposal 4. Propose to define one test case for 4 MIMO layers corresponding to handled UE.

Proposal 5. Propose to only consider MCS 17 for 4 MIMO layers.

Proposal 6. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 7. Propose to define one CQI requirements with rank4 under static channel.

Proposal 8. Propose to do not define RI requirements for 6Rx.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2508727, Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon.