# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510987

# Banalore, India, 25th – 29th August 2025

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.12.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

Frequency Generalisation

Options Option 1: To select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition. Option 2: Use 3.5GHz for all FR1 bands and duplexing, following legacy and SCM alignment.

Regarding frequency for CDL channel model, from the generation process of CDL model, two aspects are impacted by frequency, DS and Doppler. Currently, two different CDL channel models are studied in RAN4, CDL Option 1(based on Table 7.7.1-3 in TR 38.901 without truncation) and Option 2 (based on Table 7.7.1-3 in TR 38.901 with truncation). For CDL Option 2, which is based on cluster reduction to generate new CDL angular and delay profiles. Once the frequency is changed, the profiles could be different and the demodulation performance could be different. From this point of view, changes of frequency have a significant impact on performance. Therefore, we think current option 1 is more reasonable to derive desired angular spread and doppler for TDD and FDD scenario.

Proposal 1. Propose to consider current option 1 approach to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler.

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements, The conclusions are:

Proposal 1. Propose to consider current option 1 approach to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2508776, Way Forward [115][321] NR_SCM, Nokia.