3GPP TSG-RAN WG4 Meeting #116	R4-2511001

Bangalore, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-1 on Adjacent channel and subcarrier selectivity for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction ACS and ASCS requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ACS and ASCS requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUR ACS and ASCS requirements are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

7.5M	Adjacent channel and subcarrier selectivity for LP-WUR

7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

7.5M.2	Adjacent channel selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 and Table 7.5M.2-2 for LP-WUR type 1 UE and the minimum requirements specified in Table 7.5M.2-3 and Table 7.5M.2-4 for LP-WUR type 2 UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.5M.2-1: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 23 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 42.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-2: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -47.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 17 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 47 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 5: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -53.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 and 7.5M.3-2 for LP-WUR. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.5M.3-1: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS+0.5 |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

Table 7.5M.3-1: ASCS parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 0 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

<<End of change>>