# 3GPP TSG-RAN WG4 Meeting #116                                R4-2511019

# Bangalore, India, 25th – 29th August, 2025

Source:  ZTE Corporation, Sanechips

Title:   Discussion on the RRM requirements aspects of AI/ML positioning

Agenda Item:	7.17.5

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some views on the interoperability and testability for AI positioning from the general test framework perspective.

- Discussion

In previous meeting, RAN4 has reached the related agreements for AI positioning as below:

RAN4 110 meeting: Issue 3-2: Requirements for case 3a/3b - RAN4 will not define positioning accuracy requirements for case 3a/3b RAN4 110bis meeting: Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme (if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced RAN4 112bis meeting: Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. RAN4 114 meeting: Issue 3-4: Report mapping for UL SRS-RSRP Agreement: - Reuse the report mapping for UL SRS-RSRP to Case 3a For non-AI, according to TS 38.133, measurement reporting mapping are specified for UL RTOA, UL SRS RSRP measurement and UL SRS RSRPP measurement, which can be reused for case 3a. RAN4 114bis meeting: Issue 3-2: LOS/NLOS indicator Agreement: the LOS/NLOS indicator is out of scope of WID. Stop discussions on it. Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement(reporting delay) - Do not define any positioning accuracy requirement

First of all, the following represent case 1:

Figure 1 Case 1

We observed that RAN4 will not define any positioning accuracy requirements but measurement delay which is reporting delay can be discussed.

In case 1 the AI/ML model is at UE side, that is, model inference is at UE side which is up to UE implementation. For direct UE-based positioning, the model output provides the predicted/estimated UE location. Case 1 is similar to the legacy UE-based positioning. In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. Some companies deem that there is no need to define accuracy requirements for case 1 since the UE triggers the location request and it is no need to report the location to NW which is up to UE implementation. If the location request is triggered by the network, the UE shall report its location. To my understanding, the above agreement contains the implicit reporting scheme which is the NW triggers the location request, otherwise no reporting scheme is needed. To our understanding, the reporting procedure is similar to legacy positioning, the description of legacy measurement reporting delay is as below:

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

Consequently, legacy reporting delay could be the baseline.

Observation 1: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Observation 2: The legacy measurement delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

Proposal 1: For AI positioning case 1, the legacy positioning reporting delay could be as baseline.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML positioning:

Observation 1: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Observation 2: The legacy measurement delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

Proposal 1: For AI positioning case 1, the legacy positioning reporting delay could be as baseline.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2505105, “WF on requirements for AI/ML air interface normative work”