3GPP TSG-RAN WG4#116	R4-2511055

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.1.2.1.1

Source: 	Qualcomm

Title: 	Considerations on HPUE MSD progress

Document for:	Discussion

# Introduction

This contribution provides views on the progress made in HPUE MSD during Rel-19, and how the progress can be further leveraged for the cases not yet agreed.

- Discussion

A lot of work has been done during Rel-19 to reduce the huge burden to specify HPUE band combinations. Making TP’s, and especially the MSD numbers, is burdensome for the proponents as well as for companies reviewing the submissions. Now in RAN4-116 the NR CA LUT and related CR is ready for approval so it’s good time to re-cap the overall situation.

HPUE LUT and associated requirements currently cover NR Inter-band CA with 1Tx, 2Tx, and 3Tx for 1CC per band. Currently the following cases have not been discussed and hence are not part of the framework.

- 1UL band Inter-band CA with 2CC on band A (IMD of contiguous or non-contiguous Intra-band CA hitting DL band)

- 2UL band Inter-band CA with 1CC on band A and 2CC on band B (Triple beat IMD of the 3CC’s hitting DL)

- Intra-band CA with 1UL CC and 2UL CC (1UL C cross-band noise hits SCC DL, or IMD of 2 UL CC’s hits PCC and SCC

- EN-DC

In our view, the great results of the HPUE LUT and associated requirements can be further used by porting those to at least EN-DC. There are some aspects which need to be accounted due to differences between how NR CA and EN-DC band combinations and supported power classes/number of TX’es are captured in the specifications, but they should be straightforward to address after NR CA part is stable. In addition to EN-DC, other cases mentioned above should be discussed as capturing as many of those under LUT framework would further reduce the amount of HPUE specification burden going forward.

Observation 1: Group has achieved significant progress in reducing the specification burden of HPUE band combinations during Rel-19.

Observation 2: Current HPUE LUT framework can be further leveraged by porting those at least to EN-DC and possibly to some other cases not yet discussed.

- Conclusion

Considerations on the progress on HPUE MSD during Rel-19 were provided with the following observations.

Observation 1: Group has achieved significant progress in reducing the specification burden of HPUE band combinations during Rel-19.

Observation 2: Current HPUE LUT framework can be further leveraged by porting those at least to EN-DC and possibly to some other cases not yet discussed.

- Reference