3GPP TSG-RAN WG4#116	R4-2511056

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	5.3.7

Source: 	Qualcomm

Title: 	Handling of Optional bandwidths across releases

Document for:	Approval

# Introduction

Handling of few optional bandwidths across releases is discussed.

- Background

WF was agreed in RAN4-115 [1]:

Agreement 1:

- 3MHz CBW:

  - For MSD analyses, do not consider 3MHz CBW except when the DL affected band is band n106.

  - Companies are invited to share their views at next meeting whether 3MHz CBW should become mandatory for bands n26, n28, n31, n72, n85 and n100.

  - Note: For band n28, one operator can only deploy 3MHz in this band.

- 5MHz CBW for band n41/n90:

  - For MSD analyses, consider 10MHz as the smallest DL CBW.

  - Companies are invited to further study whether 5MHz CBW for bands n41/n90 could be removed from Rel-19.

- 35MHz CBW for band n71

  - Companies are invited to further study whether 35MHz CBW for band n71 should become mandatory support.

- Discussion

Agreement 1:

3MHz CBW:

Our view is that 3MHz should not be mandated as that would then be mandatory for all devices tiers and types, and it would also impact to a number of optional features related to support of 3MHz BW.

Proposal 1: Keep 3MHz optional across Releases for n26, n28, n31, n72, n85 and n100.

5MHz CBW for band n41/n90:

In our understanding, 5MHz for n41/n90 is not deployed anywhere e.g. due to 30kHz SCS not supporting 5MHz BW’s. Our preference is to keep 5MHz for n41/n90 in the specifications as it should not harm anything. We are ok to remove it, if removal does not break anything in other specifications than 38.101. We note, however, that there are several tens of band combinations whose MSD is specified using 5MHz for n41 so if 5MHz n41/n90 is removed, MSD modifications must be done at the same time.

Proposal 2: Keep 5MHz CBW for band n41/n90. If removal is confirmed not to cause issues outside 38.101 then MSD modifications need to be done at the same time with removal of 5MHz for n41/n90.

35MHz CBW for band n71:

Due to heavy desense warranted by 3GPP, our view is that it may not be needed to mandate this unless operator has clear plans to use 25/30/35MHZ UL BW’s for n71. We are certainly open for discussions during the meeting.

Proposal 3: Keep 35MHz n71 UL optional across releases unless otherwise agreed in discussions with interested parties

- Conclusion

Considerations on handling of optional bandwidths across releases were provided with the following proposals.

Proposal 1: Keep 3MHz optional across Releases for n26, n28, n31, n72, n85 and n100.

Proposal 2: Keep 5MHz CBW for band n41/n90. If removal is confirmed not to cause issues outside 38.101 then MSD modifications need to be done at the same time with removal of 5MHz for n41/n90.

Proposal 3: Keep 35MHz n71 UL optional across releases unless otherwise agreed in discussions with interested parties.

- Reference

[1] R4-2508050, “WF on optional chanel bandwidth across releases”, Skyworks