3GPP TSG-RAN WG4 Meeting #116                                        R4-2511083

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:              Discussion on performance requirements for NES

Agenda item:			7.23.4

Document for:	Discussion

1. Introduction

In this meeting, we will give some initial views on the test case scope for NES OD-SSB and SSB adaptation based on WF [1] in last meeting, separately.

2 Performance requirements for NES OD-SSB

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

In the core part, we define some new OD-SSB specific requirements with OD-SSB configuration/activation/deactivation/adaptation mechanism introduced for the following:

- OD-SSB based L1 Measurement procedure (during SCell activation / after SCell activation)

(L1-RSRP/L1-SINR)

  - SSB periodicity of OD-SSB

  - CSI report configuration was discussed in RAN1

- OD-SSB based L3 Measurement procedure

  - Deactivated SCell measurement requirement (case #1/ #2 )

    - Case #1: (Fast mode [FWM] + slow mode) , T2

    - Case #2 : Legacy requirements based on measCycleSCell +condition; Otherwise, case # 1 framework

      - Alt Time-C1, Time-C2 (pending)

  - Intra-frequency measurements

- OD-SSB based SCell activation (Case #1/#2 and Scenario #2/#2A)

  - Known condition

  - SCell activation delay requirement

- OD-SSB general issues

  - Interruption (scenario #2/ scenario #2A, in both Case 1 /Case 2)

    - OD-SSB activation/deactivation; FMW

  - Additional processing time

  - Scenario 3 (pending)

- Capability

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

From the core requirement and the corresponding agreements side:

- New TCs for SSB-based L1-RSRP measurements for Reporting are needed for NES OD-SSB, the impacted factor is Measurement period TL1-RSRP_Measurement_Period_SSB. At the same time, RAN1 reached consensus on CSI report configuration for case #1 and case # 2, separately. And they have already clearly specified how to define a CSI report configuration for the situation: 1) timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”; 2) Do not support periodic CSI reporting based on OD-SSB, for Case #1 and Case #2 when a CSI report configuration configured with reportConfigType set to periodic

From this, we have the following proposal:

Proposal 1: RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES OD-SSB

- New TCs corresponding to Case #1 and Case #2, separately, to verify the measurement procedure when OD-SSB transmission is indicated by NW

  - The test has higher layer parameter timeRestrictionForChannelMeasurements configured

  - If general test parameters: reportConfigType is set to periodic, periodic CSI reporting based on OD-SSB, for Case #1 and Case #2 is not supported

  - non-DRX is configured is with priority

  - FR1 and FR2-1 are both considered

- For SCell activation, whether the target SCell supporting OD-SSB can be known or unknown to UE should be discussed case-by-case, especially,

- For case 1: Although SCell activation delay requirement shall apply based on OD-SSB periodicity, the deactivated SCell measurement procedure and the interruption may be involved.

- For case 2: When UE receives the OD-SSB activation indication

- EMP issue (RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario)

Proposal 2: RAN4 to introduce new tests for SCell Activation and Deactivation Delay requirements for NES OD-SSB in FR1 and FR2, separately.

- Test cases for known SCell and unknown SCell should be covered

- The test cases involve case #1 and case #2

- Scenario #2 and scenario #2A should be considered

- The test type can contain at least:

  - SCell Activation and deactivation for SCell

  - Direct SCell activation at SCell addition

  - L3 reporting during activation

  - PUCCH SCell activation and deactivation

  - Fast SCell Activation Delay Requirement for Deactivated SCell

- Verify the EMP requirement in case 2 Alt Time-C1

- FFS: Whether interruption at OD-SSB activation/deactivation for scenario 2A can be merged

- New interruption requirements are defined for scenario 2 due to OD-SSB activation/deactivation, the new interruption requirement is related to the definition of FMW.

In this sense,

Proposal 3: RAN4 can to define new test case for interruption verification

- During measurements on deactivated SCC for NES OD-SSB

- How to design the test case depends on the definition of condition, duration and position of the interruption

- Based on the agreements we achieved

Issue 4-3: OD-SSB accuracy Agreement: - The legacy measurement accuracy is applied for OD-SSB based measurement.

From our understanding, there is no need to define corresponding test case to verify the measurement performance for NES OD-SSB

Proposal 4: No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB

- The core requirement of OD-SSB based NR intra-frequency measurement was agreed to define. Meanwhile, based on the RAN2 OD-SSB specific design, below

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

In this sense, the L3 measurement procedure of OD-SSB based measurement should be tested, particularly the parameter updates case. Namely, UE to receive the MAC-CE OD-SSB periodicity updating/adaptation indication in scenario #3B and to perform L3 measurement based on OD-SSB specific SMTC (i.e., additional SMTC configuration according to SSB adaptation).

- Discussion on how to/whether to define requirements for scenario 3 (OD-SSB and AO-SSB are within different frequencies) was just put on the agenda.

- The frequency location of OD-SSB is the same as the frequency location of AO-SSB at least for the case where AO-SSB is NCD-SSB

In short, we have the following proposals:

Proposal 5: RAN4 prioritize to introduce new test cases for intra-frequency measurement requirements for NES OD-SSB, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered

3 Performance requirements for NES SSB adaptation

Issue 3-3-1: Test case list RAN4 to discuss whether the following SSB adaptation test case list can be agreed. - L1-RSRP - L1-SINR - CBD - L3 measurement - Deactivated SCell measurement - normal SCell activation - direct SCell activation - PUCCH SCell activation Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection Issue 3-3-3: SMTC and SSB configuration - Proposals - Option 1: - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation. - Option 2: - SSB periodicity: 40 ms to 20 ms - SMTC periodicity: 40ms - Option 3: - SSB periodicity from 80ms to 20ms - SMTC periodicity from 80ms to 20ms

- Measurement performance

Same proposal can be given here

Proposal 6: No new test case for A.7.7 Measurement Performance requirements for NES SSB adaptation

- L1 Measurement procedure

L1-RSRP measurement on SCell were defined with the consideration of adapted SSB periodicity, which is triggered/indicated by NW signalling DCI 2_9. And for adaptation of SSB in time-domain, the UE can be configured with at most 2 additional SSB burst periodicities (from network energy saving, normal RRM measurement, low-latency SCell operation), which is different from what the periodicity can to be configured for OD-SSB. Besides, for L1-SINR, the functionality difference to legacy SSB based measurement can be verified by L1-RSRP test, so there is no need to define additional L1-SINR one. From this, we have the following proposal:

Proposal 7: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation

- New TC is to verify the measurement procedure when SSB periodicity adaptation is activated by NW signalling DCI 2_9

- non-DRX is configured is with priority

L3 Measurement procedure

Based on RAN1 agreement, SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. The first SSB burst after SSB adaptation according to the SSB burst periodicity after adaptation and associated SFN offset and half-frame index.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell. And mapping relationship between X additional SSB burst periodicity with Y additional SMTC configuration(s), provided via RRC

- The following scenario: Frequency location is same before and after SSB adaptation; No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation are supported for SSB adaptation; T SSB adaptation in SCell for NCD-SSB.

Proposal 8: RAN4 to introduce new test cases for intra-frequency measurement requirements for NES SSB adaptation, the test case may capture below:

- Without SSB time index detection

- SA event triggered reporting test without gap under DRX with NCD-SSB as starting point

SMTC and SSB configuration

From our understanding, based on RAN1 agreement that for adaptation of SSB in time-domain, UE can be configured with X (<=2) additional SSB burst periodicities for a SCell. The following candidate values can be taken into consideration to cater to different aims, e.g., typical default SSB periodicity: 20 ms for normal RRM measurement; Larger value: 80/160 ms for energy saving gain acquisition; Denser value: 5/10 ms for fast SCell measurement or SCell activation. From this, we have the following proposal:

Proposal 9: TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation

- Typical default SSB periodicity: 20 ms

- To larger SSB periodicity: 80/160 ms; To denser SSB periodicity: 5/10 ms

4 Conclusion

In this contribution, we provided our viewpoints on RRM performance requirements for NES OD-SSB and SSB adaptation. The following observations and proposals are obtained:

Performance requirements for NES OD-SSB

Proposal 1: RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES OD-SSB

- New TCs corresponding to Case #1 and Case #2, separately, to verify the measurement procedure when OD-SSB transmission is indicated by NW

  - The test has higher layer parameter timeRestrictionForChannelMeasurements configured

  - If general test parameters: reportConfigType is set to periodic, periodic CSI reporting based on OD-SSB, for Case #1 and Case #2 is not supported

  - non-DRX is configured is with priority

  - FR1 and FR2-1 are both considered

Proposal 2: RAN4 to introduce new tests for SCell Activation and Deactivation Delay requirements for NES OD-SSB in FR1 and FR2, separately.

- Test cases for known SCell and unknown SCell should be covered

- The test cases involve case #1 and case #2

- Scenario #2 and scenario #2A should be considered

- The test type can contain at least:

  - SCell Activation and deactivation for SCell

  - Direct SCell activation at SCell addition

  - L3 reporting during activation

  - PUCCH SCell activation and deactivation

  - Fast SCell Activation Delay Requirement for Deactivated SCell

- Verify the EMP requirement in case 2 Alt Time-C1

FFS: Whether interruption at OD-SSB activation/deactivation for scenario 2A can be merged

Proposal 3: RAN4 can to define new test case for interruption verification

- During measurements on deactivated SCC for NES OD-SSB

- How to design the test case depends on the definition of condition, duration and position of the interruption

Proposal 4: No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB

Proposal 5: RAN4 prioritize to introduce new test cases for intra-frequency measurement requirements for NES OD-SSB, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered

Performance requirements for NES SSB adaptation

Proposal 6: No new test case for A.7.7 Measurement Performance requirements for NES SSB adaptation

Proposal 7: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation

- New TC is to verify the measurement procedure when SSB periodicity adaptation is activated by NW signalling DCI 2_9

- non-DRX is configured is with priority

Proposal 8: RAN4 to introduce new test cases for intra-frequency measurement requirements for NES SSB adaptation, the test case may capture below:

- Without SSB time index detection

- SA event triggered reporting test without gap under DRX with NCD-SSB as starting point

Proposal 9: TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation

- Typical default SSB periodicity: 20 ms

- To larger SSB periodicity: 80 ms; To denser SSB periodicity: 5/10 ms

5 References

[1] R4-2508277 WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson