3GPP TSG-RAN WG4 Meeting #116                                          R4-2511107 Bangaluru, India, August 25 - 29, 2025

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD including BS conformance

Agenda Item:	7.21.3.1

Document for:	Approval

Introduction

During RAN4#115, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

##### Absolute limit for in-channel adjacent subband leakage Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement. ##### In-channel adjacent subband blocking requirement Agreement - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

In this contribution, we would like to share our views regarding this topic.

Discussion

In-channel adjacent subband leakage

In last meeting, RAN4 reached agreement for FR2 BS and FR1 LS BS capable of SBFD operation. However, whether 3dB relaxation on top of legacy absolute ACLR/CACLR limit still needs discussion for FR1 WA and MR SBFD-capable BS. Viewing the Rel-18 SI simulation assumptions, the analysis was based on the exemplary 5GHz operating frequency. So if we consider the BS Tx power as to output 53dBm within 100MHz RF bandwidth, then it is equivalent to 33dBm/MHz. Then roughly -5dBm/MHz could be the residual noise level in the adjacent channel by adopting the U6G 38dBc ACLR. Considering that such residual noise level is close to -6dBm/MHz limit, we can accept to reuse the legacy absolute limit as what matters the most is the interference leaked to the UL subband. However, we are also open to discuss whether further relaxation can be introduced.

Proposal 1: For FR1 WA and MR SBFD-capable BS in-channel adjacent subband leakage requirements, check if the legacy absolute ACLR limit (for DU/UD case) and absolute CACLR limit (for DUD case) can be reused unless further relaxation is deemed necessary.

In-channel adjacent subband blocking

Regarding in-channel adjacent subband blocking, discussion outcomes as captured in the WF show good convergence among companies. Apart from the values for each BS type, RAN4 also needs to figure out the rest of this requirements including e.g. the characteristics of interference signal, which could be related to the specific SBFD subband configuration. Firstly, it should be noted that for this in-channel requirement, the interference signal should be within the SBFD DL subband. Secondly, the existing in-band blocking definition can be used as reference somehow, while we prefer fixed single interfering signal bandwidth and its centre frequency offset. So the requirement and also the corresponding conformance testing can be simpler. In conclusion, we have the following proposal.

Proposal 2: For in-channel adjacent subband blocking requirement:

- It should be within the SBFD DL subband.

- A fixed offset between the edge of SBFD UL subband and the centre frequency of interference signal is preferred.

As for the value itself, we understand that since the inter-site and co-site inter-sector interferences are from the same operator, so RAN4 doesn’t need to consider the extreme case where most of the interference transmissions are concentrated in similar beam directions, not to mention that CLI handling and other mechanism can be used to mitigate such interference. In short, it is reasonable to adopt a relaxed value for this requirement, we propose 3dB relaxation on top of the one in WF.

Proposal 3: For FR1 WA BS type, adopt -31dBm as the in-channel adjacent subband blocking requirement.

The following exemplary text proposal on 20MHz and 40MHz UL subband bandwidth would demonstrate our thinking intuitively.

Proposal 4: Exemplary text proposal on in-channel adjacent subband blocking requirement with 20MHz and 40MHz UL subband bandwidth:

Table: SBFD base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 40 | PREFSENS + x dB | Wide Area BS: -31 Medium Range BS: -32 Local Area BS: -34 | DU or UD | 30 (for UD case) or -30 (for DU case) | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | 30 (for lower DL subband) and -30 (for higher DL subband) |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: The interfering signal is defined within the DL subband and the offset is from the edge of UL subband to the interfering signal centre frequency. |  |  |  |  |  |

About the FFS point on how to capture the isolation assumption for this requirement, we prefer to specifically listing the coupling loss in the specification. So as baseline, 49dBm BS transmitter power can be considered here then 49-(-31)=80dB coupling loss can be assumed.

Proposal 5: Adopt the following assumption in the specification:

- For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dB coupling loss between interfering BS transmitter and SBFD BS receiver.

Views regarding the SBFD BS RF conformance requirement

In last meeting, there are some FFS points identified by companies as summarized in the WF [1].

Way forward: - For BS RF conformance requirement, the following issues are identified can be further discussed in next meetings: - Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS. - Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. - FFS other declaration is required - Issue-2: RF channels to be tested for SBFD operation - FFS RF channels to be tested for SBFD operation, e.g. to guarantee SBFD carrier is configured at the left/right-most of the operating band. - Issue-3: Additional test models are needed for SBFD-capable BS. - FFS new parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband. - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband. - FFS the test model for SBFD DL subbband needs to be scalable - FFS signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed. - FFS how to accommodate SBFD time domain pattern with the test model. - Issue-4: UL FRC - The existing UL FRC can be reused for SBFD testing. - FFS any other impact existence on UL FRC. - Issue-5: Interference signal - FFS the interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement

Review the RF core part discussion, we list the agreements (or way forward) that may need dedicated considerations for SBFD conformance testing.

|  | Objective | Agreement/Way forward |
| --- | --- | --- |
| General | Subband configuration | “All declared configurations shall be tested” |
| BS TX requirements | Output power and radiated transmit power | PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols. - Vendors can declare different TX power values for normal and SBFD slots/symbols. |
|  | Output power dynamics | - Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement. - The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. |
|  | Transmit ON/OFF power | - Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol. |
|  | Transmitted signal quality | - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity. |
| BS RX requirements | OTA sensitivity degradation sue to SI | - 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity. |

As we can see, there are plenty of core requirements that shall be specifically considered for SBFD since a fundamental difference between TDD and SBFD is that there will be no more than two DL channels and one UL channel configured simultaneously in time domain. Further, how a specific RF requirement will be defined can impact the discussion here. For instance, interference signal bandwidth could be related to the subband bandwidth configuration; if frequency offset between wanted signal and interference signal needs to be defined, then a fixed value could save some test efforts comparing to legacy frequency sweep manner. Anyhow, further study can be carried out once the SBFD core requirements are stable.

Observation 1: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement.

Observation 2: Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

Conclusion

In this contribution we discussed on the potentially new requirements for SBFD including BS conformance general aspects for SBFD. We have the following observations and proposals:

Observation 1: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement.

Observation 2: Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

Proposal 1: For FR1 WA and MR SBFD-capable BS in-channel adjacent subband leakage requirements, check if the legacy absolute ACLR limit (for DU/UD case) and absolute CACLR limit (for DUD case) can be reused unless further relaxation is deemed necessary.

Proposal 2: For in-channel adjacent subband blocking requirement:

- It should be within the SBFD DL subband.

- A fixed offset between the edge of SBFD UL subband and the centre frequency of interference signal is preferred.

Proposal 3: For FR1 WA BS type, adopt -31dBm as the in-channel adjacent subband blocking requirement.

Proposal 4: Exemplary text proposal on in-channel adjacent subband blocking requirement with 20MHz and 40MHz UL subband bandwidth:

Table: SBFD base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 40 | PREFSENS + x dB | Wide Area BS: -31 Medium Range BS: -32 Local Area BS: -34 | DU or UD | 30 (for UD case) or -30 (for DU case) | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | 30 (for lower DL subband) and -30 (for higher DL subband) |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: The interfering signal is defined within the DL subband and the offset is from the edge of UL subband to the interfering signal centre frequency. |  |  |  |  |  |

Proposal 5: Adopt the following assumption in the specification:

- For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dB coupling loss between interfering BS transmitter and SBFD BS receiver.

- References

- R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.