3GPP TSG-RAN WG4 Meeting #116	R4-2511225

Bengaluru, India, 25th August – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3006 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 correcting 4DL configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correcting configurations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change CA_n3A-n7B-n26A-n78(2A) so that n26 refers to Table 5.3.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corrections are not done |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

##### Table 5.5A.3.3-1a

Table 5.5A.3.3-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier 4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A-n7A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3A-n5A-n7B | CA_n1A-n3A CA_n1A-n5A CA_n1A-n7A CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n5A-n28A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n28A CA_n3A-n5A CA_n3A-n28A CA_n5A-n28A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n5A-n78A | CA_n1A-n3A CA_n1A-n5A CA_n1A-n78A CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3(2A)-n7A-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7(2A)-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3(2A)-n7(2A)-n8A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A-n20A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n20A CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n26A | CA_n3B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3A-n7B-n26A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7B-n26A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3A-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7A-n26(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7B-n26(2A) | CA_n7B CA_n26(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3B-n7B-n26(2A) | CA_n7B CA_n26(2A) CA_n1A-n3A CA_n1A-n7A CA_n1A-n26A CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n3A-n7A-n28A | n35 n75 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | n35 n75 CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7B-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n3B-n7A-n28A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3B-n7B-n28A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n1A-n3A-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3B-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3B-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A)-n7A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A)-n7A-n38A7 | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n7A-n40A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n40A CA_n3A-n7A CA_n3A-n40A CA_n7A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n67A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n75A | - | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n7A-n78A | n35 n75 n785,6 CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A5 CA_n3A-n7A CA_n3A-n78A5 CA_n7A-n78A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7B-n78A | CA_n7B CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n78(2A) | n35 n75 n785,6 CA_n78(2A)5 CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A5 CA_n3A-n7A CA_n3A-n78A5 CA_n7A-n78A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3A-n7A-n78C | CA_n78C CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3A-n7B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n7A-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | CA_n3B_BCS 4 and 5 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3B-n7A-n78C | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3A-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3A-n7B-n78C | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
|  | CA_n3A-n7A CA_n78C CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n78A CA_n7A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |
| CA_n1A-n3B-n7B-n78(2A) | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | CA_n3B_BCS 4 and 5 |  |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3B-n7B-n78C | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3(2A)-n7A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7(2A)-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3(2A)-n7(2A)-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n78A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3A-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3A-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3B-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3B-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3B-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3B-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3(2A)-n7A-n79A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3(2A)-n7A-n79C | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1(2A)-n3(2A)-n7A-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1(2A)-n3(2A)-n7A-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n1A-n3A-n7A-n105A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n105A CA_n3A-n7A CA_n3A-n105A CA_n7A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n8A-n41A | CA_n1A-n3A CA_n1A-n8A CA_n1A-n41A CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n8A-n78A | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n1A-n3(2A)-n8A-n78A | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n8A-n78C | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n1A-n78C CA_n3A-n8A CA_n3A-n78A CA_n3A-n78C CA_n8A-n78A CA_n8A-n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3(2A)-n8A-n78C | CA_n1A-n3A CA_n1A-n8A CA_n1A-n78A CA_n1A-n78C CA_n3A-n8A CA_n3A-n78A CA_n3A-n78C CA_n8A-n78A CA_n8A-n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3A-n18A-n28A | CA_n1A-n3A CA_n1A-n18A CA_n1A-n28A CA_n3A-n18A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
| CA_n1A-n3A-n18A-n41A | n415 CA_n1A-n3A CA_n1A-n18A CA_n1A-n41A5 CA_n3A-n18A CA_n3A-n41A5 CA_n18A-n41A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n18A-n77A | n775 CA_n1A-n3A CA_n1A-n18A CA_n1A-n77A5 CA_n3A-n18A CA_n3A-n77A5 CA_n18A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n20A-n41A | CA_n1A-n3A CA_n1A-n20A CA_n1A-n41A CA_n3A-n20A CA_n3A-n41A CA_n20A-n41A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n20A-n67A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n20A-n71A | CA_n1A-n3A CA_n1A-n20A CA_n1A-n71A CA_n3A-n20A CA_n3A-n71A CA_n20A-n71A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n20A-n77A | CA_n1A-n3A CA_n1A-n20A CA_n1A-n77A CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n20A-n77(2A) | CA_n1A-n3A CA_n1A-n20A CA_n1A-n77A CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n20A-n78A | CA_n1A-n3A CA_n1A-n20A CA_n1A-n78A CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n20A CA_n1A-n78A CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n20A-n78(2A) | CA_n1A-n3A CA_n1A-n20A CA_n1A-n78A CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3A-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n26A-n78C | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3A-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | CA_n26(2A) CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n3A-n26(2A)-n78C | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A CA_n26(2A) CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3B-n26A-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26(2A)-n78A | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | CA_n26(2A) | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n26A-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | CA_n3B_BCS 4 and 5 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3B-n26A-n78C | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3B-n26(2A)-n78(2A) | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | CA_n26(2A) CA_n78(2A) | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n26(2A)-n78C | CA_n1A-n3A CA_n1A-n26A CA_n1A-n78A CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A CA_n26(2A) CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3A-n28A-n40A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n40A CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n28A-n41A | n415,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A5 CA_n3A-n28A CA_n3A-n41A5 CA_n28A-n41A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | n415,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A5 CA_n3A-n28A CA_n3A-n41A5 CA_n28A-n41A5 | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n28A-n77A | n775,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A5 CA_n3A-n28A CA_n3A-n77A5 CA_n28A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n28A-n77(2A) | n775 CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n77(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n28A-n77(3A) | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n1A-n3A-n28A-n78A | n35 n785,6 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  | n35 n785,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A5 CA_n3A-n28A CA_n3A-n78A5 CA_n28A-n78A5 | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202,302 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n28A-n78(2A) | n35 n785,6 CA_n78(2A) 5 CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A5 CA_n3A-n28A CA_n3A-n78A5 CA_n28A-n78A5 | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202, 302 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n3A-n28A-n78C | CA_n78C CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202, 302 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3B-n28A-n78A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3B-n28A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3B-n28A-n78C | CA_n1A-n3A CA_n1A-n28A CA_n1A-n78A CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 1 |
|  |  | n3 | CA_n3B_BCS1 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n1A-n3A-n28A-n79A | n795,6 CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A5 CA_n3A-n28A CA_n3A-n79A5 CA_n28A-n79A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n41A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n41A CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n40A-n77A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n77A CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n77(2A) | CA_n1A-n3A CA_n1A-n40A CA_n1A-n77A CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n40A-n78A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n78A CA_n3A-n40A CA_n3A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n40A-n105A | CA_n1A-n3A CA_n1A-n40A CA_n1A-n105A CA_n3A-n40A CA_n3A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n3A-n41A-n71A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n71A CA_n3A-n41A CA_n3A-n71A CA_n41A-n71A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n41A-n77A | n415,6 n775,6 CA_n1A-n3A CA_n1A-n41A5 CA_n1A-n77A5 CA_n3A-n41A5 CA_n3A-n77A5 CA_n41A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A-n77(2A) | n415,6 n775,6 CA_n1A-n3A CA_n1A-n41A5 CA_n1A-n77A5 CA_n3A-n41A5 CA_n3A-n77A5 CA_n41A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n3A-n41A-n78A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n78A CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n41A-n78C | CA_n1A-n3A CA_n1A-n41A CA_n1A-n78A CA_n1A-n78C CA_n3A-n41A CA_n3A-n78A CA_n3A-n78C CA_n41A-n78A CA_n41A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3(2A)-n41A-n78A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n78A CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3(2A)-n41A-n78C | CA_n1A-n3A CA_n1A-n41A CA_n1A-n78A CA_n1A-n78C CA_n3A-n41A CA_n3A-n78A CA_n3A-n78C CA_n41A-n78A CA_n41A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3A-n41A-n79A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n67A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n67A-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n1A-n3A-n71A-n77A | CA_n1A-n3A CA_n1A-n71A CA_n1A-n77A CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n71A-n77(2A) | CA_n1A-n3A CA_n1A-n71A CA_n1A-n77A CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n71A-n78A | CA_n1A-n3A CA_n1A-n71A CA_n1A-n78A CA_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n71A-n78C | CA_n1A-n3A CA_n1A-n71A CA_n1A-n78A CA_n1A-n78C CA_n3A-n71A CA_n3A-n78A CA_n3A-n78C CA_n71A-n78A CA_n71A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3(2A)-n71A-n78A | CA_n1A-n3A CA_n1A-n71A CA_n1A-n78A CA_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3(2A)-n71A-n78C | CA_n1A-n3A CA_n1A-n71A CA_n1A-n78A CA_n3A-n71A CA_n3A-n78A CA_n71A-n78A CA_n71A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n3A-n75A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77A-n79A | n775,6 n795,6 CA_n1A-n3A CA_n1A-n77A5 CA_n1A-n79A5 CA_n3A-n77A5 CA_n3A-n79A5 CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n77(2A)-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n77(3A)-n79A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n3A-n78A-n105A | CA_n1A-n3A CA_n1A-n78A CA_n1A-n105A CA_n3A-n78A CA_n3A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25,30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n1A-n5A-n7A-n40A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n40A CA_n5A-n7A CA_n5A-n40A CA_n7A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n7A-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n7B-n78A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n7A-n105A | CA_n1A-n5A CA_n1A-n7A CA_n1A-n105A CA_n5A-n7A CA_n5A-n105A CA_n7A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n5A-n28A-n78A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n78A CA_n5A-n28A CA_n5A-n78A CA_n28A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n28A-n79A | CA_n1A-n5A CA_n1A-n28A CA_n1A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n40A-n78A | CA_n1A-n5A CA_n1A-n40A CA_n1A-n78A CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n5A-n40A-n105A | CA_n1A-n5A CA_n1A-n40A CA_n1A-n105A CA_n5A-n40A CA_n5A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n5A-n78A-n79A | CA_n1A-n5A CA_n1A-n78A CA_n1A-n79A CA_n5A-n78A CA_n5A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n5A-n78A-n105A | CA_n1A-n5A CA_n1A-n78A CA_n1A-n105A CA_n5A-n78A CA_n5A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40 , 50 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n7A-n8A-n40A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n40A CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n7A-n8A-n78A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7(2A)-n8A-n78A | CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n20A-n67A | CA_n1A-n7A CA_n1A-n20A CA_n7A-n20A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n20A-n78A | CA_n1A-n7A CA_n1A-n20A CA_n1A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n20A-n78(2A) | CA_n1A-n7A CA_n1A-n20A CA_n1A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n7A-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n7A-n26A-n78C | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) CA_n78(2A) | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7A-n26(2A)-n78C | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n26(2A) CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | CA_n7B BCS0 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n7B-n26A-n78C | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  | CA_n26(2A) | n7 | CA_n7B_BCS0 |  |
|  | CA_n78(2A) | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n7B-n26(2A)-n78C | CA_n1A-n26A CA_n1A-n7A CA_n1A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n26(2A) CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7A-n28A-n38A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n7A-n28A-n78A | n75 n785,6 CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A5 CA_n7A-n28A CA_n7A-n78A5 CA_n28A-n78A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7B-n28A-n78A | CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n7B-n28A-n78C | CA_n7B CA_n78C CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7A-n28A-n78(2A) | n75 n785,6 CA_n78(2A) 5 CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A5 CA_n7A-n28A CA_n7A-n78A5 CA_n28A-n78A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n1A-n7A-n28A-n78C | CA_n78C CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n7A-n38A-n78A7 | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n78A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n78A CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n7A-n40A-n105A | CA_n1A-n7A CA_n1A-n40A CA_n1A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n7A-n67A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n67A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A CA_n78(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n1A-n7A-n75A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78A-n105A | CA_n1A-n7A CA_n1A-n78A CA_n1A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n8A-n40A-n78A | CA_n1A-n8A CA_n1A-n40A CA_n1A-n78A CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n8A-n41A-n78A | CA_n1A-n8A CA_n1A-n41A CA_n1A-n78A CA_n8A-n41A CA_n8A-n78A CA_n41A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n8A-n41A-n78C | CA_n1A-n8A CA_n1A-n41A CA_n1A-n78A CA_n1A-n78C CA_n8A-n41A CA_n8A-n78A CA_n8A-n78C CA_n41A-n78A CA_n41A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n8A-n78A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n8A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A-n41A | CA_n1A-n18A CA_n1A-n28A CA_n1A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n28A-n77A | n775 CA_n1A-n18A CA_n1A-n28A CA_n1A-n77A5 CA_n18A-n28A CA_n18A-n77A5 CA_n28A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n18A-n41A-n77A | n415 n775 CA_n1A-n18A CA_n1A-n41A5 CA_n1A-n77A5 CA_n18A-n41A5 CA_n18A-n77A5 CA_n41A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n20A-n41A-n71A | CA_n1A-n20A CA_n1A-n41A CA_n1A-n71A CA_n20A-n41A CA_n20A-n71A CA_n41A-n71A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n1A-n20A-n41A-n77A | CA_n1A-n20A CA_n1A-n41A CA_n1A-n77A CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n20A-n41A-n77(2A) | CA_n1A-n20A CA_n1A-n41A CA_n1A-n77A CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n20A-n41A-n78A | CA_n1A-n20A CA_n1A-n41A CA_n1A-n78A CA_n20A-n41A CA_n20A-n78A CA_n41A-n78A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n20A-n67A-n78A | CA_n1A-n20A CA_n1A-n78A CA_n20A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n20A-n67A-n78(2A) | CA_n1A-n20A CA_n1A-n78A CA_n20A-n78A CA_n78(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n1A-n20A-n71A-n78A | CA_n1A-n20A CA_n1A-n71A CA_n1A-n78A CA_n20A-n71A CA_n20A-n78A CA_n71A-n78A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n38A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n41A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n41A CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n40A-n77A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A-n77(2A) | CA_n1A-n28A CA_n1A-n40A CA_n1A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n28A-n40A-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40B-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77A | n415,6 n775,6 CA_n1A-n28A CA_n1A-n41A5 CA_n1A-n77A5 CA_n28A-n41A5 CA_n28A-n77A5 CA_n41A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n41A-n77(2A) | n415,6 n775,6 CA_n1A-n28A CA_n1A-n41A5 CA_n1A-n77A5 CA_n28A-n41A5 CA_n28A-n77A5 CA_n41A-n77A5 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n28A-n41A-n79A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n75A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77A-n79A | n775,6 n795,6 CA_n1A-n28A CA_n1A-n77A5 CA_n1A-n79A5 CA_n28A-n77A5 CA_n28A-n79A5 CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n78A-n79A | n785,6 n795,6 CA_n1A-n28A CA_n1A-n78A CA_n1A-n79A CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n28A-n77(2A)-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n28A-n77(3A)-n79A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n40A-n78A-n105A | CA_n1A-n40A CA_n1A-n78A CA_n1A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n1A-n41A-n71A-n77A | CA_n1A-n41A CA_n1A-n71A CA_n1A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n41A-n71A-n77(2A) | CA_n1A-n41A CA_n1A-n71A CA_n1A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n1 | 5, 10,15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n41A-n71A-n78A | CA_n1A-n41A CA_n1A-n71A CA_n1A-n78A CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n1 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n41A-n71A-n78C | CA_n1A-n41A CA_n1A-n71A CA_n1A-n78A CA_n1A-n78C CA_n41A-n71A CA_n41A-n78A CA_n41A-n78C CA_n71A-n78A CA_n71A-n78C | n1 | 5, 10, 15, 20, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n41A-n77A-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n41A-n77(2A)-n79A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n30A-n66A | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n30A-n66(2A) | CA_n2A-n5A CA_n2A-n30A CA_n2A-n66A CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77A | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77A | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n5A-n30A-n77(2A) | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n30A-n77(2A) | n775,6 CA_n2A-n5A CA_n2A-n30A CA_n2A-n77A5 CA_n5A-n30A CA_n5A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n5A-n48A-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48A-n66(2A) | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n2A-n5A-n48(2A)-n66(2A) | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n2A-n5A-n48B-n66(2A) | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A CA_n48B | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n2A-n5A-n48B-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n48B CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5B-n48B-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A CA_n48B | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48(2A)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5B-n48A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n2A-n48A CA_n5A-n66A CA_n5A-n48A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5B-n48(2A)-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n5A-n48(2A)-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n5A-n48B-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A CA_n48B | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48(A-B)-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(A-B)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5A-n48B-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n48B CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48B-n77C | CA_n48B CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5A-n48(2A)-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A5 CA_n5A-n48A CA_n5A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n48(2A)-n77C | CA_n77C CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5A-n66A-n77A | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5B-n66A-n77A | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n5A-n66A-n77A | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30,40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n5A-n66A-n77C | CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5A-n66(2A)-n77A | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n5A-n66A-n77(2A) | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n2A-n5A-n66(2A)-n77(2A) | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n66A-n77(2A) | n775,6 CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n5A-n66A-n77C | n775,6 CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n77A5 CA_n5A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n77A CA_n5A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5A-n66(2A)-n77C | CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n77A CA_n5A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n5B-n66A-n77C | CA_n77C CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A CA_n5A-n77A CA_n5A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n30A-n66A | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n30A-n66(2A) | CA_n2A-n12A CA_n2A-n30A CA_n2A-n66A CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n30A-n77A | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n30A-n77A | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A-n77(2A) | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n30A-n77(2A) | n775,6 CA_n2A-n12A CA_n2A-n30A CA_n2A-n77A5 CA_n12A-n30A CA_n12A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66A-n77A | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n66A-n77A | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66(2A)-n77A | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n66A-n77(2A) | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n12A-n66(2A)-n77(2A) | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66A-n77(2A) | n775,6 CA_n2A-n12A CA_n2A-n66A CA_n2A-n77A5 CA_n12A-n66A CA_n12A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n30A-n66A | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n30A-n66(2A) | CA_n2A-n14A CA_n2A-n30A CA_n2A-n66A CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n30A-n77A | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n14A-n30A-n77A | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n30A-n77(2A) | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n30A-n77(2A) | n775,6 CA_n2A-n14A CA_n2A-n30A CA_n2A-n77A5 CA_n14A-n30A CA_n14A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n66A-n77A | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n14 | n14 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n14A-n66A-n77A | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66(2A)-n77A | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n66A-n77(2A) | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n14 | n14 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n2A-n14A-n66(2A)-n77(2A) | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n66A-n77(2A) | n775,6 CA_n2A-n14A CA_n2A-n66A CA_n2A-n77A5 CA_n14A-n66A CA_n14A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n30A-n77A | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n30A-n77A | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A-n77(2A) | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n30A-n77(2A) | n775,6 CA_n2A-n30A CA_n2A-n77A5 CA_n30A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66A-n77A | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n66A-n77A | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66(2A)-n77A | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n66A-n77(2A) | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66A-n77(2A) | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n29A-n66(2A)-n77(2A) | n775,6 CA_n2A-n66A CA_n2A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A-n77A | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n30A-n66A-n77A | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66(2A)-n77A | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n66A-n77(2A) | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66(2A)-n77(2A) | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n66A-n77(2A) | n775,6 CA_n2A-n30A CA_n2A-n66A CA_n2A-n77A5 CA_n30A-n66A CA_n30A-n77A5 CA_n66A-n77A5 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n41A-n66A-n71A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n48A-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n48B-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 3 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n48B CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n48(2A)-n66A-n77A | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n775,6 CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n48A-n66A-n77C | n775,6 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n775,6 CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A5 CA_n48A-n66A CA_n66A-n77A5 | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 508, 608, 708, 808, 908, 1008 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n48B-n66A-n77C | CA_n48B CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n48(2A)-n66A-n77C | CA_n77C CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2A-n66A-n71A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n77(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66A-n71A-n78A | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A-n28A-n78A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n5A-n28A-n79A | CA_n3A-n5A CA_n3A-n28A CA_n3A-n79A CA_n5A-n28A CA_n5A-n79A CA_n28A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3(2A)-n7A-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7(2A)-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3(2A)-n7(2A)-n8A-n78A | CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n20A-n67A | CA_n3A-n7A CA_n3A-n20A CA_n7A-n20A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78A | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n20A-n78(2A) | CA_n3A-n7A CA_n3A-n20A CA_n3A-n78A CA_n7A-n20A CA_n7A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  | CA_n26(2A) | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n26A-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  | CA_n26(2A) | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  | CA_n78(2A) | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7A-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n26(2A) CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  | CA_n26(2A) | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B BCS 4 and 5 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A) BCS 4 and 5 |  |
|  | CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7B-n26A-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  | CA_n26(2A) ) CA_n78(2A) | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n7B-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n26(2A) CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3B-n7A-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  | CA_n26(2A) | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) | n3 | CA_n3B_BCS 4 and 5 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3B-n7A-n26A-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3B-n7A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n26(2A) CA_n78(2A) | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7A-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n26(2A) CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3B-n7B-n26A-n78A | CA_n3A-n7A CA_n3A-n26A CA_n3A-n78A CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  | CA_n26(2A) | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n78(2A) | n3 | CA_n3B_BCS 4 and 5 | 4 and 5 |
|  |  | n7 | CA_n7B_BCS 4 and 5 |  |
|  |  | n26 | n26 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3B-n7B-n26A-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3B-n7B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  | CA_n26(2A) | n7 | CA_n7B_BCS0 |  |
|  | CA_n78(2A) | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7B-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n7A CA_n3A-n78A CA_n7A-n26A CA_n26A-n78A CA_n7A-n78A CA_n7B CA_n26(2A) CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n7A-n28A-n38A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n7A-n28A-n78A | n35 n75 n785,6 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n35 n75 n785,6 CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A5 CA_n7A-n28A CA_n7A-n78A5 CA_n28A-n78A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n28A-n78(2A) | n35 n75 n785,6 CA_n78(2A)5 CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A5 CA_n7A-n28A CA_n7A-n78A5 CA_n28A-n78A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n7A-n28A-n78C | CA_n78C CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n7B-n28A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n7B CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n7B-n28A-n78C | CA_n7B CA_n78C CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3B-n7A-n28A-n78A | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7A-n28A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7B-n28A-n78A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3B-n7B-n28A-n78C | CA_n7B CA_n78C CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3B-n7A-n28A-n78C | CA_n78C CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3B | n3 | CA_n3B_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n3A-n7A-n38A-n78A7 | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n40A-n78A | CA_n3A-n7A CA_n3A-n40A CA_n3A-n78A CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n40A-n105A | CA_n3A-n7A CA_n3A-n40A CA_n3A-n105A CA_n7A-n40A CA_n7A-n105A CA_n40A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n7A-n67A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n67A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n67 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3A-n7A-n75A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78A-n105A | CA_n3A-n7A CA_n3A-n78A CA_n3A-n105A CA_n7A-n78A CA_n7A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n3A-n8A-n39A-n41A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n39A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n8A-n41A-n78A | CA_n3A-n8A CA_n3A-n41A CA_n3A-n78A CA_n8A-n41A CA_n8A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n8A-n41A-n78C | CA_n3A-n8A CA_n3A-n41A CA_n3A-n78A CA_n3A-n78C CA_n8A-n41A CA_n8A-n78A CA_n8A-n78C CA_n41A-n78A CA_n41A-n78C | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n8A-n41A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n18A-n28A-n41A | n415 CA_n3A-n18A CA_n3A-n28A CA_n3A-n41A CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n28A-n77A | n775 CA_n3A-n18A CA_n3A-n28A CA_n3A-n41A5 CA_n18A-n28A CA_n18A-n41A5 CA_n28A-n41A5 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n18A-n41A-n77A | n415 n775 CA_n3A-n18A CA_n3A-n41A5 CA_n3A-n77A5 CA_n18A-n41A5 CA_n18A-n77A5 CA_n41A-n77A5 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n41A-n71A | CA_n3A-n20A CA_n3A-n41A CA_n3A-n71A CA_n20A-n41A CA_n20A-n71A CA_n41A-n71A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
| CA_n3A-n20A-n41A-n77A | CA_n3A-n20A CA_n3A-n41A CA_n3A-n77A CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n41A-n77(2A) | CA_n3A-n20A CA_n3A-n41A CA_n3A-n77A CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n20A-n41A-n78A | CA_n3A-n20A CA_n3A-n41A CA_n3A-n78A CA_n20A-n41A CA_n20A-n78A CA_n41A-n78A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n67A-n78A | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n20A-n67A-n78(2A) | CA_n3A-n20A CA_n3A-n78A CA_n20A-n78A CA_n78(2A) | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n67 | n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS 4 and 5 |  |
| CA_n3A-n20A-n71A-n78A | CA_n3A-n20A CA_n3A-n71A CA_n3A-n78A CA_n20A-n71A CA_n20A-n78A CA_n71A-n78A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10,15, 20 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n38A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n40A-n41A | CA_n3A-n28A CA_n3A-n40A CA_n3A-n41A CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n28A-n40A-n77A | CA_n3A-n28A CA_n3A-n40A CA_n3A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n40A-n77(2A) | CA_n3A-n28A CA_n3A-n40A CA_n3A-n77A CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n40 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n28A-n41A-n77A | n415,6 n775,6 CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n77A5 CA_n28A-n41A5 CA_n28A-n77A5 CA_n41A-n77A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n77(2A) | n415,6 n775,6 CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n77A5 CA_n28A-n41A5 CA_n28A-n77A5 CA_n41A-n77A5 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  | n415,6 n775,6 CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n77A5 CA_n28A-n41A5 CA_n28A-n77A5 CA_n41A-n77A5 | n3 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n28A-n41A-n78A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A-n78(2A) | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n28A-n41A-n79A | n415,6 n795,6 CA_n3A-n28A CA_n3A-n41A5 CA_n3A-n79A5 CA_n28A-n41A5 CA_n28A-n79A5 CA_n41A-n79A5 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n28A-n77A-n79A | n775,6 n795,6 CA_n3A-n28A CA_n3A-n77A5 CA_n3A-n79A5 CA_n28A-n77A5 CA_n28A-n79A5 CA_n77A-n79A5 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n3 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n28A-n77(2A)-n79A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A CA_n77(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n28A-n77(3A)-n79A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A CA_n77(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n39A-n41A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n40A-n78A-n105A | CA_n3A-n40A CA_n3A-n78A CA_n3A-n105A CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25,30, 35 |  |
| CA_n3A-n41A-n71A-n77A | CA_n3A-n41A CA_n3A-n71A CA_n3A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n71A-n77(2A) | CA_n3A-n41A CA_n3A-n71A CA_n3A-n77A CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n3 | 5, 10,15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10,15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n41A-n71A-n78A | CA_n3A-n41A CA_n3A-n71A CA_n3A-n78A CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20, |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n71A-n78C | CA_n3A-n41A CA_n3A-n71A CA_n3A-n78A CA_n3A-n78C CA_n41A-n71A CA_n41A-n78A CA_n41A-n78C CA_n71A-n78A CA_n71A-n78C | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20, |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n41A-n77A-n79A | n415,6 n775,6 n795,6 CA_n3A-n41A5 CA_n3A-n77A5 CA_n3A-n79A5 CA_n41A-n77A5 CA_n41A-n79A5 CA_n77A-n79A5 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n41A-n77(2A)-n79A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

---End of changes---