3GPP TSG-RAN WG4 Meeting #116	R4-2511315

Bangaluru, India, August 25th - 29th, 2025

Source:	Nokia

Title:	Release independence of 7 MHz channel bandwidth

Agenda item:	7.4.2

Document for:	Approval

Discussion

RAN4 has indicated that support for 7MHz as the maximum or minimum channel bandwidth may be necessary for carrier aggregation, but would be acceptable to support from Rel-18 or Rel-19 in the LS reply to RAN2 [1].

Assuming RAN2 agrees to support the max/min channel bandwidth for 7 MHz from Rel-18 as proposed in our companion paper submitted to RAN2 [2], the release independence of 7 MHz for BCS5 needs further clarification in the following.

The latest version of clause 4 of TS 38.307 v19.1.0 states that:

Configurations with BCSs other than BCS5 are release independent from Rel-15, where the BCSs for configurations are defined in TS 38.101-1 [2] and/or TS 38.101-3 [4]. Configurations with BCS5 are release independent from Rel-17, and BCS5 with signalling is allowed for early implementation from Rel-15. Note that BCS4/BCS5 is not applicable to intra-band EN-DC configurations.

If the max/min signalling is only introduced from Rel-18, the release independence can be only from Rel-18 but can be early implementable from Rel-15. This needs to be clarified in the following.

Configurations with BCSs other than BCS5 are release independent from Rel-15, where the BCSs for configurations are defined in TS 38.101-1 [2] and/or TS 38.101-3 [4]. Configurations with BCS5 are release independent from Rel-17 if 7 MHz is not a maximum or minimum channel bandwidth of any band otherwise from Rel-18, and BCS5 with signalling is allowed for early implementation from Rel-15. Note that BCS4/BCS5 is not applicable to intra-band EN-DC configurations.

TS 38.307 shall clarify that the max/min bandwidth signalling with 7 MHz is only available from Rel-18 but is early implementable from Rel-15.

- References

- R4-2508088 Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth, T-Mobile USA, RAN4#115

- R2-2505903 Discussion on 7 MHz channel bandwidth capabilities, Nokia, RAN2#131

- Annex: draft CR to TS 38.307

3GPP TSG-RAN/WG4 Meeting #116	R4-2511315

# Bangaluru, India,  August 25th - 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release independence of 7 MHz channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Core |  |  |  |  |  | Date: |  |  | August 15, 2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Assuming RAN2 agree to support the max/min channel bandwidth for 7 MHz from Rel-18 as proposed in R2-2505903, the release independence of 7 MHz is further clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that “Configurations with BCS5 are release independent from Rel-17 if 7 MHz is not a maximum or minimum channel bandwidth of any band otherwise from Rel-18, and BCS5 with signalling is allowed for early implementation from Rel-15.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is misunderstood that the max/min signalling with 7 MHz is available from Rel-17 even without early implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 4	General

TSG-RAN has agreed for certain features (see the following clauses) to introduce them in a "release independent way".

This means for each feature:

-	it is "introduced" in a release N, i.e. TS 38.101 [2-5, 8] or TS 38.133 [6] of release N define certain UE requirements for this feature; the feature is indicated in the tables of the following clauses;

-	it is "release independent" starting from a release M (M<N); M for the given feature is provided in the tables of the following clauses;

-	UEs supporting this feature have to fulfil additional requirements in release M or higher which are specified in one or more Annexes of TS 38.307 of release N; the applicable Annexes for a given feature are provided in the tables of the following clauses.

The applicable UE Categories are specified in TS 38.306 [7] according to the release to which the UE conforms.

In the table of release independent features in subsequent clauses, "FDD, TDD" refers to CA or EN-DC configuration composed by only FDD bands or only TDD bands, respectively. "FDD and TDD" refers to CA or EN-DC configuration including both FDD and TDD bands. "SDL and FDD, SDL and TDD" refers to CA configuration including both SDL and FDD bands or both SDL and TDD bands, respectively.  "TDD and SUL" refers to SUL configuration including both TDD and SUL bands. "FDD and TDD and SUL" refers to EN-DC configuration including both FDD, TDD and SUL bands. Unless stated otherwise, the release independent for the band combinations are from Rel-15. Configurations with BCSs other than BCS5 are release independent from Rel-15, where the BCSs for configurations are defined in TS 38.101-1 [2] and/or TS 38.101-3 [4]. Configurations with BCS5 are release independent from Rel-17 if 7 MHz is not a maximum or minimum channel bandwidth of any band otherwise from Rel-18, and BCS5 with signalling is allowed for early implementation from Rel-15. Note that BCS4/BCS5 is not applicable to intra-band EN-DC configurations.

When a new release independent feature is introduced, only the latest release of release independent specification shall be updated. The latest release of release independent specification refers to "release N", i.e. the release in which a feature is introduced into TS 38.101 [2-5, 8] or TS 38.133 [6]. And the common UE requirements tables are also specified from “release N” in the annexes, starting from annex B.

Editor’s note: When introducing new release independent features into this specification with non-category A CRs, the <Release> information on the CR cover page shall be aligned with the <Related_WIs> mentioned on it.