3GPP TSG-RAN WG4 Meeting #116	R4-2511342

# Bengaluru, IN, Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to 38.101-1, LPWUS IBB, OBB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce blocking requirements for LP-WUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add blocking requirements, including IBB, OBB and NBB for LP-WUS, based on legacy requirement (because LR and MR must withstand the same intereferers). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete LP-WUS specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of change>>

## 7.6M	Blocking characteristics for LP-WUS

### 7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive an LP-WUS signal at its configured RBs inside the configured channel, in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

### 7.6M.2	In-band blocking for LP-WUS

The UE shall meet the MDR criterion and side conditions as specified in sub-clause 7.1M with parameters from table Tables 7.6M.2-1 and 7.6.2-2, or with parameters from Table 7.6M.2-2 and 7.6.2-4, depending on band.

Table 7.6M.2-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config 3 | dBm | REFSENSLP-WUS + 6 dB |  | REFSENSLP-WUS + 7 dB | REFSENSLP-WUS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.6M.2-2: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config 3 | dBm | REFSENSLP-WUS + 6 dB3 |
| BWinterferer | MHz | BWChannel |
| FIoffset, case 1 | MHz | (3/2)*BWChannel |
| FIoffset, case 2 | MHz | (5/2)*BWChannel |
| NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For Band n104, the power in transmission bandwidth configuration is REFSENSLP-WUS + 9 dB |  |  |

### 7.6M.3	Out-of-band blocking for LP-WUS

The out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The UE shall meet the MDR criterion and side conditions as specified in sub-clause 7.1M with parameters from table Tables 7.6M.3-1 and 7.6.3-2, or with parameters from Table 7.6M.2-2 and 7.6.2-4, depending on band.

Table 7.6M.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in LP-WUS transmission BW config (dBm) |
| --- | --- |
| 3 MHz | REFSENSLP-WUS + 6.0 dB |
| 5 MHz | REFSENSLP-WUS + 6.0 dB |
| 10 MHz | REFSENSLP-WUS + 6.0 dB |
| 15 MHz | REFSENSLP-WUS + 7.0 dB |
| 20 MHz | REFSENSLP-WUS + 9.0 dB |
| 25 MHz | REFSENSLP-WUS + 10.0 dB |
| 30 MHz | REFSENSLP-WUS + 11.0 dB |
| 35 MHz | REFSENSLP-WUS + 11.5 dB |
| 40 MHz | REFSENSLP-WUS + 12.0 dB |
| 45 MHz | REFSENSLP-WUS + 12.5 dB |
| 50 MHz | REFSENSLP-WUS + 13.0 dB |
| 60 MHz | REFSENSLP-WUS + 14.0 dB |
| 70 MHz | REFSENSLP-WUS + 14.5 dB |
| 80 MHz | REFSENSLP-WUS + 15.0 dB |
| 90 MHz | REFSENSLP-WUS + 15.5 dB |
| 100 MHz | REFSENSLP-WUS + 16.0 dB |

Table 7.6M.3-2: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS + 6 dB | REFSENSLP-WUS + 7 dB | REFSENSLP-WUS + 9 dB |

The number of allowed exceptions to this requirement are defined in clause 7.6.3. For these exceptions, the requirements in clause 7.7M apply.

### 7.6M.4	Narrow band blocking for LP-WUS

This requirement is measure of a receiver's ability to receive an LP-WUS signal at its configured RBs inside the configured channel, in the presence of an unwanted narrow band CW interferer at a frequency which is less than the nominal channel spacing.

The UE shall meet the MDR criterion and side conditions as specified in sub-clause 7.1M with parameters from table Tables 7.6M.4-1.

Table 7.6M.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n72, n74, n75, n76, n85, n100, n101, n106 | Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 4: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |  |  |  |

## <<End of change>>