3GPP TSG-RAN WG4 Meeting #116	R4-2511367

Bengaluru, India, 25 – 29 August 2025

Source:	Ericsson, BT plc

Title:	TP for TR 38.750 to add PC2 2BUL for CA_n3A-n7A-n28A

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to add PC2 2BUL for CA_n3A-n7A-n28A as specified in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.X	CA_n3-n7-n28

### 6.X.1	UE maximum output power

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band |  | Channel bandwidth (MHz) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n7A-n28A | n37 n77 |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n28 |  | 5, 10, 15, 20 |  |
|  | n37 n77 CA_n3A-n7A7 CA_n3A-n28A7 CA_n7A-n28A7 |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n28 |  | 5, 10, 15, 20 |  |
|  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n28 |  | 5, 10, 15, 20 |  |
|  |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n28 |  | n28 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |

### 6.X.2	Reference sensitivity requirements

The coexistence analysis for PC3 DL CA_n3A-n7A-n28A with 2BUL are shown in table 6.x.2-1.

Table 6.X.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n28 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2682 | 17.0 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 26 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |

MSD for PC2 UL CA is calculated from MSD values for PC3 as follows:

If the input signal increases by 3 dB, IMD2 increases by 3*2=6dB and IMD3 increases by 3*3=9dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 5.X.2-2:

Table 5.X.2.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n28 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 796 | 26.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2682 | 25.9 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 32 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |

### 5.X.3	∆TIB and ∆RIB values (Void)

---End of changes---

# Reference

[1] 	RP-251450, Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary Uplink (UL); China §Telecom