3GPP TSG-RAN WG4 Meeting #116	R4-2511396

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.27.3.1

Source: 		Qualcomm Inc.

Title: 		NR NTN UE RF requirements

Document for:		Approval

## 1	Introduction

This contribution discusses the remaining open items for NR_NTN_PH3, focusing on power boosting and sparser sync raster.

## 2	Discussion

Power boosting for NR NTN

Agreements:

- Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting

  - FFS for details, capabilities will be needed for feature list

- When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased

  - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations]

  - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost

- Boost is 0.5 dB

  - FFS on duplex modes

  - For information only: HD-FDD/FD-FDD support is indicated per band

    - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting

- Rel-18 and Rel-19 power boosting schemes can be simultaneously supported

  - FFS for details

- PC3 ACLR requirements apply for power boosted transmissions

- Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

In RAN4#115 a WF [1] was agreed outlining the further study for power boosting:

PC3 power boosting was evaluated in detail in rel-18, and in the end up to 1 dB power boosting was specified for pi/2 BPSK and QPSK modulation orders for inner allocations. Furthermore, boost is mandatory within so called enhanced inner region and optional within the rest of the inner allocations. Evaluations in rel-18 assumed that the transmitter is calibrated as usual to just meet the minimum performance requirements. There could be further potential for power boosting for cases where transmitter was margin to 3GPP minimum requirements.

Observation 1: Power boosting potential beyond rel-18 power boosting can be available if UE has margins to 3GPP minimum requirements.

Given that it has been agreed that the boosting is optional, the most straightforward approach to enable the boosting would be similar to used for allocations outside the enhanced inner region in rel-18:

- Reference power is increased by the amount of power boosting

- Allowed MPR is increased by the amount of power boosting

However, the downside of this approach is that there is no guaranteed power boost, only an allowance to UE. As such a more preferable approach would be to have a set of RB allocations where boosting is mandatory, if supported. To differentiate the UEs which support power boosting, new UE capabilities are needed. Similar to rel-18 power boosting, the boost could be enable by network indication. This also enables RAN5 to define test cases for UEs indicating support for the feature and require improvement for the performance.

Proposal 1: Define new UE capabilities per band

- For UE to indicate support for the additional power boosting

- For network to enable the boosting

Proposal 2: When support for rel-19 boosting is indicated by UE and enabled by network, boosting is required for all RB allocations for DFT-s-OFDM QPSK and pi/2 BPSK.

The specification impacts expected from this approach are similar to rel-18 power boosting: MPR clause needs to describe the impact to reference power and MPR, and configured max output power can further detail the conditions when power boost applies and include the boost to Pcmax formulas.

It is necessary to examine which additional conditions the additional power boosting requires. If the two power boosting schemes are simultaneously supported (boost on top of boost), the conditions also should not be conflicting. If the two schemes consider different modulation orders and RB allocations and as such are not simultaneously active, the conditions can be different.

Proposal 3: Exact conditions for power boosting to apply needs to be defined, including also simultaneous support of rel-18 and rel-19 power boosing schemes.

The rel-18 scheme includes multiple conditions in configured maximum output power scheme. We considered that as a starting point, and made a suggestion on which conditions would be sufficient for rel-19 scheme. These are shown with change marks in the text box below. A lot of simplification is possible as only PC3 operation is considered, and all modulation orders and waveforms types are in scope.

ΔPPowerBoost is defined as 0.5 dB for power class 3, when all of the following conditions are met:

-	If the UE indicates support for UE capability [supportBoost-r19], and if IE [enableBoost-r19]is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 3

-

-	The percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-

-	0 dB otherwise;

In addition to these, MPR increase needs to be included for all modulation orders for CP-OFDM as well as DFT-s-OFDM 16QAM and 64QAM. One implementation option is table notes:

NOTE X: For a UE indicating support for UE capability [supportBoost-r19], and if IE [enableBoost-r19] is set to 1, the reference power is increased by 0.5 dB.

Note X would apply to DFT-s-OFDM QPSK and pi/2 BPSK. This would be combined with MPR increase:

For a UE indicating support for UE capability [supportBoost-r19], and if IE [enableBoost-r19] is set to 1, MPR from Table 6.2.2-1 is increased by the value of ΔPPowerBoost.

Proposal 4: Consider the conditions above for rel-19 power boosting definition.

As the new power boosting scheme is already agreed to be optional, it can be defined for both FD-FDD and HD-FDD. It is expected that there is no impact to Rx requirements, but it is up to RAN5 whether to define test cases for this.

It should be noted that by default UE supports FD-FDD but it can indicate to instead support half-duplex FDD per band. That is, for each band, UE is either HD-FDD or FD-FDD, and same UE can support HD-FDD in band A and FD-FDD in band B.

Proposal 5: New boosting scheme applies for both FD-FDD and HD-FDD

Generally RedCap implementation is considered as simplified from non-RedCap, implicit already from the name REDuced CAPability. As such, we see that if RedCap device can enable power boosting, it is reasonable to apply the same option for the “more capable” non-RedCap NR NTN UEs.

Proposal 6: New boosting scheme applies for both PC3 RedCap and non-RedCap PC3 NTN UEs.

Sync raster and 160ms SSB periodicity

Increased SSB periodicity leads to larger latency of initial access. With 160ms SSB periodicity, dwelling time at each synch raster point will have to be at least 160ms. Taking band n256 as an example, there are 66 synchronization raster points. One round of frequency scan over the band needs about 11 seconds. This is a very large delay and also indicates significant UE power consumption. As such, reduction of sync raster points where 160ms dwell time is required would be beneficial for the system.

The rel-15 sync raster design has a cluster of 3 points every 1.2 MHz apart. The three points of the cluster are separated by 100 kHz. For 3 MHz CBW, similar principle applies but cluster are only 600 kHz apart.

The impact of removing a sync raster point(s) is summarized below oin Table 1.

Table 1: Impact of sync raster puncturing

| CBW [MHz] | Potential to puncture sync raster |
| --- | --- |
| 3 | Removing a single sync raster point makes 2 RF channels unavailable, i.e. there are 2 points on the 100 kHz channel raster where no remaining SSB location is within the channel and subcarrier-aligned |
| 5 | Removing a single raster point makes 4 RF channels unavailable |
| 10 | 80% (12 out of 15 contiguous raster points) can be removed with all 100 kHz RF channel points remaining available |
| 15 | 88.8% of sync raster points (24 out of 27) can be removed with all RF raster points remaining available |
| 20 | 92.3% (36/39) of raster points can be removed with all RF raster points remaining available |

For 3 and 5 MHz CBW, removing a sync raster point completely would make some RF channel positions unavailable. However, this is not a concern if some sync raster points are made unavailable only for the extended 160ms SSB periodicity, i.e. there is no loss in available spectrum.

Observation 2: There is no reduction in available spectrum and RF channel positions if reduced raster point density is applied only for 160ms SSB.

In RAN1 design backwards compatibility is not assumed. If a legacy UE receives the SSB, it is not expected to camp on the cell. As such, some sync raster points can be specified to support both 20ms and 160ms SSB, the consequence is that legacy UE is not excepted to access if 160 ms SSB is used.

Observation 3: RAN1 design does not assume backwards compatibility to older release UEs

Given that there is a lot of redundancy in 10 MHz and wider CBW, analysis is focused on 3 and 5 MHz CBW.

There are multiple options how the raster could be made more sparse. The sync raster is defined as N*1200kHz + M*50 kHz, where N=1:2499 and M ϵ {1, 3, 5}. For 3 MHz CBW sync raster is N * 600 kHz + M * 50 kHz + 300 kHz, M ϵ {1, 3, 5}.

If a single N value is punctured away, the results is 600 kHz contiguous gap for 3 MHz RF channels and 1200 kHz contiguous gap for 5 MHz RF channels. If and M value is punctured away, 3 RF channels with 300 kHz spacing are punctured away. Leaving only single M value would result in 2/3 reduction in raster points with 160ms dwell time possibility to place RF channel supporting 160 ms SSB every 300 kHz. Both approaches would reduce the search time, but they have a downside of not enabling RF channel placement in regular manner e.g. every 500 kHz or 1 MHz – similar to how at least terrestrial spectrum is typically licensed and how placement of channels would be most efficient.

When looking at the general principle how RF channels and sync raster points are linked together, we end up with figure 1. In figure one the bolded headers are M and N values of the sync raster for 5 MHz channel bandwidth, and the numbers below then are a running RF channel index for 100 kHz raster.

Figure 1: Example how 100 kHz RF channel index maps to sync raster points for 5 MHz CBW. NOTE: N,M and GSCN values in the above illustrate the principle of the sync raster and do not accurately match the with exact GCSN values in RAN4 specification.

It can be seen that to enable RF channel every 1 MHz, the punctured raster would need to keep (in this example) combinations sync points {N=1, M=1,3}, {N=2, M=5}, {N=3, M=1}, {N=4, M=3}, {N=5, M=5}, i.e. the cases where RF channel equals K*10. Then from N=6 the pattern repeats. From the exemplified running GSCN the indices to be kept would be 1, 2, 6, 7, 11 and 15. Mathematically, this kind of repeating puncturing pattern can be expressed as

L = 15*x+y, where x is an integer, y ϵ {1, 2, 6, 7, 11, 15} and 0 ≤ 15*x+y ≤ max(GSCN)

That is, L gives the GSCN indices which are required to place RF channels 1 MHz apart. When the above is expanded, it is as follows:

For x = 0, L = {1, 2, 6, 7, 11, 15}

For x = 1, L = {16, 17, 21, 22, 26, 30}

etc.

From Figure 1 two further observations can be made

- In addition to 1 MHz RF channel raster, a lot of additional RF channel will be enabled as each sync raster points enables 4 RF raster points (with 100kHz channel raster)

- aiming for 500 kHz RF channel raster would not meaningfully reduce sync raster points for 5 MHz CBW.

Figure 2 below illustrates the sync raster for 3 MHz CBW

Figure 2: Example how 100 kHz RF channel index maps to sync raster points for 3 MHz CBW. NOTE: N,M and GSCN values in the above illustrate the principle of the sync raster and do not accurately match the with exact GCSN values in RAN4 specification.

Using the same approach as for 5 MHz CBW, sync raster enabling 500 kHz RF channel raster would be

L = 15*x+y, where x is an integer, y ϵ {1, 3, 5, 7, 12, 14} and 0 ≤ 15*x+y ≤ max(GSCN)

And for 1 MHz RF channel raster

L = 15*x+y, where x is an integer, y ϵ {1, 5, 12} and 0 ≤ 15*x+y ≤ max(GSCN)

While mapping the principle in Figures 1 and 2 to the actual sync raster definition, care is needed in matching the repeating pattern the GSCN aligned with Figure 1 and 2. For this reason the formula needs an additional offset resulting in form L = z + 15x+y, where z maps the pattern to the point where the GSCN uses M=1 and lowest RF channel raster point for that GSCN is possible, i.e. the top left corner of figures 1 and 2. Different sync raster puncturing options are listed in Table 2, which also includes the baseline rasters for both 5 MHz and 3 MHz CBWs

Table 2: Sync raster puncturing options

| Scope |  | Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- | --- | --- |
| 5 MHz and wider CBW, baseline | 20 ms SSB | 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} | 3N + (M-3)/2 | 2 – 7498 |
| 3 MHz CBW, baseline | 20 ms SSB | 0 – 3000 MHz | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} | 26638+3N + (M-3)/2 | 26640 – 41636 |
| 5 MHz and wider CBW, enables 1 MHz RF raster | 160 ms SSB | 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} | 3N + (M-3)/2 NOTE X | 2 – 7498 |
| 5 MHz and wider CBW, enables 300 kHz RF raster | 160 ms SSB | 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M = {3} | 26638+3N + (M-3)/2 | 26640 – 41636 |
| 3 MHz CBW, enables 500 kHz RF raster | 160 ms SSB | 0 – 3000 MHz | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} | 3N + (M-3)/2 NOTE Y | 2 – 7498 |
| 3 MHz CBW, enables 300 kHz RF raster | 160 ms SSB | 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M = {3} | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE X: Only combinations of N and M resulting in GSCN = 7+15*x+y, x ≥ 0, y ϵ {1, 2, 6, 7, 11, 15} are valid for 160ms SSB NOTE Y: Only combinations of N and M resulting in GSCN = 26654+15*x+y, x ≥ 0, y ϵ {1, 3, 5, 7, 12, 14} are valid for 160ms SSB |  |  |  |  |  |

Proposal 7: Allow 160ms SSB in limited set of sync raster points to reduce UE cell search latency and improve user experience

Proposal 8: Choose from the following options in Table 2

- Allow 160ms SSB only with single value of M, resulting in 300 kHz RF raster channel

- Allow 160ms SSB on sync raster points which enables in 1 MHz channel raster for 5 MHz CBW and 500 kHz channel raster for 3 MHz CBW.

## 3	Conclusion

In this contribution reply to RAN2 on simulataneous operation between GNSS and NR NTN was discussed. Following observations and proposals were made.

Power boosting for NR NTN

Observation 1: Power boosting potential beyond rel-18 power boosting can be available if UE has margins to 3GPP minimum requirements.

Proposal 1: Define new UE capabilities per band

- For UE to indicate support for the additional power boosting

- For network to enable the boosting

Proposal 2: When support for rel-19 boosting is indicated by UE and enabled by network, boosting is required for all RB allocations for DFT-s-OFDM QPSK and pi/2 BPSK.

Proposal 3: Exact conditions for power boosting to apply needs to be defined, including also simultaneous support of rel-18 and rel-19 power boosing schemes.

Proposal 4: Consider the conditions above for rel-19 power boosting definition.

Proposal 5: New boosting scheme applies for both FD-FDD and HD-FDD

Proposal 6: New boosting scheme applies for both PC3 RedCap and non-RedCap PC3 NTN UEs.

Sync raster and 160ms SSB periodicity

Observation 2: There is no reduction in available spectrum and RF channel positions if reduced raster point density is applied only for 160ms SSB.

Observation 3: RAN1 design does not assume backwards compatibility to older release UEs

Proposal 7: Allow 160ms SSB in limited set of sync raster points to reduce UE cell search latency and improve user experience

Proposal 8: Choose from the following options in Table 2

- Allow 160ms SSB only with single value of M, resulting in 300 kHz RF raster channel

- Allow 160ms SSB on sync raster points which enables in 1 MHz channel raster for 5 MHz CBW and 500 kHz channel raster for 3 MHz CBW.

## 4	References

[1]	R4-2508764 Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc.