3GPP TSG-RAN4 Meeting #116	R4-2511406

# Bangalore, India, August 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0093 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.102: Adding FCC SEM mask to band 252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | B252 requirements introduced to TS 36.102 in RAN4#115, but the CR did not capture FCC SEM mask. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FCC SEM mask is added to be part of NS_06N, NS_07N and NS_08N. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory requirement is missing from the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.2B.3, 6.5A.3.3.1, 6.5B.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change1>

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_06N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_07N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_08N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N | 6.5A.4.4.6 | 254 | Table 5.3A-1 | N/A |  |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5A.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5A.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5A.4.4.5 | 254 | N/A |  |
| NS_06N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | N/A |  |
| NS_11N | 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_12N | 6.5A.4.4.6 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |

### 6.2A.4	Configured transmitted Power for category M1

<End of Change1>

<Start of Change2>

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 |  |  |
| NS_05N | 6.5B.4.4.6 | 254 |  |  |
| NS_06N | 6.5B.4.4.7 6.5B.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.7 6.5B.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.7 6.5B.3.3.1 | 252 | N/A |  |
| NS_11N | 6.5B.4.4.5 | 254 |  |  |
| NS_12N | 6.5B.4.4.6 | 254 |  |  |

### 6.2B.4	Configured transmitted Power for category NB1 and NB2

<End of Change2>

<Start of Change3>

##### 6.5A.3.3.1	Requirements for network signalling value "NS_02N”, “NS_03N”, “NS_06N”, “NS_07N” and “NS_08N”

When "NS_02N", "NS_03N", “NS_06N”, “NS_07N” or “NS_08N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1-1.

Table 6.5A.3.3.1-1: Additional requirements for "NS_02N","NS_03N", “NS_06N”, “NS_07N” and “NS_08N”

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

##### 6.5A.3.3.2	Requirements for network signalling value "NS_04N"

<End of Change3>

<Start of Change4>

##### 6.5B.3.3.1	Requirements for network signalling value "NS_02N”, “NS_06N”, “NS_07N” and “NS_08N”

When "NS_02N", “NS_06N”, “NS_07N” or “NS_08N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.1-1.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N", “NS_06N”, “NS_07N” and “NS_08N”

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

<End of Change4>