# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511441

# Bengaluru, India, August 25-29, 2025

Source:		Ericsson

Title:			Other NTN UE RF impact

Agenda item:		7.27.3.2

Document for:		Approval

# Introduction

In this paper, we present our general overview on OCC RF requirement and also the view on the sync raster for extended SSB periodicity.

# Discussion

## OCC2/4

In LS[1], RAN1 ask RAN4 to evaluate the feasibility of reusing the DMRS bundling on OCC2 /OCC4, or a new requirement would be needed.

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

- FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.

RAN1 agreed the following:

2. Actions:

RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

In WF, the OCC RF impact is stated as below:

Agreements:

- As a starting point, re-use DMRS-bundling requirement values and side conditions

  - Add “No UE specific and common TA updates” to side conditions

  - FFS for further updates based on inputs in next meeting

- UE needs to be able to indicate OCC2/4 feature and/or OCC duration support for GSO deployment scenario only, for NGSO deployment scenario only, or for both GSO and NGSO deployment scenario

  - Further check and optimize alignment with RAN1 capabilities

The simulation is illustrated in Figure 1 with the two options below. It can be observed that the SNR degradation compared to baseline is around 1 dB when DMRS bundling requirement of option 1 is used (e.g 30 degree between slot 0 and slot p). And SNR degradation compared to baseline is around 1.5 dB when DMRS bundling requirement of option 2 is used (e.g 25 degree between slot p and slot p). If the SNR degradation of half dB should be used to set the OCC performance, the phase offset requirement should be 20 degree for both option 1 and option 2. Comparing to DMRS bundling, there is 5 degree tightening between slot p and slot p-1. The reason is that there is additional CFO offset to distort the phase offset from the interferer UE in the combined received signal (e.g the phase offset is CFO*time +phase offset for interfering UE).

As the tightening (5 degree) is not much and we believe for a UE supporting DMRS bundling can support OCC as usually UE perform better than requirement. To reduce the UE test, we think reusing the DMRS bundling phase continuity requirement would be fine when UE capability of the length of maximum duration >= 4 for OCC4.

- Option 1: Phase difference between slot 0 and any slot p

- Option 2: Phase difference between any slot p-1 and slot p

DMRS bundling requirements in TS 38.101-1 cause around 1 dB SNR degradation compared to baseline.

Phase offset of 20 degree cause 0.5 dB SNR degradation for OCC4 BLER performance compared to baseline.

The phase offset of 25 degree of DMRS bundling requirement can apply to the OCC4 requirement.

Figure 1: OCC performance with different phase offset

For DMRS bundling requirements, there is a time duration capability reported to network and such time duration is up to 8 time slots between slot p-1 and 16 ms between slot 0 and slot p. As the test is only done for 2 slots for OCC2 or 4 slots for OCC4, there seems no need to report the time duration for phase continuity by UE.

No need to report the time duration window and reuse the RAN1 OCC2 /OCC4 capability is fine.

For the differentiation of the OCC2/4 support on the GSO and NGSO, there is ongoing discussion in RAN1, there is no need to parallel discussion on this issue and RAN4 can wait RAN1 conclusion.

Delegate RAN1 progress on GSO/NGSO differentiation.

## SSB raster

In WF[2], there is also discussion point on the SSB sync raster optimization

Agreements:

- Discuss updates to sync raster due to impact of cell search delay with 160ms SSB periodicity in next meeting

- Encourage companies to provide detailed proposal on sync raster update

  - Note: also 3 MHz is being introduced in rel-19

The SSB raster are defined in Rel-15 and GSCN number is specified per band. The default SSB periodicity is 20ms in Rel-15. For the UE does not support the extended SSB periodicity, the NBC issue is created when network deploy the 160ms SSB in a cell. Such UE may miss the detection of the cell configured with 160ms SSB. Regarding this NBC issue, RAN1 has an observation below:

Observation

Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

The backward compatibility is not required by legacy UE in RAN1 understanding and therefore, the discussion on the sync raster only apply to the UE supporting the extended 160ms SSB periodicity UE. The background of the introducing the extended SSB periodicity is due to total number of X active beams supported in the hardware is limited and to cover a certain area, the total number of Y beam is needed. 20ms SSB periodicity means the beam covering the same area needs to be illuminated every 20ms. Within each 20ms, the beams with total number of Y divided by X should be illuminated in turn and one can see that maximum number of such beams is 20. If more than 20 beams ( equals to Y divided by X) will be needed, the SSB periodicity needs to be prolonged to accommodate more beams. One can observe that NBC issue and coverage limitation are two issues that cannot be solved at the same time if Y divided by X is greater than 20.

NBC and coverage limitation cannot be solved at the same time if Y divided by X is greater than 20.

On the other hand, even without the NBC issue, to further optimize the SSB raster may be difficult. If a scheme would exist, it is not necessary to associate this scheme with the extended SSB periodicity. In TR 38.817, the SSB maximum step is calculated with the channel BW and the SSB size, together with the channel raster size. For illustrative purpose, the Figure 4.3.1.1-1 is reproduced below. Basically, the raster step size is bounded by the equation 4.3.1.1-1 in TR 38.817.

ΔFSC,Raster ≤ BWConfig – BWPBCH + ΔFCH,Raster	(4.3.1.1-1)

The SSB step size cannot be further increased.

Figure 4.3.1.1-1 [6]: Possible shifts for the PBCH as the transmitted carrier is shifted 
on the RF carrier raster.

Current 3MHz raster is specified separately in TN and it is agreed to reuse the TN 3MHz raster in NTN also. It is observed that the 3MHz raster is not overlapped with the > 3MHz CBW sync raster, this makes the search time for the band containing 3MHz even worse for 160ms SSB. However, RAN4 decide on the non-overlapping raster design for 3MHz CBW, so no room to optimize anything as we see.

Reduction of the sync raster size is not possible.

Keep the current sync raster and no further SSB raster would be needed for extended 160ms SSB.

# Conclusions

In this contribution, we present our general overview for NTN UE RF impact for UL capacity objective with below observations and proposals:

Observation 1 DMRS bundling requirements in TS 38.101-1 cause around 1 dB SNR degradation compared to baseline.

Observation 2 Phase offset of 20 degree cause 0.5 dB SNR degradation for OCC4 BLER performance compared to baseline.

Proposal-1: The phase offset of 25 degree of DMRS bundling requirement can apply to the OCC4 requirement.

Proposal-2: No need to report the time duration window and reuse the RAN1 OCC2 /OCC4 capability is fine.

Proposal-3:Delegate RAN1 progress on GSO/NGSO differentiation.

Observation 3 NBC and coverage limitation cannot be solved at the same time if Y divided by X is greater than 20.

Observation 4 The SSB step size cannot be further increased.

Observation 5 Reduction of the sync raster size is not possible.

Proposal-4: Keep the current sync raster and no further SSB raster would be needed for extended 160ms SSB.

# References

- R1-2503070, Draft LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, MTK

- R4-2508764, Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc.

- R4-2507501,other NTN UE RF requirement, Ericsson

- R4-2307006, LS on PUSCH DMRS bundling for NR NTN coverage enhancement

- R4-1805980, WF on sync raster for NR, Ericsson

- TR 38.817

- R1-2410848, Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode, Huawei