3GPP TSG-RAN WG4 Meeting # 116	R4-2511442

# Bangalore, India,  August 25 - 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | <CR#> | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-5 on introduction of power boosting scheme |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-8-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is agreed to add the power boosting scheme for RedCap UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | The power boosting scheme is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No power boosting scheme will be supported for RedCap UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/ change start/

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2 except for 256QAM.

For a UE indicating support for UE capability [powerBoosting-Pi2BPSK-QPSK-NTN-r19] and if the IE [powerBoost Pi2BPSKQPSKNTNr19] is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] in Table 6.2.2-1 in 3GPP TS 38.101-1[5] in clause 6.2.2 for DFT-s-OFDM waveforms and modulation scheme of QPSK and pi/2 BPSK.

/ next change /

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in 3GPP TS 38.101-1 [5] clause 6.2.2 except for 256QAM. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. When IE [powerBoost Pi2BPSKQPSKNTNr19] is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

/ next change /

### 6.2.4	Configured transmitted power

The requirements for configured transmitted power defined in subclause 6.2.4 of 3GPP TS 38.101-1 [5] clause 6.2.4 shall apply to NTN satellite UE.

ΔPPowerBoost defined in clause 6.2.4 of 3GPP TS 38.101-1 [5] equals

-	0.5 dB for power class 3, when all of the following conditions are met:

-	If the UE indicates support for UE capability [powerBoosting-Pi2BPSK-QPSK-NTN-r19] and if IE [powerBoost Pi2BPSKQPSKNTNr19] is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass defined in clause 6.2.4 of 3GPP TS 38.101-1 [5] is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 9080%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

- 1.5 dB for power class 3, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1

-	and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

- 	If the UE indicates support for UE capability [powerBoosting-Pi2BPSK-QPSK-NTN-r19] and if IE [powerBoost Pi2BPSKQPSKNTNr19] is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 70%

-	0 dB otherwise;

/ Next change /

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

When the IE [powerBoost Pi2BPSKQPSKNTNr19]s set to 1 for a UE supporting the capability of [powerBoosting-Pi2BPSK-QPSK-NTN-r19], for power class 3 UE, the ACLR requirement of PC3 applies.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 3 |
| --- | --- |
| NR ACLR | 30 dB |

/ change end/