# 3GPP TSG-RAN WG4 Meeting #116	R4-2511450

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.9.1

Source:	Moderator (Huawei)

Title:	Topic summary for [116][303] NR_BS_RF_Part2_CLTA_SE

Document for:	Information

# Introduction

Focus of this thread is the co-location test antenna (CLTA) topic of the NR_BS_RF_req_evo work item, as covered in Topic #1.

Additionally, this thread covers discussion on the TX co-existence spurious emissions requirements re-evaluation in Topic #2.

The following agenda items were covered in this summary: 7.9.3.2, 7.9.3.3, 7.9.4.2.2, 7.9.4.3.

Related objectives were extracted from the WID in RP-243270 are provided for reference:

| Core | OTA test enhancement - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition. Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. |
| --- | --- |
| Perf | OTA test enhancement - OTA testing scope reduction: Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for TX IM, RX out of band blocking, for NR FR1. - CLTA: Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1. - TRP simplification: Simplify BS TRP estimation methods (e.g., Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed. |

# Topic #1: CLTA

## Companies’ contributions summary

The following CLTA-related contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2510227 | Ericsson | Observation 1: It can be noticed that the coupling tables are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements. Observation 2: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling vales based on measurements results, due to the large scope and different combination of bands and antenna configuration. Observation 3: For transmitter spurious emission requirement, it can be noticed that the 9 dB additional noise would change the meaning of the original thought behind the requirement. With 9 dB additional noise, the impact on the victim receiver will be significantly larger than 0.5 dB degradation of the sensitivity. Observation 4: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the transmitter spurious emission requirement level derivation. This would indicate that the transmitter spurious emission requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements. Observation 5: To evolve BS co-location transmitter spurious emission requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for transmitter spurious emission requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable. Observation 6: The requirement level for receiver out-of-band blocking interferer level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size. Observation 7: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the out-of-band receiver blocking requirement level derivation. This would indicate that the out-of-band receiver blocking requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements. Observation 8: To evolve BS co-location out-of-band receiver blocking requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for out-of-band receiver blocking requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable. Observation 9: For AAS the BS-to-BS port coupling loss assumed for the transmitter OFF power requirement level derivation should be established based on proper assumptions. The coupling loss should be established based on statistical analysis capturing spatial selectivity, while neighbouring BS(s) provide coverage to UE(s) dropped within each sector. Proposal 1: In respect to transmitter spurious emission for BS type 1-O and bands in the upper region of FR1, consider one of following approaches: - Define co-location transmitter spurious emission requirement based on TRP level considering relevant BS-to-BS coupling loss value. - Define co-location transmitter spurious emission requirement based on field strength level considering co-location scenario. Proposal 2: In respect to receiver out-of-band blocking for BS type 1-O and bands in the upper region of FR1, consider defining the interferer signal based on EIS. Proposal 3: In respect to transmitter OFF power for BS type 1-O and bands in the upper region of FR1, consider defining the requirement based on EIRP. Proposal 4: In respect to transmitter intermodulation requirement for BS type 1-O and bands in the upper region of FR1, carefully investigate the technical background and relevance of the requirement and study test aspects with the intention to simplify OTA testing. |
| R4-2510229 | Ericsson | Discussion on BS RF evolution general aspects Observation 1: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location reference antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz. Observation 2: The height difference between to co-located base stations at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification. Observation 3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time. Observation 4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency. Observation 5: The CLTA antenna efficiency is not captured by the description in current specification. Observation 6: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee robust BS-to-BS co-location at the same site, the CLTA used should be of similar type and have the same characteristics or the requirements should be based on regular OTA parameters. Observation 7: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed. Proposal 1: For BS RF in Rel-20 define a WI with clear goal to resolve open issues identified for BS type 1-O co-location requirements using outcome from Rel-19 coupling loss investigations for bands in upper region of FR1. Proposal 2: For co-location requirement establish requirement derivation framework based on simulations aiming towards the goal to define requirements based on regular OTA parameters testable in traditional OTA test environments. |
| R4-2510228 | Ericsson | Draft CR to TR 37.941: Addition of technical background for enhancement of co-location requirements in clause 6.4 |
| R4-2511132 | ZTE Corporation, Sanechips | Further discussion on OTA test enhancement |
| R4-2511133 | ZTE Corporation, Sanechips | Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements |
| R4-2511524 | Nokia | On OTA test enhancements Observation 1: -46dB can be accepted as isolation in horizontal domain for 4.9GHz. Proposal 1: Do not remove any requirements for Rel-19. |
| R4-2511551 | Huawei, HiSilicon | Discussion towards conclusions on the Core objectives related to CLTA Proposal 1: Consider the following BS-BS isolation data summary as the final version to draw conclusions during RAN4#116 meeting: Table 7: Intra-band BS-BS isolation results 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); -54; -52; -56. Isolation in horizontal domain (dB); -31; -34; -46; -55. Proposal 2: Remove OTA TX IMD requirement for BS type 1-O in FR1 bands above 5 GHz, reuse the existing approach implemented for FR2 requirement. Proposal 3: Arrange work-split for the implementation of the radiated TX IMD requirement optimization for high frequency bands in FR1. |
| R4-2511552 | Huawei, HiSilicon | Draft CR to TR 37.941: Background and motivation for CLTA alternatives discussion |
| R4-2511553 | Huawei, HiSilicon | Big CR to TR 37.941: Rel-19 CLTA-related agreements |
| R4-2510233 | Ericsson | Observation 1: The CLTA is defined by physical size and far-field parameters, whereas co-location is a nearfield problem. Observation 2: The antenna efficiency is not specified for the CLTA which may lead to arbitrary test results. Observation 3: The spatial performance of the CLTA is not specified by considering adequate nearfield characteristics which may lead to arbitrary test results. Observation 4: The co-location requirements are not described in terms of a metric that can be related to the radiated electromagnetic nearfield. Observation 5: The coupling loss depends on the electrical field strength. Observation 6: The reaction integral formulation is identical to the Friis transmission formula in the far-field region. Observation 7: Field-current reaction provides a powerful tool to understand and devise co-location test methods. Proposal 1: Beam steering effects should be integrated into the co-location testing and potential test simplifications should be further studied. Proposal 2: The coupling loss seems to be higher than 30 dB in upper FR1 band. This should be further studied. Proposal 3: Requirements should be derived from the new coupling loss figures in the upper FR1 band. Proposal 4: The correct power reference planes should be defined and agreed Proposal 5: The AAS performance distribution should be analysed statistically and reflected in the requirement derivation. Proposal 6: A percentile level to use in the coupling loss CDF should be agreed upon. Proposal 7: The use of beam steering should be further studied for requirement derivation and test definition. |
| R4-2511525 | Nokia | DraftCR to TR 37.941 on CLTA alternatives - wideband horn antenna |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 1.1.

### Sub-topic 1-1: BS-to-BS isolation analysis

Open issues and candidate options before meeting:

  - Proposal 1: Consider the following BS-BS isolation data summary as the final version to draw conclusions during RAN4#116 meeting. (R4-2511551, Huawei)

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |  |
| Isolation in horizontal domain (dB) | -31 | -34 | -46 | -55 |

  - Proposal 2: The coupling loss seems to be higher than 30 dB in upper FR1 band. This should be further studied. (R4-2510233, Ericsson)

  - Proposal 3: A percentile level to use in the coupling loss CDF should be agreed upon. (R4-2510233, Ericsson)

Moderator:

- WF in R4-2508657 (RAN4#115, Malta) was agreed with the summary of simulation data and measurement results. Proposal 1 claim to be based on WF in R4-2508657.

- WF in R4-2502279 (RAN4#114, Athens) was agreed capturing the following:

  - For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements.

  - Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116).

Recommended WF:

- Based on previous agreement on timeline, double-check online if the BS-to-BS isolation analysis can be concluded this meeting.

### Sub-topic 1-2: Beam steering

Open issues and candidate options before meeting:

- Proposal 1: Beam steering effects should be integrated into the co-location testing and potential test simplifications should be further studied. (R4-2510233, Ericsson)

- Proposal 2: The use of beam steering should be further studied for requirement derivation and test definition. (R4-2510233, Ericsson)

Moderator:

- Clarify if the above proposals are related only to test requirements, or to core and test requirements. Rel-19 core requirements are to be concluded this meeting.

- Beam steering was included in the BS-BS isolation data collection.

- Re-definition of requirements (to consider) beam steering is not explicitly captured in the WID objectives.

Recommended WF: Clarify on the core vs. performance aspects before proceeding with further discussion on the beam steering.

### Sub-topic 1-3: Impacted RF requirements

Proposals: General aspects:

- Proposal 1: Do not remove any requirements for Rel-19. (R4-2511524, Nokia)

- Proposal 2: Requirements should be derived from the new coupling loss figures in the upper FR1 band. (R4-2510233, Ericsson)

- Proposal 3: The correct power reference planes should be defined and agreed. (R4-2510233, Ericsson)

- Proposal 4: The AAS performance distribution should be analysed statistically and reflected in the requirement derivation. (R4-2510233, Ericsson)

Proposals: Requirement-specific aspects:

- Proposal 5: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider Tx OFF power and Tx IMD requirement only. (R4-2511132, ZTE)

- Proposal 6: In respect to transmitter spurious emission for BS type 1-O and bands in the upper region of FR1, consider one of following approaches (R4-2510227, Ericsson):

  - Define co-location transmitter spurious emission requirement based on TRP level considering relevant BS-to-BS coupling loss value.

  - Define co-location transmitter spurious emission requirement based on field strength level considering co-location scenario.

- Proposal 7: In respect to receiver out-of-band blocking for BS type 1-O and bands in the upper region of FR1, consider defining the interferer signal based on EIS (R4-2510227, Ericsson).

- Proposal 8: In respect to transmitter OFF power for BS type 1-O and bands in the upper region of FR1, consider defining the requirement based on EIRP (R4-2510227, Ericsson).

- Proposal 9: In respect to transmitter intermodulation requirement for BS type 1-O and bands in the upper region of FR1, carefully investigate the technical background and relevance of the requirement and study test aspects with the intention to simplify OTA testing (R4-2510227, Ericsson).

- Proposal 10: Remove OTA TX IMD requirement for BS type 1-O in FR1 bands above 5 GHz, reuse the existing approach implemented for FR2 requirement. (R4-2511551, Huawei)

- Proposal 11: Arrange work-split for the implementation of the radiated TX IMD requirement optimization for high frequency bands in FR1. (R4-2511551, Huawei)

Recommended WF

- General aspects:

  - Proposals 1/2: Check with the group on the expected timeline (i.e., Rel-19, Rel-20) for Core co-location requirements decisions, based on the agreed BS-BS isolation data.

  - Proposals 3: Check with the group on the need to define power reference planes.

  - Proposals 4: Check with the group on the need to update (co-location only, or all) requirements with the statistical AAS performance distribution.

- Requirement-specific aspects:

  - Depending on the decision on Proposals 1/2, discuss requirement-specific proposals one-by-one.

### Sub-topic 1-4: Draft CRs

The following Draft CR were submitted:

  - R4-2510228	Ericsson	Draft CR to TR 37.941: Addition of technical background for enhancement of co-location requirements in clause 6.4

  - R4-2511525	Nokia	DraftCR to TR 37.941 on CLTA alternatives - wideband horn antenna

  - R4-2511552	Huawei, HiSilicon	Draft CR to TR 37.941: Background and motivation for CLTA alternatives discussion

  - R4-2511553	Huawei, HiSilicon	Big CR to TR 37.941: Rel-19 CLTA-related agreements

Recommended WF

- Review Draft CRs during the meeting, aiming to Endorse aggregable content.

- Merge agreeable content into the Big CR (aim for the Approval during the meeting).

- In case of conflicting text proposals, baseline tdoc to follow previously agreed work-split. Other inputs can be merged.

### Sub-topic 1-5: Rel-20 related proposals

The following proposals were proposed for Rel-20 work on co-location requirements (R4-2510229, Ericsson):

- Proposal 1: For BS RF in Rel-20 define a WI with clear goal to resolve open issues identified for BS type 1-O co-location requirements using outcome from Rel-19 coupling loss investigations for bands in upper region of FR1.

- Proposal 2: For co-location requirement establish requirement derivation framework based on simulations aiming towards the goal to define requirements based on regular OTA parameters testable in traditional OTA test environments.

Recommended WF:

- As Rel-20 scoping in RAN-level task, only offline discussion on the proposals is encouraged.

- No formal handling of the above proposals.

# Topic #2: TX co-existence spurious emissions requirements re-evaluation

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509190 | Nokia | Observation: The 53 dB MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency. Proposal: To keep the 53 dB MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. |
| R4-2510213 | Ericsson | Observation 1: Some companies raised concerns that with assumptions of ρ(f)= 0 or ρ(f)= 1 outside the wanted carrier is an oversimplified modelling of OOB emissions from an AAS perspective. Observation 2 : A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside. Observation 3 : The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain. Observation 4 : RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions. Observation 5 : In Release-99 approach to evaluate the Minimum Coupling Loss which refers to 100% on the CDF curve, it was reasonable to assume non-AAS passive fixed beam antennas because of the predictable and stable nature of the beam’s directions and unavailability of AAS antennas. Observation 6 : Release-99 approach to evaluate the MCL for AAS antennas has large variations, due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal. Observation 7 : Network grid shift 100% is an unrealistic case that does not depict a real deployment, while the Network grid shift 50% makes more sense from deployment perspective. Observation 8 : The BS-to-BS coupling loss with 100% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~7 dB compared to Rel-99 values. However, as stated before Network grid shift of 50% better reflects the reuse of existing deployment infrastructure. Observation 9 : The BS-to-BS coupling loss with 100% grid shift is higher due to greater spatial separation and increased path loss. Observation 10: BS-to-BS coupling is more dominant than the BS-to-UE coupling, as BS receivers are more sensitive and the use of AAS BSs with high gain directional antennas and higher transmit powers, results in stronger coupling. Proposal 1 : RAN4 to discuss the modelling of antenna gain for unwanted emissions assuming correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations. Proposal 2: Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99.99th percentile of the coupling loss CDF, instead of the minimum value. Proposal 3: RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation. Proposal 4: The BS-to-BS coupling loss with 50% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~5 dB compared to Rel-99 values. Proposal 5 : With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~5 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements. |
| R4-2511134 | ZTE Corporation, Sanechips | Observation 1: antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz. Observation 2: the antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal cancellation within the antenna. Proposal 1: propose to further consider the beamforming gain for harmonic signal based on the OTA measurement results. Proposal 2: propose not to consider the beamforming gain for the non-harmonic signal; Proposal 3: propose to consider the 7GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation. Proposal 4: propose to consider the 2~4GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation. Observation 3: the BS2BS MCL under the different grid shift are summarized as following: Table 2.3-1. the summarized BS2BS MCL under the different grid shift Grid shift; 99% CDF [dB]; Baseline [dB]; Relaxation [dB]. 100%; -94.69; -67; 27.6. 50%; -91; -67; 24. 10%; -77.76; -67; 10.76. Proposal 5: for BS2UE CLI analysis, propose to consider the Micro deployment for the assumption. Observation 4: the BS2UE MCL values are summarized as following: Table 2.4.-1. the summarized BS2UE MCL Deterministic coupling loss [dB]; Baseline [dB]; Relaxation [dB]. -78.6; -53; 25.6. Statistical coupling loss [dB]; Baseline [dB]; Relaxation [dB]. -85.3; -53; 32.3. Observation 5: the required BS2BS spurious emission and BS2UE spurious emission requirements are calculated in the following table. Table 2.5.-1. the required BS2BS spurious emission requirement Grid shift; 99% CDF [dB]; Baseline [dB]; Relaxation [dB]; legacy NF; NF for band n104; legacy one dBm/MHz; new one dBm/MHz. 100%; -94.69; -67; 27.6; 5; 6; -49; -20.31. Table 2.5.-2. the required BS2UE spurious emission requirement coupling loss [dB]; Baseline [dB]; Relaxation [dB]; legacy NF; NF for band n104; legacy one dBm/MHz; new one dBm/MHz. -78.6; -53; 25.6; 9; 12.5; -52; -22.9. -85.3; -53; 32.3; 9; 12.5; -52; -16.2. Proposal 6: for non-harmonic spurious emission requirement at least, the coexistence spurious emission requirement could be relaxed down to -23dBm/MHz under the 100% grid shift with isolation distance as 288m. |
| R4-2511554 | Huawei, HiSilicon | Observation 1: using the 99% percentile approach the coexistence spurious emission output power in UL could be changed to -40dBm. Observation 2: using the 99% percentile approach the coexistence spurious emission output power in DL could be changed to -39dBm. Observation 3: It should be discussed further if micro cell scenarios should be investigated. Observation 4: using the 99% percentile approach the coexistence spurious emission output power in TDD band could be changed to -40dBm. Proposal 1: The co-existence spurious emissions requirement in band n104 could be -40dBm. |
| R4-2510214 | Ericsson | Observation 1: The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations. |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 2.1.

### Sub-topic 2-1: Performance metrics

Open issues and candidate options before meeting:

- Proposal 1: Define a coupling loss approach, similar to the legacy MCL approach, where we discuss around 99.99th percentile of the coupling loss CDF, instead of the minimum value. (R4-2510213, Ericsson)

Recommended WF: Collect CDF percentile values from other companies providing results, and agree on the performance metric value.

### Sub-topic 2-2: Grid shift

Open issues and candidate options before meeting:

- Proposal 1: RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation. (R4-2510213, Ericsson)

Moderator: During previous discussion, some companies were commenting that consistency needs to be kept with the Rel-99 requirements, and grid shift shall be kept as 100%.

Recommended WF: Verify Rel-99 assumptions before agreeing on the grid shift value.

### Sub-topic 2-3: Antenna / array gain considerations

Open issues and candidate options before meeting:

- Proposal 1: RAN4 to discuss the modelling of antenna gain for unwanted emissions assuming correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations. (R4-2510213, Ericsson)

- Proposal 2: propose to further consider the beamforming gain for harmonic signal based on the OTA measurement results. (R4-2511134, ZTE)

- Proposal 3: propose not to consider the beamforming gain for the non-harmonic signal; (R4-2511134, ZTE)

- Proposal 4: propose to consider the 7GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation. (R4-2511134, ZTE)

- Proposal 5: propose to consider the 2~4GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation. (R4-2511134, ZTE)

Recommended WF:

- Check if Proposal 1 is agreeable (only for the purpose of simulation alignment within this objective of the WI).

- Discuss online on remaining proposals.

### Sub-topic 2-7: Micro BS scenario consideration

Open issues and candidate options before meeting:

Proposals

- Proposal 1: To keep the 53 dB MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. (R4-2509190, Nokia)

- Proposal 2: for BS2UE CLI analysis, propose to consider the Micro deployment for the assumption. (R4-2511134, ZTE)

Recommended WF: Discuss online.

### Sub-topic 2-7: Other system-level simulation assumptions

As multiple contribution captures wide range of simulation assumptions, keep this placeholder for related discussion not addressed by other sub-topics.

Recommended WF: TBD.

### Sub-topic 2-7: Tx spurious emissions co-ex requirement

Open issues and candidate options before meeting:

Proposals

- Proposal 1: for non-harmonic spurious emission requirement at least, the coexistence spurious emission requirement could be relaxed down to -23dBm/MHz under the 100% grid shift with isolation distance as 288m. (R4-2511134, ZTE)

- Proposal 2: The co-existence spurious emissions requirement in band n104 could be -40dBm. (R4-2511554, Huawei)

- Proposal 3: The BS-to-BS coupling loss with 50% grid shift assumption at 99.99th percentile on the CDF indicates a potential correction of ~5 dB compared to Rel-99 values. (R4-2510213, Ericsson)

- Proposal 4: With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~5 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements. (R4-2510213, Ericsson)

Recommended WF: Discuss online.

### Sub-topic 2-8: Conformance testing aspects

Selected observations related to the discussion:

Observation 1:	The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations.

Recommended WF: Discuss offline if needed; focus on Core aspects first.