3GPP TSG-RAN WG4 Meeting #116	R4- 2511601

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	7.22.3.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

As per the WF [2] in the last meeting, RAN4 will finalise the following RF requirements for A-IoT device 1:

Receiver:

- Reference sensitivity

- Maximum input power

Transmitter:

- Backscattering loss

- Emission requirements

# 2	Discussion

### 2.1 Receiver RF Requirements

Receiver Sensitivity:

Issue 3-4-1: Reference sensitivity

Agreement in RAN4#115:

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

- Define the minimum sensitivity requirement for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional sensitivity levels can be added in the future if needed.

Table 1: Device Sensitivity at peak antenna gain direction

| Sensitivity Level | Minimum requirement (dBm) |
| --- | --- |
| L1 | -34 |
| Note: It’s assumed that the antenna polarisation of the reader and device are matched. |  |

If the antenna polarisation of the reader and the device are not matched, the EIS over two orthogonal polarisation directions may be measured as per TR 38.810.

3)	Establish a connection between the DUT and the SS with the downlink signal applied to the PolLink= θ-polarization of the measurement antenna.

4)	Position the UE so that the beam is formed towards the measurement antenna in the RX beam peak direction.

5)	Determine EIS(PolMeas= PolLink= for θ-polarization, i.e., the power level for the θ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel.

6)	Switch the downlink to the PolLink=ϕ-polarization of the measurement antenna.

7)	Determine EIS(PolMeas= PolLink= for ϕ-polarization, i.e., the power level for the ϕ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel.

8)	Calculate the resulting averaged EIS as:

EIS = 2*[1/EIS(PolMeas= PolLink= +1/EIS(PolMeas= PolLink=]-1

If the EIS is averaged over two polarisation direction, additional 3dB should be added to the peak EIS requirement. To avoid such a change, it’s recommended that the average EIS is calculated as follows.

- The EIS measurement is performed for both θ-polarization and ϕ-polarization. And the average EIS is calculated as:

EIS = [1/EIS(PolMeas= +1/EIS(PolMeas=]-1

For UHF RFID tags, half-wavelength dipole antennas are typically used due to its simplicity and high efficiency. The antenna pattern is the shape of a donut, and the peak gain direction is not unique.

- The peak antenna gain direction is declared by the device vendor.

Orientation property:

Due to the directivity of its antenna, the OTA sensitivity of the device exhibits orientation variation which follows the antenna gain pattern. In 3GPP, two methods have been studied to characterise the orientation property in the Rx sensitivity requirements for NR, i.e. TRS in FR1 and EIS spherical coverage in FR2.

Both methods need to scan the sphere with several tens of grid points surrounding the device during the measurement. For a given grid point at θ,ϕ, the spherical coverage method may check the device sensitivity against the prefefined threshold, while the TRS method need to measure the actual EISθ,ϕ by fine tuning the input power level.

- The EIS spherical coverage method has lower measurement complexity than the TRS method.

Use the widely used dipole antenna as an example. The gain pattern of an ideal half-wave dipole can be expressed as (max gain normalised to 0dB):

Gθ= cosπ2cosθ2sin2θ

The HPBW (half-power beamwidth) of the ideal half-wave dipole is around 78 degrees. Hence, for 43% (=78/180) spherical coverage, the degradation is ≤ 3dB. In other words, the degradation is >3dB for 67% coverage.

The gain pattern and the CDF of the gain distribution are shown in the Figure 1 and 2, respectively. It can be seen that the gain at 50%-tile CCDF is -4dB.

Figure 1: Gain pattern of the dipole antenna

Figure 2: CCDF of the dipole antenna gain

- Consider to define the minimum EIS partial spherical coverage requirements for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional coverage levels can be added in the future if needed.

Table 2: EIS partial spherical coverage

| Spherical Coverage Level | EIS at 50th %-tile CCDF |
| --- | --- |
| E1 | [-30+2] dBm |
| Note: The EIS is measured in the upper hemisphere only. |  |

- Alternatively, the partial sphere coverage requirement can be defined as the max EIS measured over the partial sphere of an angular width of [+/-45] degrees.

- In order to reduce the test burden, the EIS partial sphere coverage requirement is only verified at the middle frequency of the supported band(s).

Measurement procedure:

The device cannot access the network unless being triggered by the reader. Unlike other 3GPP RATs, there’re no continuous DL data transmissions for AIoT. One way to evaluate the receiver performance is to measure the success rate of the device that correctly responds to the paging that triggers the random-access procedure.

(a)	A-IoT CBRA				(b)	A-IoT CFA

Figure 3: A-IoT Random Access Procedures

This is similar to the Select-Query-Ack message sequence for measuring Rx sensitivity for RFID tags.

- The AIoT CFA (Contention-Free Access) procedure is used to evaluate the receiver sensitivity with success rate >=90%. There is no need to consider false alarm rate.

Regarding the FRC for the sensitivity measurement, the following parameters were proposed as the starting point.

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

The A-IoT paging message which triggers the CFA shall contain multiple fields as specified in TS 38.391.

Figure 6.2.1.1-2: MAC PDU of A-IoT Paging message indicating CFA

It can be seen that the length of the MAC PDU is bound to be larger than 20 bits.

Additionally, large M-values are targeted for high data rates. Since there is no FEC for R2D, larger M-values also requires higher SNR to demod, as shown in RAN1’s contributions. On the other hand, the FRC in NR/LTE for different channel BWs typically have the same SNR threshold for demod (e.g. -1dB for REFSENS FRC).

Figure 6．Performance comparison of M = 2, 6 and 24 for the R2D transmission [R1-2502233]

- Use the following parameters for the FRC for Rx sensitivity measurement.

| PRB | 1 |
| --- | --- |
| M | Down-select from [2 or 6] |
| SCS | 15 |
| TBS | Depending on the size of the MAC PDU of A-IoT paging message indicating CFA |
| CRC | 16 |

Maximum input power:

The device receiver has very limited dynamic range, which is estimated to be around 30 to 40dB. Larger range is possible such as using AGC, but at the cost of increased complexity and power consumption. Given the sensitivity requirement, we propose the max input level as:

- Set the maximum input level to -4 dBm for AIoT devices.

This does not mean that the device will stop working for higher input power levels, but the performance might be degraded.

### 2.2 Transmitter RF requirements

Backscatter output power:

Issue 3-1-1: Transmit output power

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

For A-IoT D1T1-B, the CW node is outside the topology. And the CW node is assumed not to be co-located with the active reader. The CW2D and D2R path loss are likely to be different.

The backscatter loss in dB scale can be defined as: incident CW power - the backscattered power.

As seen from the measurement data for RFID tags [8], the backscatter loss is dependent on the input power of the CW signal, due to the nonlinear behaviour of the chips. The loss ranges from around10dB to 40dB for incident CW power increasing from around -15dBm to 20dBm.

- Consider variable requirements on backscatter loss subject to the incident CW power Pcw.

For OOK modulation, the backscatter loss Lback is

Lback≤10 dB, Pcw≤-15dBm10+ΔdB, Δ=Pcw+15,-15dBm<Pcw≤-5dBm

Alternatively, the requirements can be defined for the backscatter power.

- For OOK modulation, the backscatter power Pbs meet the following requirements:

Pback≥Pcw-10, -35dBm ≤Pcw≤-15dBm-25,  -15dBm<Pcw≤-5dBm

This can be illustrated in the figure below.

Backscatter power Pbs

-15dBm

-25dBm

Incident CW power Pcw

-5dBm

-35dBm

-45dBm

Figure 4: Illustration of the minimum requirement on backscatter power

In theory, the backscatter loss for BPSK modulation should be less than that for OOK modulation. In practice, the BPSK modulation is more difficult to implement for ultra-low cost devices. For example, nearly all commercial RFID tags support only OOK.

- For BPSK modulation, the minimum backscatter power is increased by [4] dB relative to the minimum requirement for OOK.

The above requirements do not consider the loss caused by antenna orientation or polarization mismatch. Hence, the backscatter power/loss requirements should be verified based on the condition that: the CW Tx antenna and the backscatter Rx antenna are aligned with the peak gain direction of the device antenna; the polarization direction of the aforementioned antennas are all aligned.

- The backscatter power/loss requirements do not include the loss caused by antenna orientation or polarization mismatch.

As per [9], the backscatter efficiency ꭓ can be defined as the ratio of the backscattered power to the incident CW power:

ꭓ=ρ2G2αρ1-ρ22,

where ρ is the polarisation efficiency between reader and tag antenna, α is the waveform modulation efficiency, and ρ1 and ρ2 are the complex reflection coefficients between tag antenna and chip in modulating states 1 and 2.

Let β=ρ1-ρ22 , we have ꭓ=ρ2G2αβ. In other words, the backscattered power pback and the incident CW power pcw can be expressed as:

pbackpcw=ρ2G2αβ

The above equation assumes that the reader antenna and the CW antenna share the same polarisation. During the OTA measurement, both antennas can be linearly polarised. The CW signal may be transmitted in two orthogonal polarisation directions in separate steps. And for each CW stimulation, the backscattered power is measured in two orthogonal directions.

More explicitly, for the first set of measurements, we have:

pback,1(PolMeas=θ)=pcw,1(PolMeas=θ)ρ2G2αβ

pback,1(PolMeas=φ)=pcw,1(PolMeas=θ)ρ(1-ρ)G2αβ

Hence, the total backscatter power over two polarisation directions is:

pback,1=pback,1PolMeas=θ+pback,1PolMeas=φ

=pcw,1ρG2αβ

Similarly, for the 2nd set of measurements, we have

pback,2(PolMeas=θ)=pcw,2(PolMeas=φ)ρ(1-ρ)G2αβ

pback,2(PolMeas=φ)=pcw,2(PolMeas=φ)ρ2G2αβ

Hence, the total backscatter power over two polarisation directions is:

pback,2=pback,2PolMeas=θ+pback,2PolMeas=φ

=pcw,2(1-ρ)G2αβ

Combine the two sets of measurements, we have the backscatter efficiency ꭓ without polarisation factor:

pback,1pcw,1+pback,2pcw,2= ρG2αβ+1-ρG2αβ

=G2αβ

ꭓ=pback,1pcw,1+pback,2pcw,2

Note that the incident CW power level pcw,1and pcw,2 should be close enough so that the reflection coefficients ρ1 and ρ2 remain stable during the measurements. For a similar CW power level  pcw (e.g. pcw=( pcw,1+ pcw,2)/2), the backscatter power can be calculated as:

pback= ꭓ*pcw.

For the special case of pcw,1=pcw,2, the backscatter power is:

pback= pback,1+pback,2.

- For backscatter power measurement, the CW signal is transmitted in two orthogonal polarisation directions in separate steps. And for each CW stimulation, the backscattered power is measured in two orthogonal directions. The following equations are used for derive the backscatter power pback:

pback,1=pback,1PolMeas=θ+pback,1PolMeas=φ

pback,2=pback,2PolMeas=θ+pback,2PolMeas=φ

pback= pback,1+pback,2.

Modulation quality:

Issue 3-2-1: SFO requirement

Agreement in RAN4#115:

- No need to directly test SFO if SFO can be indirectly verified via other test.

- FFS whether to explicitly define SFO requirements or not.

Issue 3-2-2: Modulation quality requirements

Agreement in RAN4#115:

- The modulation quality won’t be introduced, and it can be verified by the backscattering loss for OOK or BPSK.

- The timing error caused by SFO can be discussed/defined in the RRM requirements.

SEM + Spurious emissions:

Issue 3-3-1: SEM requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- The applicable frequency range for SEM: from the CBW edge to [OOB boundary]

- Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

Issue 3-3-1: spurious emission requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- OOB boundary

  - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW

  - Other options are not precluded

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

The AIoT system operates in NR/LTE bands. Regarding the spurious emission limits, the general requirements as defined for NR and LTE should apply, which are in line with the Category B limits defined in ITU-R SM.329.

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

It’s also stated in ITU-R SM.329 that:

However, for practical measurement purposes only, the frequency range of the spurious domain may be restricted. As guidance for practical purposes, the following frequency ranges of measurement, as given in Table 1, are normally recommended.

Based on the recommendation in ITU-R SM.329, and considering that AIoT devices operates in FDD bands, the spurious emission requirement can be simplified as Table 3 below. This is also in line with ETSI requirements for UHF RFID [7].

- The general spurious emission limits for AIoT devices are defined as in Table 3.

Table 3: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5 GHz | -30 dBm | 1 MHz |  |
| 5 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 1 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 1 GHz and less than or equal to 2.55 GHz. |  |  |  |

As per the WF [2], the following equation is considered to derive the D2R CBW based on bit duration Tb, chip duration Tc and SFS factor R.

Issue 2-2-1: D2R bandwidth

Agreement in RAN4#115:

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

Assuming 10% SFO, the possible CBWs for D2R can be populated as below:

Table 4: D2R CBWs with 10% SFO

| Tb (μs) | Tc (μs) /Channel Bandwidth (kHz) |  |  |  |  |  |  |  |  | 1SB=2/Tb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 | (kHz) |
| 266.67 | 16.5 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  | 7.5 |
| 133.33 |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  | 15 |
| 66.67 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  | 30 |
| 33.33 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  | 60 |
| 16.67 |  |  |  |  | 264 | 396 | 660 | 1188 |  | 120 |
| 8.33 |  |  |  |  |  | 528 | 792 | 1320 |  | 240 |
| 4.17 |  |  |  |  |  |  | 1056 | 1584 |  | 480 |
| 1.39 |  |  |  |  |  |  |  |  | 3168 | 1440 |
| SFS=1/(2Tc) (kHz) | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |  |
| NOTE 1: Tb and Tc are nominal values without SFO. |  |  |  |  |  |  |  |  |  |  |

Figure 5 illustrates the one-sided spectrum of D2R signals with Tb=266.67 μs (1SB=7.5kHz) and various SFS from 3.75kHz to 480kHz.

Figure 5: The one-sided spectrum of D2R signals with Tb=266.67 μs and various SFS. The harmonic components are centered at n*SFS (n=3,5,7, …)

It can be seen that the power of the odd harmonics relative to the primary frequency component follows: -20log10n for n=3,5,7….

| Harmonic order | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Harmonic power (dBc) | -9.5 | -14.0 | -16.9 | -19.1 | -20.8 | -22.3 | -23.5 | -24.6 | -25.6 | -26.4 |

Assume that the maximum backscatter power is -10dBm for 2SB of the D2R signal (i.e. -13dBm for 1SB). The power of the sidebands at the odd harmonics can be derived as follows:

| Harmonic order / Max power (dBm) of the sideband at the harmonic frequency |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 |
| -13 | -22.5 | -27.0 | -29.9 | -32.1 | -33.8 | -35.3 | -36.5 | -37.6 | -38.6 | -39.4 |

It can be seen that the power of the sideband at the 15th and higher order harmonic frequencies become lower than -36dBm.

- Due to the presence of harmonic frequency components, it’s very difficult for AIoT devices to meet the -36dBm/100kHz spurious emission limit, unless the limit applies beyond the 13th or higher harmonic frequency.

The general spurious emission limit is either -36dBm/100kHz or -30dBm/1MHz. For 1SB=7.5kHz to 120kHz, the majority of backscatter power is included in the 100kHz MBW. For larger 1SBs, the MBW only partially covers the backscattered signal. Hence, the OOB boundary frequency can be defined as follows.

Table 5: Frequency offsets for meeting the general spurious emission limit

| Tb (μs) | 1SB (kHz) | FOOB (offset from Fc) | FOOB (kHz) |
| --- | --- | --- | --- |
| ≤τ/8 | ≤120 | 14*SFS | 52.5~6720 |
| τ/16 | 240 | 10*SFS | 1200/2400/4800 |
| τ/32 | 480 | 6*SFS | 1440/2880 |
| τ/96 | 1440 | 4*SFS | 2880 |

For AIoT, the reader can dynamically schedule any combination of Tb and Tc listed in Table 4. Hence, it’s feasible to view the channel BW for the device is around 3MHz, and the 2SB of 15kHz to 2.88MHz are the transmission BW within the 3MHz CBW. For NR/LTE, the OOB boundary is 6MHz, i.e., 7.5MHz from the carrier frequency.

- Define the OOB boundary frequency (FOOB) as 7.5MHz from the carrier frequency independent of the transmission BW, from which the general spurious emission limit applies.

For emissions between the edge of the transmission BW and the OOB boundary frequency, the SEM can be defined as follows.

- If defining flat SEM, consider to define the requirement for AIoT devices as in Table 6.

Table 6: Spectrum emission mask for AIoT devices (option 1)

| ΔfOOB | Emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| 0-FOOB | [12.5+1.5] | 1SB |
| Note 1: The carrier power includes the two sidebands centred at +/-SFS, but excludes the spectrum around the carrier frequency. |  |  |

- 1SB=2/Tb, SFS=1/(2Tc)

- ΔfOOB =0 is defined as the offset of  (SFS+1SB/2)*1.1 = (1+R)/Tb*1.1 from the carrier frequency;

- FOOB is the OOB boundary frequency

Alternatively, SEM similar to that for NR 3MHz CBW can be defined as follows.

- Alternatively, the SEM can be defined as Table 7, assuming CBW=3MHz.

Table 7: Spectrum emission mask for AIoT devices (option 2)

| ΔfOOB (MHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-1 | [-20] | 30kHz |
| 1-5 | [-25] | 1MHz |
| 5-6 | [-30] | 1MHz |
| Note 1: ΔfOOB =0 is defined as the offset of 1.5MHz from the carrier frequency. |  |  |

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Proposal 1: Define the minimum sensitivity requirement for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional sensitivity levels can be added in the future if needed.

Table 1: Device Sensitivity at peak antenna gain direction

| Sensitivity Level | Minimum requirement (dBm) |
| --- | --- |
| L1 | -34 |
| Note: It’s assumed that the antenna polarisation of the reader and device are matched. |  |

Proposal 2: The EIS measurement is performed for both θ-polarization and ϕ-polarization. And the average EIS is calculated as:

EIS = [1/EIS(PolMeas= +1/EIS(PolMeas=]-1

Proposal 3: The peak antenna gain direction is declared by the device vendor.

Observation 1: The EIS spherical coverage method has lower measurement complexity than the TRS method.

Proposal 4: Consider to define the minimum EIS partial spherical coverage requirements for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional coverage levels can be added in the future if needed.

Table 2: EIS partial spherical coverage

| Spherical Coverage Level | EIS at 50th %-tile CCDF |
| --- | --- |
| E1 | [-30+2] dBm |
| Note: The EIS is measured in the upper hemisphere only. |  |

Proposal 5: Alternatively, the partial sphere coverage requirement can be defined as the max EIS measured over the partial sphere of an angular width of [+/-45] degrees.

Proposal 6: In order to reduce the test burden, the EIS partial sphere coverage requirement is only verified at the middle frequency of the supported band(s).

Proposal 7:The AIoT CFA (Contention-Free Access) procedure is used to evaluate the receiver sensitivity with success rate >=90%. There is no need to consider false alarm rate.

Proposal 8:Use the following parameters for the FRC for Rx sensitivity measurement.

| PRB | 1 |
| --- | --- |
| M | Down-select from [2 or 6] |
| SCS | 15 |
| TBS | Depending on the size of the MAC PDU of A-IoT paging message indicating CFA |
| CRC | 16 |

Proposal 9:Set the maximum input level to -4 dBm for AIoT devices.

Proposal 10:Consider variable requirements on backscatter loss subject to the incident CW power Pcw.

For OOK modulation, the backscatter loss Lback is

Lback≤10 dB, Pcw≤-15dBm10+ΔdB, Δ=Pcw+15,-15dBm<Pcw≤-5dBm

Proposal 11: For OOK modulation, the backscatter power Pbs meet the following requirements:

Pback≥Pcw-10, -35dBm ≤Pcw≤-15dBm-25,  -15dBm<Pcw≤-5dBm

Proposal 12:For BPSK modulation, the minimum backscatter power is increased by [4] dB relative to the minimum requirement for OOK.

Proposal 13:The backscatter power/loss requirements do not include the loss caused by antenna orientation or polarization mismatch.

Proposal 14:For backscatter power measurement, the CW signal is transmitted in two orthogonal polarisation directions in separate steps. And for each CW stimulation, the backscattered power is measured in two orthogonal directions. The following equations are used for derive the backscatter power pback:

pback,1=pback,1PolMeas=θ+pback,1PolMeas=φ

pback,2=pback,2PolMeas=θ+pback,2PolMeas=φ

pback= pback,1+pback,2.

Proposal 15: The timing error caused by SFO can be discussed/defined in the RRM requirements.

Proposal 16:The general spurious emission limits for AIoT devices are defined as in Table 3.

Table 3: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5 GHz | -30 dBm | 1 MHz |  |
| 5 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 1 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 1 GHz and less than or equal to 2.55 GHz. |  |  |  |

Observation 2:Due to the presence of harmonic frequency components, it’s very difficult for AIoT devices to meet the -36dBm/100kHz spurious emission limit, unless the limit applies beyond the 13th or higher harmonic frequency.

Proposal 17:Define the OOB boundary frequency (FOOB) as 7.5MHz from the carrier frequency independent of the transmission BW, from which the general spurious emission limit applies.

Proposal 18:If defining flat SEM, consider to define the requirement for AIoT devices as in Table 6.

Table 6: Spectrum emission mask for AIoT devices (Option 1)

| ΔfOOB | Emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| 0-FOOB | [12.5+1.5] | 1SB |
| Note 1: The carrier power includes the two sidebands centred at +/-SFS, but excludes the spectrum around the carrier frequency. |  |  |

- 1SB=2/Tb, SFS=1/(2Tc)

- ΔfOOB =0 is defined as the offset of  (SFS+1SB/2)*1.1 = (1+R)/Tb*1.1 from the carrier frequency;

- FOOB is the OOB boundary frequency

Proposal 19:Alternatively, the SEM can be defined as Table 7, assuming CBW=3MHz.

Table 7: Spectrum emission mask for AIoT devices (option 2)

| ΔfOOB (MHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-1 | [-20] | 30kHz |
| 1-5 | [-25] | 1MHz |
| 5-6 | [-30] | 1MHz |
| Note 1: ΔfOOB =0 is defined as the offset of 1.5MHz from the carrier frequency. |  |  |

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN#106

- R4-2508116 WF on A-IoT device requirements, CMCC, RAN4#115

- R4-2507722 On the RF requirements for ambient IoT device, Huawei, HiSilicon, RAN4#115

- R1-2502233 Physical channels design on modulation, Huawei, HiSilicon, RAN1#120bis

- 3GPP TR 38.810 Study on test methods, v16.7.0

- ETSI EN 302 208, Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum

- UHF RFID tags backscattered power measurement in reverberation chamber, L. Kone, et al, Electronic Letters, vol. 51, No. 13, June 2015

- On the measurement of the Delta Radar Cross Section for UHF tags, S. Skali, et al, IEEE International Conference on RFID, 2009

- Effect of Gen2 Protocol Parameters on RFID Tag Performance, P. V. Nikitin and K. V. S. Rao, IEEE International conference on RFID, 2009