3GPP TSG-RAN WG4 Meeting #116	R4-2511606

Bengaluru, IN, August 25 – 29, 2025

Agenda Item:	7.24.5.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In the last meeting, RAN4 discussed the simulation for LP-WUS/WUR in FR1 and the progress is captured in the WF [1]. In this contribution, we further discuss the remaining aspects related to the simulation assumption/results for RRM performance evaluation for LP-WUS/WUR UE.

# Simulation assumptions and results

Regarding the target SNR for LP-WUR RRM measurements, it was agreed (in RAN4#114) to include -6dB as side condition requirement in addition to the other agreed values (-3dB, 0.5dB, 2dB dB), which align with RAN1 conclusion. The agreement in RAN4#114 is captured below [2]:

Issue 2-1-1: SINR setting Agreement: Include -6 dB SINR in simulation assumptions SINR and -6 dB SINR is low priority and depending on company input; Treat -0.5dB and 2dB SINR as low priority cases;

Therefore, considering the worst-case scenario, we provide our simulation results based on two values of SNR (-3dB and -6dB) for RRM measurement accuracy as shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Abs (Max of 5% and 95%) | 5% | 50% | 95% | Abs (Max of 5% and 95%) |  |  |
| M=1, L=6 | Serving cell: 101010 Interference Cell: 010101 | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | Serving cell: 01011001 Interference cell: 01100101 | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | Serving cell: 1001010110011001 Interference cell: 1001100101100101 | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | Serving cell: 0110100110101010 Interference Cell: 0110101010011010 | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | Serving cell: 01011010101010011010011001100101 Interference Cell: 01100101011001011001101010100101 | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Serving cell: 00011000000100011000000100010100 Interference Cell: 00010100100010000100010000010100 | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

The simulation results are demonstrated in terms of CDF percentage for the measured RSRP accuracy for LP-SS, including 5%, 50%, 95% as well as the Absolute accuracy. Note that Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point (based on the agreement in Issue 2-2-1 from the WF [2]). The simulation results above demonstrate the RSRP error using different number of samples in TDL-C channel (3 and 5 samples). Regarding the LP-SS binary sequence, we use the corresponding LPSS sequences agreed in [2]. However, for M=4, L=32, we also provide the results based on the set 2 LP-SS sequence discussed in RAN1 [3] as captured below:

Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

Proposal 1: For LP-SS binary sequence, consider the following candidates:

- For M=1, L=6:

  - Serving cell: 		[1 0 1 0 1 0]

  - Interference cell: [0 1 0 1 0 1]

- For M=2, L=8:

  - Serving cell: 		[0 1 0 1 1 0 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1]

- For M=2, L=16:

  - Serving cell: 		[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

  - Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

- For M=4, L=16:

  - Serving cell:		[0110100110101010]

  - Interference cell: [0110101010011010]

- For M=4, L=32 (set 1):

  - Serving cell:		[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

- For M=4, L=32 (set 2):

  - Serving cell:		[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

  - Interference cell: [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

Proposal 2: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Abs (Max of 5% and 95%) | 5% | 50% | 95% | Abs (Max of 5% and 95%) |  |  |
| M=1, L=6 | Serving cell: 101010 Interference Cell: 010101 | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | Serving cell: 01011001 Interference cell: 01100101 | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | Serving cell: 1001010110011001 Interference cell: 1001100101100101 | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | Serving cell: 0110100110101010 Interference Cell: 0110101010011010 | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | Serving cell: 01011010101010011010011001100101 Interference Cell: 01100101011001011001101010100101 | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Serving cell: 00011000000100011000000100010100 Interference Cell: 00010100100010000100010000010100 | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

It can be noticed that even with 3 sample measurement LP-WUS capable UE can achieve satisfactory results for the given simulation assumption in Table 2, 3 and 4. However, to select number of samples for RRM performance evaluation for LP-WUS capable UE, we propose to use at least 3 or 5 samples to guarantee achieving the required performance.

Proposal 3: For OOK-based LP-WUR, use 3 samples for RSRP measurements.

Furthermore, the overall simulation assumption list for RRM performance evaluation for LP-WUS/WUR UE is provided below including the general parameters, cell specific parameters and UE specific parameters.

Table 2: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | 3/5 |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 3: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 1,2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 |
| LP-SS binary sequence |  | Indicated in accuracy table | Indicated in accuracy table |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK based receiver 10 ppm | N/A |
| Timing error |  | T= 5us for OOK-4 with M=1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
| Ês/Iot | dB | -3dB / -6dB | N/A |
| Propagation conditions | - | FR1: TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 4: UE-specific parameters

| [Receiver Filter] | 3rd Order Butterworth with 3.84MHz bandwidth |
| --- | --- |
| [Receiver ADC bit width] | 4-bit ADC |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: For LP-SS binary sequence, consider the following candidates:

- For M=1, L=6:

  - Serving cell: 		[1 0 1 0 1 0]

  - Interference cell: [0 1 0 1 0 1]

- For M=2, L=8:

  - Serving cell: 		[0 1 0 1 1 0 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1]

- For M=2, L=16:

  - Serving cell: 		[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

  - Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

- For M=4, L=16:

  - Serving cell:		[0110100110101010]

  - Interference cell: [0110101010011010]

- For M=4, L=32 (set 1):

  - Serving cell:		[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

- For M=4, L=32 (set 2):

  - Serving cell:		[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

  - Interference cell: [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

Proposal 2: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |
|  |  | 5% | 50% | 95% | Abs (Max of 5% and 95%) | 5% | 50% | 95% | Abs (Max of 5% and 95%) |  |
| M=1, L=6 | Serving cell: 101010 Interference Cell: 010101 | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |
| M=2, L=8 | Serving cell: 01011001 Interference cell: 01100101 | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |
| M=2, L=16 | Serving cell: 1001010110011001 Interference cell: 1001100101100101 | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |
| M=4, L=16 | Serving cell: 0110100110101010 Interference Cell: 0110101010011010 | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |
| M=4, L=32 | Serving cell: 01011010101010011010011001100101 Interference Cell: 01100101011001011001101010100101 | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |
| M=4, L=32 | Serving cell: 00011000000100011000000100010100 Interference Cell: 00010100100010000100010000010100 | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |

Proposal 3: For OOK-based LP-WUR, use 3 samples for RSRP measurements.

# References

- R4-2508280, WF on RRM requirements for NR_LPWUS, vivo.

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo.

- R1-2501583, Summary #5 of discussions on LP-WUS and LP-SS design, vivo